London Borough of Tower Hamlets

Air Quality Annual Status Report for 2022

Date of publication: 27 June 2023

This report provides a detailed overview of air quality in the London Borough of Tower Hamlets during 2022. It has been produced to meet the requirements of the London Local Air Quality Management (LLAQM) statutory process.

Contact details:

Nicoletta Vianello, and Arya Dayanandan

Pollution Team, Environmental Health and Trading Standards

Tower Hamlets Town Hall, 160 Whitechapel Road, London E1 1BJ

environmental.protection@towerhamlets.gov.uk

Contents

Abbreviations………………………………………………………………………...………5

Introduction: Air Quality in the London Borough of Tower Hamlets……………………7

- Air Quality Monitoring………………………………………………………………..…9

  - Locations…………………………………………………….………......9

  - Comparison of Monitoring Results with AQOs………………….….18

- Action to Improve Air Quality…………………………………………………………41

  - Air Quality Action Plan Progress……………………………………..41

- Planning Update and Other New Sources of Emissions………………………….66

  - New or Significantly Changed Industrial or Other Sources…..…...67

- Additional Activities to Improve Air Quality……………………………………...….68

  - London Borough of Tower Hamlets Fleet……………….………….68

  - NRMM Enforcement Project……………………………….…...……68

  - Air Quality Alerts……………………………………………….……...68

Appendix A. Details of Monitoring Site Quality QA/QC………………………….…….69

A1. Automatic Monitoring Sites……………………………..…………….69

A2. Diffusion Tubes……………………………………………………..….69

A3. Adjustments to the Ratified Monitoring Data……………………….73

Appendix B. Full Monthly Diffusion Tube results for 2022…………………………….92

Appendix C. Monitoring Sites……………………………………………………………..99

Tables

Table A. Summary of National Air Quality Standards and Objectives………………...6

Table B. Details of Automatic Monitoring Sites for 2022………………………………..9

Table C. Details of Non-Automatic Monitoring Sites for 2022………………………...10

Table D. Annual mean NO2 Ratified and Bias-adjusted Monitoring Results………...18

Table E. NO2 Automatic Monitoring Results: Comparison with 1-hour Mean Objective, Number of 1-hour Means > 200 μg m-3……………………….….32

Table F. Annual Mean PM10 Automatic Monitoring Results (μg m-3)………………...34

Table G. Automatic Monitoring Results: Comparison with 24-hour Mean Objective, Number of PM10 24-hour Means > 50 μg m-3………………………………..37

Table H. Annual mean PM2.5 Automatic Monitoring Results (μg m-3)…………….....39

Table I. Delivery of Air Quality Action Plan Measures………………...………….……41

Table J. Planning requirements Met by Panning Applications in the London Borough of Tower Hamlets in 2022……………………………………………………...66

Table K. Local Bias Adjustment Factors in 2022…………………………………….....71

Table L. Bias Adjustment factors 2016 – 2022…………………………………………72

Table M. Short-Term to Long-Term Monitoring Data Adjustment (9 sub-tables included)………………………………………………………………………….75

Table N. No2 Fall Off With Distance Calculations……………………………………...85

Table O. No2 Diffusion Tubes Results………………………………………..………....92

Figures

Figure 1. Annual mean NO2 results at automatic monitoring sites…………………...26

Figure 2. NO2 diffusion tube annual mean ratified and bias-adjusted monitoring results at sites 1 – 30 from 2016 to 2022………………………………….....29

Figure 3. NO2 diffusion tube annual mean ratified and bias-adjusted monitoring results at sites 31 – 60 from 2016 to 2022……………………………..….....30

Figure 4. NO2 diffusion tube annual mean ratified and bias-adjusted monitoring results at sites 61 – 92 from 2016 to 2022………………….……………......31

Figure 5. Annual mean PM10 Automatic Monitoring results (μg m-3)…………….…..36

Figure 6. Location of automatic monitoring sites…………………………………...…99

Figure 7. Location of non-automatic monitoring sites ………………………………..100

# Abbreviations

| Abbreviation | Description |
| --- | --- |
| AQAP | Air Quality Action Plan |
| AQMA | Air Quality Management Area |
| AQO | Air Quality Objective |
| BEB | Buildings Emission Benchmark |
| CAB | Cleaner Air Borough |
| EV | Electric Vehicle |
| GLA | Greater London Authority |
| LAEI | London Atmospheric Emissions Inventory |
| LAQM | Local Air Quality Management |
| LLAQM | London Local Air Quality Management |
| NRMM | Non-Road Mobile Machinery |
| PM10 | Particulate matter less than 10 micron in diameter |
| PM2.5 | Particulate matter less than 2.5 micron in diameter |
| TEB | Transport Emissions Benchmark |
| TfL | Transport for London |

Table A.	Summary of National Air Quality Standards and Objectives

| Pollutant | Standard / Objective (UK) | Averaging Period | Date(1) |
| --- | --- | --- | --- |
| Nitrogen dioxide (NO2) | 200 μg m-3 not to be exceeded more than 18 times a year | 1-hour mean | 31 Dec 2005 |
| Nitrogen dioxide (NO2) | 40 μg m-3 | Annual mean | 31 Dec 2005 |
| Particles (PM10) | 50 μg m-3 not to be exceeded more than 35 times a year | 24-hour mean | 31 Dec 2004 |
| Particles (PM10) | 40 μg m-3 | Annual mean | 31 Dec 2004 |
| Particles (PM2.5) | 20 μg m-3 | Annual mean | 2020 |
| Particles (PM2.5) | Target of 15% reduction in concentration at urban background locations | 3-year mean | Between 2010 and 2021 |
| Sulphur dioxide (SO2) | 266 μg m-3 not to be exceeded more than 35 times a year | 15-minute mean | 31 Dec 2005 |
| Sulphur dioxide (SO2) | 350 μg m-3 not to be exceeded more than 24 times a year | 1-hour mean | 31 Dec 2004 |
| Sulphur dioxide (SO2) | 125 μg m-3 mot to be exceeded more than 3 times a year | 24-hour mean | 31 Dec 2004 |

Notes:

(1) Date by which to be achieved by and maintained thereafter

# Introduction: Air Quality in the London Borough of Tower Hamlets

This report provides an annual review and update on air quality in the London Borough of Tower Hamlets.

Air pollution is associated with a number of adverse health impacts; it is recognised as a contributing factor in the onset of heart disease and cancer. Additionally, air pollution particularly affects the most vulnerable in society: children and older people, and those with heart and lung conditions. There is also often a strong correlation with equalities issues, because areas with poor air quality are also often the less affluent areas (Environmental equity, air quality, socioeconomic status and respiratory health, 2010; Air quality and social deprivation in the UK: an environmental inequalities analysis, 2006).

Poor air quality is a serious public health emergency. It affects everybody, especially our most vulnerable residents including children. It has a significant impact on the health and quality of life of all.

The London Borough of Tower Hamlets is determined to tackle poor air quality in the borough. Raising awareness and providing education about the causes and impacts of poor air quality are crucial to achieving this. We want to support our residents and businesses to make a difference to the air quality that is affecting all of us, to enhance our local environment.

Due to Tower Hamlet’s strategic location in London, most pollution in our jurisdiction is from traffic travelling through the Borough. Therefore, we will continue to work in partnership with the Greater London Authority and Transport for London, as well as lobby national government to improve air quality in Tower Hamlets.

It is important to take a comprehensive approach of air quality measures and associated issues, and that is why the Council fully embraces an approach across transport, planning, and development strategies. Therefore, the Council has put in place a clear set of priorities to manage the impact of air pollution and to tackle its negative impact.

Improving the quality of air in the Borough is a priority for the Council to improve the health and well-being of our residents. While there are many factors contributing to poor air quality which can be beyond the control of the Council, we must do everything we can to address the air pollution situation locally.

The most polluting roads in the Borough are along the arterial roads including A13, the Blackwall Tunnel Approach and the Highway.  Air quality exceedances occur along most of the major roads in the Borough. Larger annual mean concentrations are in the West of the Borough. These high concentrations are not only limited to the road centrelines but also spread to residential areas.

In this report, commentaries have been provided about concentration levels of NO2, PM10 and PM2.5 at automatic monitoring sites and / or non-automatic monitoring sites. Since 2016, we have noticed an overall reduction in air pollution concentration levels within the Borough both in terms of annual mean and annual exceedances.

Reasons for the air quality improvement could be linked to the ULEZ (Ultra Low Emission Zone) expansion, NRMM planning conditions in place, requirements to monitor for construction dust at construction sites, air quality / transport / highways projects, implementations of the Air Quality Action Plan (AQAP) actions, and to a comprehensive approach of air quality measures embraced by the Council.

# Air Quality Monitoring

1.1	Locations

Table B.	Details of Automatic Monitoring Sites for 2022

| Site ID | Site Name | X (m) | Y (m) | Site Type | In AQMA? | Distance from monitoring site to relevant exposure(m) | Distance to kerb of nearest road (N/A if not applicable) (m) | Inlet height (m) | Pollutants monitored | Monitoring technique |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TH2 and TH2P | Mile End | 535927 | 182221 | Roadside | Y | 1m (offices) (40m residential) | 3 | 3 | NO2, NO, NOx, PM2.5 | Chemiluminescence; BAM for PM2.5 |
| TH004 | Blackwall | 538290 | 181452 | Roadside | Y | 29m (residential) | 3 | 3 | NO2, NO, NOx, PM2.5, PM10, O3 | Chemiluminescence; UV photometric; FDMS TEOM (for PM) |
| TH002 | Victoria Park | 536487 | 184238 | Background | Y | 290m (residential) | 300 | 2 | NO2, NO, NOx, PM2.5, PM10 | Chemiluminescence; BAM for PM2.5andPM10 |
| TH001 | Millwall Park | 538052 | 178559 | Background | Y | 60m (residential) | 60 | 1.5 | NO2, NO, NOx, PM10, O3 | Chemiluminescence; BAM UV absorption |

Table C.	Details of Non-Automatic Monitoring Sites for 2022

| Site ID | Site Name | X (m) | Y (m) | Site Type | In AQMA? | Distance from monitoring site to relevant exposure(m) | Distance to kerb of nearest road (N/A if not applicable)(m) | Inlet height (m) | Pollutants monitored | Tube co-located with an automatic monitor? (Y/N) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Colombia Rd / Gossett Street | 533883 | 182815 | Kerbside | Y | 5 | 0.5 | 2.4 | NO2 | N |
| 2 | Calvert Ave / Boundary Street | 533507 | 182569 | Kerbside | Y | 4 | 0.5 | 2.3 | NO2 | N |
| 3 | Bethnal Green Rd / Brick Lane | 533860 | 182442 | Kerbside | Y | 3 | 0.5 | 2.3 | NO2 | N |
| 4 | Commercial St / Calvin St | 533611 | 182037 | Kerbside | Y | 7 | 0.5 | 2.4 | NO2 | N |
| 5 | Whitechapel High St (KFC) | 533985 | 181426 | Kerbside | Y | 3 | 0.5 | 2.3 | NO2 | N |
| 6 | Mansell St | 533800 | 181021 | Kerbside | Y | 6 | 0.5 | 2.2 | NO2 | N |
| 7 | St Katherine's Way | 533992 | 180376 | Roadside | Y | 10 | 10 | 2.3 | NO2 | N |
| 8 | Wapping High St / Sampson St | 534444 | 180122 | Kerbside | Y | 3 | 0.5 | 2.4 | NO2 | N |
| 9 | Cartwright Street | 533955 | 180805 | Kerbside | Y | 5 | 0.5 | 2.4 | NO2 | N |
| 10 | Whitechapel Rd / Adler St | 534133 | 181509 | Kerbside | Y | 6 | 0.5 | 2.3 | NO2 | N |
| 11 | Brick Lane / Princelet St | 533866 | 181860 | Kerbside | Y | 5 | 0.5 | 2.3 | NO2 | N |
| 12 | Buckfast St / Bethnal Green Rd | 534259 | 182580 | Kerbside | Y | 4 | 0.5 | 2.5 | NO2 | N |
| 13 | Squirries St / Gosset St | 534313 | 182810 | Kerbside | Y | 4 | 0.5 | 2.3 | NO2 | N |
| 14 | Warner Place/Hackney Rd | 534255 | 183130 | Kerbside | Y | 17 | 0.5 | 2.4 | NO2 | N |
| 15 | Parmiter St / Cambridge Heath Road | 534881 | 183240 | Kerbside | Y | 4 | 0.5 | 2.2 | NO2 | N |
| 16 | Paradise Row / Bethnal Green Rd | 534959 | 182757 | Kerbside | Y | 3 | 0.5 | 2.3 | NO2 | N |
| 17 | Finnis St / Three Colts Lane | 534783 | 182385 | Kerbside | Y | 2 | 0.5 | 2.2 | NO2 | N |
| 18 | Sidney St / Mile End Rd | 534968 | 181878 | Roadside | Y | 6 | 2 | 2.3 | NO2 | N |
| 19 | Philpot St / Commercial Road | 534816 | 181321 | Kerbside | Y | 8 | 0.5 | 2.3 | NO2 | N |
| 20 | Dellow St / The Highway | 534951 | 180779 | Roadside | Y | 4 | 2 | 2.2 | NO2 | N |
| 21 | Queensbridge Rd / Hackney Rd | 533985 | 183122 | Kerbside | Y | 4 | 0.5 | 2.2 | NO2 | N |
| 22 | Wapping Wall / Garnet St | 535133 | 180376 | Kerbside | Y | 3 | 0.5 | 2.4 | NO2 | N |
| 23 | Brodlove Lane | 535598 | 180816 | Kerbside | Y | 3 | 0.5 | 2.2 | NO2 | N |
| 24 | Jubilee Street / Commercial Rd | 535174 | 181290 | Kerbside | Y | 5 | 0.5 | 2.3 | NO2 | N |
| 25 | Cavell St / Stepney Way | 534884 | 181667 | Kerbside | Y | 20 | 1 | 2.3 | NO2 | N |
| 26 | Hannibal Rd / Mile End Rd | 535386 | 182021 | Kerbside | Y | 3 | 0.5 | 2.2 | NO2 | N |
| 27 | Roman Rd / Globe Road | 535296 | 182793 | Kerbside | Y | 12 | 0.5 | 2.2 | NO2 | N |
| 28 | Bonner Road | 535356 | 183223 | Kerbside | Y | 7 | 0.5 | 2.7 | NO2 | N |
| 29 | Grove Rd / Old Ford Rd | 535930 | 183385 | Kerbside | Y | 12 | 0.5 | 2.4 | NO2 | N |
| 30 | Fieldgate Street | 534239 | 181565 | Kerbside | Y | 8 | 0.5 | 2.3 | NO2 | N |
| 31 | Whitechapel Market | 534516 | 181744 | Roadside | Y | 15 | 1.5 | 2.2 | NO2 | N |
| 32 | Globe Rd / Mile End Rd | 535634 | 182148 | Kerbside | Y | 4 | 0.5 | 2.3 | NO2 | N |
| 33 | Stepney Green | 535545 | 181604 | Urban background | Y | 30 | 15 | 2.4 | NO2 | N |
| 34 | Pitsea St / Commercial Rd | 535797 | 181164 | Kerbside | Y | 4 | 0.5 | 2.3 | NO2 | N |
| 35 | Narrow St / Limehouse Link | 535977 | 180879 | Roadside | Y | 15 | 1.5 | 2.6 | NO2 | N |
| 36 | Locksley St / St Paul's Way | 536704 | 181647 | Kerbside | Y | 40 | 0.5 | 2.9 | NO2 | N |
| 37 | Rhodeswell Rd | 536577 | 181379 | Kerbside | Y | 40 | 1 | 2.4 | NO2 | N |
| 38 | Ben Johnson Road | 536080 | 181721 | Kerbside | Y | 4 | 0.5 | 2.6 | NO2 | N |
| 39 | Harford St / Mile End Rd | 536089 | 182258 | Roadside | Y | 3 | 1.5 | 2.2 | NO2 | N |
| 40 | Thoydon Rd | 536105 | 183049 | Kerbside | Y | 7 | 0.5 | 2.4 | NO2 | N |
| 41 | Ford Close / Roman Rd | 536457 | 183301 | Roadside | Y | 2 | 1.5 | 2.3 | NO2 | N |
| 42 | Victoria Park (Co-location site) | 536494 | 184170 | Urban background | Y | 330 | 320 | 2.15 | NO2 | Y |
| 43 | Victoria Park (Co-location site) | 536494 | 184170 | Urban background | Y | 330 | 320 | 2.1 | NO2 | Y |
| 44 | Parnell Rd/Old Ford Rd | 536875 | 183740 | Kerbside | Y | 4 | 0.5 | 2.4 | NO2 | N |
| 45 | St Stephen's Rd / Tredegar Rd | 536713 | 183070 | Kerbside | Y | 3 | 0.5 | 2.3 | NO2 | N |
| 46 | Rhondda Grove / Mile End Rd | 536542 | 182589 | Kerbside | Y | 5 | 0.5 | 2.5 | NO2 | N |
| 47 | Wentworth Mews | 536452 | 182454 | Kerbside | Y | 15 | 0.5 | 2.5 | NO2 | N |
| 48 | Ackroyd Drive | 536768 | 181772 | Kerbside | Y | 40 | 0.5 | 2.5 | NO2 | N |
| 49 | Dod St / Burdett Rd | 537049 | 181292 | Kerbside | Y | 5 | 0.5 | 2.5 | NO2 | N |
| 50 | Rich Street | 536937 | 180987 | Roadside | Y | 3 | 1.5 | 2.2 | NO2 | N |
| 51 | Watney Market | 534938 | 181257 | Roadside | Y | 10 | 15 | 2.2 | NO2 | N |
| 52 | Wick Lane / Autumn St | 537304 | 183619 | Kerbside | Y | 3 | 0.5 | 2.4 | NO2 | N |
| 53 | Fairfield Road / Tredegar Road | 537159 | 183415 | Kerbside | Y | 4 | 0.5 | 2.4 | NO2 | N |
| 54 | Bow Rd / Glebe Terrace | 537525 | 182887 | Kerbside | Y | 5 | 0.5 | 2.4 | NO2 | N |
| 55 | TH Cemetery Park | 536732 | 182361 | Roadside | Y | 15 | 5 | 2.5 | NO2 | N |
| 56 | Bow Common Lane / St Paul's Way | 537248 | 181820 | Kerbside | Y | 30 | 0.5 | 2.3 | NO2 | N |
| 57 | Augusta St / Giraud St | 537516 | 181392 | Kerbside | Y | 15 | 1 | 2.4 | NO2 | N |
| 58 | Dolphin Lane | 537539 | 180688 | Kerbside | Y | 7 | 1 | 2.9 | NO2 | N |
| 59 | Westferry Road / Limehouse Link Jnct | 537100 | 180791 | Kerbside | Y | 7 | 1 | 2.2 | NO2 | N |
| 60 | Cascades, Westferry Road | 537115 | 180074 | Kerbside | Y | 18 | 0.5 | 2.4 | NO2 | N |
| 61 | Bow Rd / Alfred St | 537056 | 182773 | Kerbside | Y | 6 | 0.5 | 2.4 | NO2 | N |
| 62 | Mast House Terrace | 537348 | 178690 | Kerbside | Y | 5 | 0.5 | 2.7 | NO2 | N |
| 63 | Millwall Park | 538246 | 178689 | Urban background | Y | 300 | 250 | 2.3 | NO2 | N |
| 64 | Lime harbour | 537953 | 179357 | Kerbside | Y | 10 | 0.5 | 2.2 | NO2 | N |
| 65 | Manchester Road / East Ferry Road | 538032 | 178360 | Kerbside | Y | 2 | 0.5 | 2.3 | NO2 | N |
| 66 | Millwall Park | 538258 | 178689 | Urban background | Y | 300 | 250 | 2.3 | NO2 | N |
| 67 | Seyssel Street | 538544 | 178767 | Kerbside | Y | 15 | 0.5 | 2.3 | NO2 | N |
| 68 | Manchester Road / Ollife Street | 538431 | 179044 | Kerbside | Y | 3 | 0.5 | 2.3 | NO2 | N |
| 69 | Lawnhouse Close | 538190 | 179750 | Kerbside | Y | 30 | 0.5 | 2.3 | NO2 | N |
| 70 | Admirals Way | 537424 | 179910 | Kerbside | Y | 15 | 0.5 | 2.3 | NO2 | N |
| 71 | Toynbee St / Commercial St | 533689 | 181705 | Roadside | Y | 10 | 2 | 2.5 | NO2 | N |
| 72 | Prestons Road / Coldharbour | 538364 | 180188 | Kerbside | Y | 4 | 0.5 | 2.2 | NO2 | N |
| 73 | John Smith Mews | 538742 | 180756 | Kerbside | Y | 10 | 0.5 | 2.3 | NO2 | N |
| 74 | Poplar High St / Cotton St | 538244 | 180761 | Kerbside | Y | 10 | 0.5 | 2.2 | NO2 | N |
| 75 | Hale Street | 537661 | 180768 | Kerbside | Y | 7 | 0.5 | 2.3 | NO2 | N |
| 76 | Chrisp Street / E India Dock Road | 537940 | 181021 | Kerbside | Y | 20 | 0.5 | 2.7 | NO2 | N |
| 77 | Morris / Barchester Street | 537731 | 181761 | Kerbside | Y | 4 | 0.5 | 2.5 | NO2 | N |
| 78 | Devons Road / Campbell Road | 537577 | 182232 | Kerbside | Y | 10 | 0.5 | 2.4 | NO2 | N |
| 79 | Hatfield Terrace / Fairfield Road | 537355 | 183059 | Kerbside | Y | 3 | 0.5 | 2.4 | NO2 | N |
| 80 | Wrexham Road | 537581 | 183209 | Kerbside | Y | 3 | 0.5 | 2.4 | NO2 | N |
| 81 | Bromley High Street / St Leonards | 537868 | 182912 | Kerbside | Y | 5 | 0.5 | 2.4 | NO2 | N |
| 82 | Devas Street / Devons road | 537821 | 182332 | Kerbside | Y | 7 | 0.5 | 2.4 | NO2 | N |
| 83 | Zetland Street / A12 | 538178 | 181747 | Kerbside | Y | 50 | 0.5 | 2.3 | NO2 | N |
| 84 | Blair Street (End of Street) | 538365 | 181180 | Roadside | Y | 15 | 5 | 2.5 | NO2 | N |
| 85 | Portree Street | 538895 | 181296 | Kerbside | Y | 4 | 0.5 | 2.3 | NO2 | N |
| 86 | Newport Avenue | 538954 | 180872 | Kerbside | Y | 15 | 0.5 | 2.6 | NO2 | N |
| 87 | Mile End Road Corner Bancroft Rd | 535929 | 182220 | Kerbside | Y | 30 | 0.5 | 2.3 | NO2 | N |
| 88 | Shirbutt St o/s Holy Family School | 537555 | 180892 | Kerbside | Y | 10 | 0.5 | 2.3 | NO2 | N |
| 89 | Thames Path Storers Quay | 538730 | 178733 | Roadside | Y | 4 | 10 | 2.3 | NO2 | N |
| 90 | Sextant Avenue | 538674 | 178888 | Kerbside | Y | 4 | 1 | 2.3 | NO2 | N |
| 91 | At the entrance of MOT station | 539007 | 181146 | Kerbside | Y | 8 | 1.9 | 2.5 | NO2 | N |
| 92 | At the exit of MOT station | 538907 | 181127 | Roadside | Y | 12 | 3.7 | 2.3 | NO2 | N |

1.2	Comparison of Monitoring Results with AQOs

The results presented are after adjustments for “annualisation” and for distance to a location of relevant public exposure (if required), the details of which are described in Appendix A.

Table D.	Annual Mean NO2 Ratified and Bias-adjusted Monitoring Results

|  |  |  |  |  |  |  | Annual Mean Concentration (μg m-3) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Site ID | Site type | Valid data capture for monitoring period % a | Valid data capture 2022 % b | 2016 | 2017 | 2018 | 2019 | 2020 | 2021 | 2022 |
| Automatic Monitoring Sites |  |  |  |  |  |  |  |  |  |  |
| TH2 Mile End | Automatic | N/A | 99 | 52 | 48 | 47 | 35 | 25 | 26 | 23 |
| TH004 Blackwall | Automatic | N/A | 62 | 59 | 56 | 51 | 47 | 39 | 37 | 28 |
| TH002 Victoria Park | Automatic | N/A | 98 | 32 | 32 | 26 | 24 | 17 | 16 | 17 |
| TH001 Millwall Park | Automatic | N/A | 78 | 25.3 | 26 | 23 | 24 | 17 | 17 | 20 |
| Non-Automatic Monitoring Sites |  |  |  |  |  |  |  |  |  |  |
| 1 | Colombia Rd / Gossett Street | N/A | 92 | 37 | 39 | 34 | 33 | 25 | 22 | 20 |
| 2 | Calvert Ave / Boundary Street | N/A | 92 | 41 | 40 | 37 | 35 | 26 | 22 | 21 |
| 3 | Bethnal Green Rd / Brick Lane | N/A | 67 | 46 | 45 | 36 | 37 | 27 | 26 | 26 |
| 4 | Commercial St / Calvin St | N/A | 67 | 60 | 60 | 53 | 48 | 34 | 33 | 30 |
| 5 | Whitechapel High St (KFC) | N/A | 100 | 64 | 62 | 61 | 48 | 34 | 38 | 37 |
| 6 | Mansell St | N/A | 100 | 71 | 75 | 50 | 45 | 35 | 30 | 32 |
| 7 | St Katherine's Way | N/A | 33 | 34 | 30 | 28 | 28 | 20 | 19 | 25 |
| 8 | Wapping High St / Sampson St | N/A | 100 | 36 | 33 | 31 | 30 | 22 | 21 | 21 |
| 9 | Cartwright Street | N/A | 75 | - | - | 33 | 34 | 25 | 22 | 23 |
| 10 | Whitechapel Rd / Adler St | N/A | 57 | - | - | 46 | 40 | 29 | 28 | 31 |
| 11 | Brick Lane / Princelet St | N/A | 75 | 44 | 40 | 35 | 32 | 24 | 22 | 24 |
| 12 | Buckfast St/Bethnal Green Rd | N/A | 100 | 42 | 39 | 35 | 32 | 24 | 23 | 22 |
| 13 | Squirries St / Gosset St | N/A | 92 | - | - | 38 | 38 | 27 | 25 | 26 |
| 14 | Warner Place / Hackney Rd | N/A | 92 | 42 | 41 | 38 | 35 | 26 | 23 | 26 |
| 15 | Parmiter St / Cambridge Heath Road | N/A | 92 | - | - | 45 | 41 | 30 | 29 | 27 |
| 16 | Paradise Row / Bethnal Green Rd | N/A | 83 | 50 | 42 | 41 | 36 | 28 | 28 | 28 |
| 17 | Finnis St / Three Colts Lane | N/A | 92 | 35 | 35 | 29 | 31 | 21 | 20 | 21 |
| 18 | Sidney St / Mile End Rd | N/A | 100 | 47 | 46 | 40 | 37 | 29 | 28 | 26 |
| 19 | Philpot St / Commercial Road | N/A | 100 | 54 | 51 | 44 | 41 | 31 | 30 | 30 |
| 20 | Dellow St / The Highway | N/A | 100 | 69 | 59 | 52 | 49 | 34 | 38 | 38 |
| 21 | Queensbridge Rd / Hackney Rd | N/A | 100 | - | - | 55 | 35 | 26 | 24 | 24 |
| 22 | Wapping Wall / Garnet St | N/A | 100 | 37 | 34 | 32 | 30 | 23 | 25 | 24 |
| 23 | Brodlove Lane | N/A | 92 | 45 | 46 | 43 | 40 | 30 | 29 | 29 |
| 24 | Jubilee Street / Commercial Rd | N/A | 100 | 65 | 62 | 64 | 47 | 33 | 37 | 35 |
| 25 | Cavell St / Stepney Way | N/A | 67 | 45 | 45 | 40 | 38 | 30 | 26 | 29 |
| 26 | Hannibal Rd / Mile End Rd | N/A | 100 | 50 | 50 | 44 | 42 | 29 | 27 | 26 |
| 27 | Roman Rd / Globe Rd | N/A | 92 | - | - | 36 | 34 | 26 | 27 | 24 |
| 28 | Bonner Road | N/A | 100 | 41 | 40 | 37 | 35 | 26 | 32 | 32 |
| 29 | Grove Rd / Old Ford Rd | N/A | 92 | 48 | 46 | 43 | 40 | 28 | 27 | 27 |
| 30 | Fieldgate Street | N/A | 92 | 48 | 42 | 46 | 38 | 28 | 28 | 29 |
| 31 | Whitechapel Market | N/A | 100 | 68 | 69 | 63 | 54 | 40 | 40 | 38 |
| 32 | Globe Rd / Mile End Rd | N/A | 100 | 54 | 52 | 48 | 42 | 30 | 30 | 31 |
| 33 | Stepney Green | N/A | 100 | 34 | 37 | 39 | 28 | 24 | 21 | 21 |
| 34 | Pitsea St / Commercial Rd | N/A | 92 | - | - | 37 | 35 | 26 | 26 | 25 |
| 35 | Narrow St Limehouse Link | N/A | 100 | - | - | 86 | 77 | 54 | 61 | 59 |
| 36 | Locksley St / St Paul's Way | N/A | 92 | 38 | 36 | 35 | 32 | 26 | 26 | 21 |
| 37 | Rhodeswell Rd | N/A | 100 | 39 | 36 | 34 | 30 | 27 | 25 | 24 |
| 38 | Ben Johnson Road | N/A | 100 | 45 | 44 | 36 | 36 | 29 | 28 | 27 |
| 39 | Harford St / Mile End Rd | N/A | 100 | 41 | 41 | 42 | 36 | 26 | 28 | 27 |
| 40 | Thoydon Rd | N/A | 100 | - | - | 36 | 33 | 24 | 24 | 24 |
| 41 | Ford Close / Roman Rd | N/A | 100 | 41 | 40 | 38 | 34 | 26 | 29 | 28 |
| 42 | Victoria Park Co-location site | N/A | 92 | 24 | 24 | 22 | 21 | 17 | 15 | 15 |
| 43 | Victoria Park Co-location site | N/A | 75 | 25 | 23 | 22 | 21 | 16 | 15 | 14 |
| 44 | Parnell Rd / Old Ford Rd | N/A | 92 | 41 | 42 | 35 | 34 | 28 | 28 | 28 |
| 45 | St Stephen's Rd / Tredegar Rd | N/A | 100 | 47 | 45 | 56 | 39 | 31 | 29 | 29 |
| 46 | Rhondda Grove / Mile End Rd | N/A | 83 | 41 | 37 | 48 | 33 | 26 | 24 | 24 |
| 47 | Wentworth Mews | N/A | 100 | 51 | 46 | 48 | 41 | 32 | 32 | 29 |
| 48 | Ackroyd Drive | N/A | 92 | 44 | 44 | 38 | 37 | 32 | 30 | 29 |
| 49 | Dod St / Burdett Rd | N/A | 92 | 38 | 38 | 33 | 30 | 25 | 22 | 21 |
| 50 | Rich Street | N/A | 92 | 45 | 42 | 42 | 35 | 29 | 30 | 29 |
| 51 | Watney Market | N/A | 100 | 37 | 34 | 33 | 29 | 23 | 24 | 23 |
| 52 | Wick Lane / Autumn St | N/A | 92 | 45 | 42 | 40 | 37 | 30 | 30 | 29 |
| 53 | Fairfield Road/Tredegar Road | N/A | 92 | 52 | 50 | 42 | 43 | 35 | 35 | 32 |
| 54 | Bow Rd / Glebe Terrace | N/A | 100 | 49 | 57 | 60 | 50 | 37 | 38 | 41 |
| 55 | TH Cemetery Park | N/A | 100 | 26 | 25 | 23 | 22 | 17 | 17 | 17 |
| 56 | Bow Common Lane / St Paul's Way | N/A | 92 | 43 | 40 | 37 | 32 | 26 | 25 | 25 |
| 57 | Augusta St / Girauld St | N/A | 100 | - | - | 28 | 27 | 23 | 22 | 21 |
| 58 | Dolphin Lane | N/A | 83 | 36 | 32 | 29 | 28 | 23 | 22 | 22 |
| 59 | Westferry Road / Limehouse Link Jnct | N/A | 83 | 39 | 40 | 37 | 31 | 28 | 28 | 27 |
| 60 | Cascades, Westferry Road | N/A | 100 | 45 | 41 | 39 | 36 | 34 | 32 | 31 |
| 61 | Bow Rd / Alfred St | N/A | 100 | 44 | 41 | 35 | 35 | 28 | 25 | 25 |
| 62 | Mast House Terrace | N/A | 100 | 35 | 34 | 29 | 32 | 27 | 27 | 25 |
| 63 | Millwall Park | N/A | 83 | 29 | 26 | 22 | 24 | 21 | 20 | 19 |
| 64 | Lime harbour | N/A | 100 | 42 | 40 | 38 | 37 | 36 | 37 | 33 |
| 65 | Manchester Road/East Ferry Road | N/A | 75 | 34 | 32 | 28 | 29 | 25 | 23 | 23 |
| 66 | Millwall Park | N/A | 58 | 30 | 29 | 25 | 22 | 18 | 20 | 20 |
| 67 | Seyssel Street | N/A | 100 | 34 | 34 | 30 | 31 | 27 | 26 | 25 |
| 68 | Manchester Road / Ollife Street | N/A | 100 | 34 | 33 | 32 | 34 | 26 | 27 | 25 |
| 69 | Lawnhouse Close | N/A | 100 | 41 | 41 | 34 | 31 | 27 | 26 | 24 |
| 70 | Admirals Way | N/A | 92 | - | - | 27 | 29 | 24 | 22 | 23 |
| 71 | Toynbee St / Commercial St | N/A | 100 | - | - | 54 | 45 | 35 | 33 | 32 |
| 72 | Prestons Road / Coldharbour | N/A | 100 | 39 | 40 | 39 | 38 | 30 | 31 | 28 |
| 73 | John Smith Mews | N/A | 100 | 38 | 40 | 32 | 31 | 25 | 26 | 22 |
| 74 | Poplar High St / Cotton St | N/A | 100 | - | - | 64 | 71 | 59 | 55 | 56 |
| 75 | Hale Street | N/A | 100 | 33 | 34 | 34 | 29 | 24 | 24 | 22 |
| 76 | Chrisp Street / E India Dock Road | N/A | 92 | 48 | 49 | 45 | 39 | 33 | 33 | 31 |
| 77 | Morris / Barchester Street | N/A | 100 | 39 | 40 | 37 | 31 | 26 | 23 | 22 |
| 78 | Devons Road / Campbell Road | N/A | 100 | 48 | 47 | 43 | 36 | 32 | 31 | 30 |
| 79 | Hatfield Terrace / Fairfield Road | N/A | 100 | 31 | 33 | 32 | 28 | 24 | 25 | 23 |
| 80 | Wrexham Road | N/A | 100 | 41 | 40 | 38 | 35 | 28 | 30 | 26 |
| 81 | Bromley High Street / St Leonards | N/A | 100 | 39 | 38 | 38 | 34 | 29 | 31 | 27 |
| 82 | Devas Street / Devons road | N/A | 100 | 50 | 48 | 45 | 37 | 29 | 32 | 29 |
| 83 | Zetland Street / A12 | N/A | 100 | 63 | 62 | 63 | 52 | 41 | 43 | 41 |
| 84 | Blair Street (End of Street) | N/A | 100 | 48 | 52 | 44 | 39 | 36 | 32 | 30 |
| 85 | Portree Street | N/A | 92 | 48 | 48 | 45 | 38 | 34 | 34 | 32 |
| 86 | Newport Avenue | N/A | 100 | 34 | 33 | 30 | 28 | 22 | 25 | 23 |
| 87 | Mile End Road Corner Bancroft Rd | N/A | 92 | - | - | 49 | 37 | 31 | 30 | 28 |
| 88 | Shirbutt St o/s Holy Family School | N/A | 100 | - | - | 28 | 26 | 21 | 21 | 20 |
| 89 | Thames Path Storers Quay | N/A | 100 | 30 | 29 | 26 | 26 | 23 | 22 | 22 |
| 90 | Sextant Avenue | N/A | 100 | 28 | 28 | 25 | 24 | 20 | 21 | 19 |
| 91 | At the entrance of MOT station | N/A | 83 | - | - | - | - | - | 31 | 23 |
| 92 | At the exit of MOT station | N/A | 83 | - | - | - | - | - | 39 | 28 |

Notes:

The annual mean concentrations are presented as μg m-3.

Exceedances of the NO2 annual mean AQO of 40 μg m-3 are shown in bold.

NO2 annual means in excess of 60 μg m-3, indicating a potential exceedance of the NO2 hourly mean AQS objective are shown in bold and underlined.

Means for diffusion tubes have been corrected for bias.

All means have been “annualised” in accordance with LLAQM Technical Guidance if valid data capture for the calendar year is less than 75% and greater than 25%.

Results have been distance corrected where applicable.

(a) Data capture for the monitoring period, in cases where monitoring was only carried out for part of the year.

(b) data capture for the full calendar year (e.g. if monitoring was carried out for six months the maximum data capture for the full calendar year would be 50%).

Commentary: Automatic Monitoring Sites

In 2022, data capture rate of more than 75% was achieved at all nitrogen dioxide automatic monitoring stations except for Blackwall automatic monitoring site, where annual data capture was 62%. Therefore, data have been “annualised” in accordance with LLAQM Technical Guidance.

In 2021, the automatic monitoring sites of Blackwall and Victoria Park demonstrated an overall reduction in annual mean NO2 concentration from previous years. However, still in 2021, there was no changes at Millwall Park site, and there was a minimal increase in annual mean NO2 concentration at Mile End site.

In 2022, data shows a reduction at roadside sites (Mile End and Blackwall), but a slight increase in the background sites (Victoria Park and Millwall Park) annual mean NO2 concentrations, still showing relatively low concentrations at background locations and below the UK legal limits for NO2.

Blackwall site, in 2022, had annual mean NO2 concentration of 28μg m-3. Compared to 59μg m-3 in 2016, therefore, the last 7 years has seen a significant reduction.

Similarly, Mile End site, in 2022, had annual mean NO2 concentration of 23μg m-3, compared to 52μg m-3 in 2016, showing a significant reduction.

In 2022, annual mean NO2 concentrations at all monitoring sites were below the National Objective of 40µg/m³, even on roadside monitors, which show a decline in levels of NO2. There has been a significant reduction at both the Mile End and Blackwall roadside sites since 2016.

Reasons for the air quality improvement could be linked to the ULEZ (Ultra Low Emission Zone) expansion, NRMM conditions in place, requirements to monitor for construction dust at construction sites (construction dust conditions), air quality / transport / highways projects, implementations of the AQAP actions, etc.

Figure 1 below shows the trend in annual mean NO2 concentrations over the last 7 years  at the automatic monitoring sites in comparison with the UK legal limit.

Figure 1. Annual mean NO2 results at automatic monitoring sites 2016-2022.

Commentary: Non-Automatic Monitoring Sites (NO2 diffusion tubes)

Monitoring results for the 92 diffusion tubes were bias adjusted using the national bias-adjustment factor. The reasons for choosing the national bias-adjustment over the local bias-adjustment is given in the section ‘Discussion of Choice of Factor to Use’.

Raw monthly data for all diffusion tube sites can be found in Table O (NO2 Diffusion Tube Results) at the end of this report.

The minimum data capture rate of 75% was not achieved at the following 6 monitoring sites:

- Site ID 3 Bethnal Green Rd/ Brick Lane, data capture 67%

- Site ID 4 Commercial St/Calvin St, data capture 67%

- ID 7 St Katherine's Way, data capture 33%

- Site ID 10 Whitechapel Rd/Adler St, data capture 67%

- Site ID 25 Cavell St/Stepney Way, data capture 67%

- Site ID 66 Millwall Park, data capture 58%

Therefore, it has been necessary to annualise the results of these sites in accordance with the procedure described in LAQM TG (19) (see Appendix A3 ‘Adjustments to the Ratified Monitoring Data’ in this report).

In 2022, of the 92 diffusion tubes monitoring sites in the borough, there were 4 exceedances of the NO2 annual mean of 40 μg m-3 and there was noNO2 annual means more than 60 μg m-3, thus indicating a significant air quality improvement within the borough.

Monitoring data from all sites show a downward trend compared to 2016. Comparison to 2021 show for most sites (except for ten)  a minor reduction in NO2 annual mean concentration. Some sites also show   no change in the NO2 level measured with the 2021 levels.. Ten sites however show a modest increase in  NO2 annual mean  of about 1-3 μg m-3 compared to 2021.

Reasons for the air quality improvement could be linked to the ULEZ (Ultra Low Emission Zone) expansion, NRMM conditions in place, requirements to monitor for construction dust at construction sites (construction dust conditions), Council air quality / transport / highways projects, implementations of the AQAP actions, etc.

Figures 2 -4 below show trend in NO2 diffusion tube annual mean ratified and bias-adjusted monitoring results at all 92 sites over the last 7 years and how they compare with the UK legal limits.

Figure 2. NO2 diffusion tube annual mean ratified and bias-adjusted monitoring results at sites 1- 30 from 2016 to 2022.

Figure 3. NO2 diffusion tube annual mean ratified and bias-adjusted monitoring results at sites 31- 60 from 2016 to 2022.

Figure 4. NO2 diffusion tube annual mean ratified and bias-adjusted monitoring results at sites 61- 92 from 2016 to 2022.

Table E.	NO2 Automatic Monitoring Results: Comparison with 1-hour Mean Objective, Number of 1-Hour Means > 200μgm-3

|  |  |  |  |  |  | Number of Hourly Means > 200μg m-3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Site ID | Valid data capture for monitoring period % a | Valid data capture 2022 % b | 2016 | 2017 | 2018 | 2019 | 2020 | 2021 | 2022 |  |
| TH2 Mile End | N/A | 99 | 0 | 2 | 0 | 1 | 0 | 0 | 0 |  |
| TH004 Blackwall | N/A | 62 | 9 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| TH002 Victoria Park | N/A | 98 | 0 | 24 | 1 | 0 | 0 | 0 | 0 |  |
| TH001 Millwall Park | N/A | 78 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |

Notes

Results are presented as the number of 1-hour periods where concentrations greater than 200 μg m-3 have been recorded.

Exceedance of the NO2 short term AQO of 200 μg m-3 over the permitted 18 hours per year are shown in bold.

If the period of valid data is less than 85%, the 99.8th percentile of 1-hour means is provided in brackets.

(a) Data capture for the monitoring period, in cases where monitoring was only carried out for part of the year

(b) Data capture for the full calendar year (e.g. if monitoring was carried out for six months the maximum data capture for the full calendar year would be 50%)

Commentary

For the past three years (2020, 2021, and 2022), there were no exceedances of the 1-hour mean, 200μg m-3 at any of the automatic monitoring site.

In 2019, there was only 1 exceedance at Mile End automatic monitoring site, although significantly below the permitted 18 days per year.

Also In 2018, there was only 1 exceedance at Victoria Park automatic monitoring site, again significantly below the permitted 18 days per year.

Table F.	Annual Mean PM10 Automatic Monitoring Results (μg m-3)

|  |  |  |  |  |  | Annual Mean Concentration (μg m-3) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Site ID | Valid data capture for monitoring period % a | Valid data capture 2022 %b | 2016 | 2017 | 2018 | 2019 | 2020 | 2021 | 2022 |
| TH004 Blackwall | N/A | 19 | 23 | 25 | 20 | 20 | 17 | 18 | non-representative |
| TH001 Millwall Park | N/A | 60 | 17 | 20 | 18 | 18 | 18 | 18 | 13 |
| TH002 Victoria Park | N/A | 89 | 16 | 17 | 18 | 18 | 17 | 16 | 16 |

Notes

The annual mean concentrations are presented as μg m-3.

Exceedances of the PM10 annual mean AQO of 40 μg m-3 are shown in bold.

All means have been “annualised” in accordance with LLAQM Technical Guidance, if valid data capture is less than 75% and more than 25%.

(a) Data capture for the monitoring period, in cases where monitoring was only carried out for part of the year.

(b) Data capture for the full calendar year (e.g. if monitoring was carried out for six months the maximum data capture for the full calendar year would be 50%).

Commentary

Annual mean PM10 concentrations at all monitoring sites have been significantly below the National Objective of 40µg/m³ since 2016.

In 2022, Blackwall PM10 data were non-representative of the full year and could not be “annualised” in accordance with LLAQM Technical Guidance, as the valid annual data capture was less than 25%.

Millwall Park site has shown a slow downward trend of annual mean PM10 from 2016.

The Victoria Park site, since 2016, has shown an almost consistent trend of annual mean PM10 (between 18 and 16 μg m-3. Figure 5 below shows annual mean PM10 results at the automatic monitoring sites with a comparison with the UK legal limit.

Reasons for the air quality improvement could be linked to the ULEZ (Ultra Low Emission Zone) expansion, NRMM conditions in place, requirements to monitor for construction dust at construction sites (construction dust conditions), Council air quality / transport / highways projects, implementations of the AQAP actions, etc.

Figure 5. Annual Mean PM10 Automatic Monitoring Results (μg m-3).

Table G.	PM10 Automatic Monitoring Results: Comparison with 24-Hour Mean Objective, Number of PM10 24-Hour Means > 50 μg m-3

|  |  |  |  |  |  | Number of Daily Means > 50μg m-3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Site ID | Valid data capture for monitoring period % a | Valid data capture 2022 % b | 2016 | 2017 | 2018 | 2019 | 2020 | 2021 | 2022 |
| TH004 Blackwall | N/A | 19 | 10 | 10 | 10 | 8 (35) | 4 | 0 (27.1) | non-representative |
| TH001 Millwall Park | N/A | 60 | 0 (27.9) | 8 | 1 | 7 (30) | 7 | 5 | 4 (31) |
| TH002 Victoria Park | N/A | 89 | 3 (28.6) | 2 | 1 | 7 | 5 | 1 | 3 |

Notes

Exceedances of the PM10 24-hour mean objective (50 μg m-3 over the permitted 35 days per year) are shown in bold.

Where the period of valid data is less than 85% of a full year, the 90.4th percentile is provided in brackets.

(a) data capture for the monitoring period, in cases where monitoring was only carried out for part of the year

(b) data capture for the full calendar year (e.g. if monitoring was carried out for six months the maximum data capture for the full calendar year would be 50%).

Commentary

In 2022, the number of PM10 24-hour means over 50μg m-3 were significantly below the national air quality objective level (permitted 35 days per year) at the Millwall Park and Victoria Park monitoring sites.

In the Blackwall monitoring site, data capture was only 19% in 2022 due to pump failure and pump parts supply issues in early part of year, and flow sensor issues later in year. Therefore, Blackwall PM10 data were non-representative of the full year and could not be “annualised” in accordance with LLAQM Technical Guidance, as the valid annual data capture was less than 25%. From 2016 to 2021, data suggest that there was a significant decrease in the number of PM10 daily mean values over 50μg m-3.

At Millwall Park site, from 2017 to 2020, data showed a slight increase in the number of daily mean values exceeding 50μg m-3. However, from 2021, the number of exceedances decreased.

AT Victoria Park site, in 2019 and 2020, there was a slight increase in the number of daily mean values exceeding 50μg m-3 (7 and 5 exceedances respectively . In 2021, at Victoria Park site there was only 1 exceedance, and 3 exceedances in 2022, still significantly below the national air quality objective level.

Table H.	Annual Mean PM2.5 Automatic Monitoring Results (μg m-3)

|  |  |  |  |  |  | Annual Mean Concentration (μg m-3) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Site ID | Valid data capture for monitoring period % a | Valid data capture 2022 % b | 2016 | 2017 | 2018 | 2019 | 2020 | 2021 | 2022 |
| TH2P Mile End | N/A | 91 | - | - | - | 10 | 12 | 11 | 9 |
| TH004 Blackwall | N/A | 46 | 20 | 13 | 13 | 12 | 9 | 11 | 8 |
| TH002 Victoria Park | N/A | 99 | - | - | - | 10 | 12 | 9 | 9 |

Notes

The annual mean concentrations are presented as μg m-3.

Exceedances of the PM2.5 annual mean AQO of 20 μg m-3 are shown in bold.

All means have been “annualised” in accordance with LLAQM Technical Guidance, if valid data capture is less than 75% and more than 25%.

(a) Data capture for the monitoring period, in cases where monitoring was only carried out for part of the year.

(b) Data capture for the full calendar year (e.g. if monitoring was carried out for six months the maximum data capture for the full calendar year would be 50%).

Commentary

There is now a focus on monitoring PM2.5 as this pollutant according to the WHO can be most damaging to human health. Therefore, the Council In 2019, installed two PM2.5 monitors into the existing monitoring site locations of Victoria Park (TH002), and Mile End Road (TH2P).

Annual Mean PM2.5 concentrations at all monitoring sites are below the national air Quality objective of 25 μg/m3 and shows a downward trend.

# Action to Improve Air Quality

2.1	Air Quality Action Plan Progress

Table J provides a brief summary of the London Borough of Tower Hamlets progress against the Air Quality Action Plan, showing progress made this year. New projects which commenced in 2022 are shown at the bottom of the table

Table I.	Delivery of Air Quality Action Plan Measures

| Measure | Action | Progress - Emissions/Concentration data - Benefits - Negative impacts / Complaints | Further information |
| --- | --- | --- | --- |
| 1. | Develop and implement a communications strategy for disseminating air quality information in the borough to raise awareness of the impacts of poor air quality and encourage behaviour change. | Ongoing. (1) Strategy was proposed and developed. AQ Partnership Board Meeting in Dec 2017: a presentation was made by the Comms Team. March 2020 meeting: the ‘Breath Clean’ Strategy’ was outlined. ‘Breathe Clean’ Strategy’s Objectives: - To define the School Streets project committed in the Mayor’s Manifesto, and to improve air quality at the school gate, particularly at entry /exit times, in order to improve the health of the pupils. - To improve road safety for vulnerable pedestrians, reducing congestion, encouraging walking and cycling and thus facilitating behavioural change in travel choices and increased physical activity. - To introduce school streets at 50 primary schools in the borough, outlining a proposed prioritisation methodology and identifying the first schools to be included in the programme. - To lobby Mayor of London and/or government for funding/changes to policy that will help Tower Hamlets improve local air quality. - The “Breathe Clean” Strategy will be implemented over 3 phases. - Phases 1 and 2 will raise awareness about air pollution and educate residents about what can be done to help reduce it locally and across London. - Phase 3 will lobby the government and the London Mayor for better support to reduce air pollution, and the negative health impacts it brings in Tower Hamlets (2) We also have the ongoing awareness raising campaign. The campaign needs to address two groups: (i) vulnerable people, and (ii) those who are causing air pollution – educating this second group is about bringing about behaviour change which is effective. (3) Anti-idling campaign / enforcement was carried out in 2022: - It included an anti-idling pledge pledge that generated over 100 sign ups to install no-idling signs around sensitive receptors - As part of activity idling action days were organised at local schools - Information leaflets on idling were created and content about idling and harmful impacts were shared across key Council channels including social media, resident e-newsletters, ‘Our East End’ magazine and the Council website See also Actions 10 and 63. (4) Several air pollution campaigns were delivered in 2022 on social media, on various sites and in Tower Hamlets Town Hall to raise awareness of air pollution and to promote behaviour change: wood burning and/or solid fuels campaign, Clean Air Day, Car Free Day, Play Street school events, no-idling campaign, living green walls installation in primary schools, air quality monitoring, airText service and air pollution alerts, AQAP public consultation, Breathe London Network, etc. See also Action 6. | https://www.towerhamlets.gov.uk/lgnl/environment_and_waste/environmental_health/pollution/air_quality/Air_quality_information_and_campaigns/Breathe_Clean.aspx https://www.towerhamlets.gov.uk/lgnl/environment_and_waste/environmental_health/pollution/air_quality/Air_quality_information_and_campaigns/Anti_idling.aspx https://www.airtext.info/ https://www.breathelondon.org/ |
| 2 | Director of Public Health to have responsibility for ensuring their Joint Strategic Needs Assessment (JSNA) has up to date information on air quality impacts on the population – Air Quality officer to be consulted on JSNA. | JSNA specific reference to air quality published in 2016. | Measure Completed – No further action See: https://www.towerhamlets.gov.uk/lgnl/health__social_care/joint_strategic_needs_assessme/joint_strategic_needs_assessme.aspx |
| 3. | Strengthening co-ordination with Public Health by ensuring that at least one public health specialist within the borough has air quality responsibilities outlined in their job profile. | Public Health embedded into Local Air Quality. Three officers in Public Health including the Director of Public Health have a specific remit to work on Air Quality and they sit on all steering groups and boards that discuss air pollution. The Air Quality Steering Board has been integrated within the Public Health driven Health and Wellbeing Board. | Measure Completed – No further action |
| 4. | Director of Public Health to sign off all new Air Quality Action Plans. | Air quality action plan signed off by Public Health in 2017. | Measure Completed – No further action |
| 5. | Support patients with heart and lung conditions by providing air quality advice to discharged patients, particularly vulnerable & those with heart/lung conditions. This would be a continuation of the ‘Protecting Patient’ work stream from the Barts Project. | Public Health funded Barts to conduct a Clean Air Hospital Framework in 2019, in order for them to develop an action plan to better consider opportunities to reduce air pollution and improve understanding of the harms of poor air pollution on health. Action plan is completed. Barts to present action plan to HWB. Public Health also sponsored the Tower Hamlets CCG to develop a work programme in partnership with Global Action Plan to improve awareness and understanding of the health harms of air pollution for children with asthma. An awareness campaign was launched in 2020 by the CCG. Barts ran Protecting Patients as a Pilot and decided not to continue the programme post evaluation. It did demonstrate some success in engagement clinicians on the health harms of air pollution. Revised Action: “Public Health to work with NHS Partners to support vulnerable patients, specifically, those with lung and heart conditions” Barts decided to not continue with the ‘Protecting Patient’ workstream in 2017. Public Health have since worked with Barts to develop a Clean Air Hospital Framework, to help them develop an action plan to reduce air pollution inside and outside the Trust as well as support and training for health professionals to improve health outcomes for vulnerable patients. Furthermore, Public Health have worked with the NHS to develop an innovative programme of work to raise awareness of the harms of air pollution for children with asthma, specifically, to influence how health professionals can assist children in management of their underlying health conditions. | No further action |
| 6. | Support and Promotion of air quality awareness programmes such as AirTEXT | In 2022, airText service was promoted on the Council's air quality webpage and also on social media by the Pollution Team and Comms, in order to raise awareness of air pollution. Annual airTEXT statistics for Tower Hamlets at the end of 2022: Number of subscribers; 396. Number of new subscribers; 35. Number of airText Tower Hamlets alert days; 22. Number of emails; 1,540. Number of sms; 3,518. Number of voicemails; 56. | AirText services is promoted on the Council's air quality webpage: https://www.towerhamlets.gov.uk/lgnl/environment_and_waste/environmental_health/pollution/air_quality/air_text_-_air_pollution_forec.aspx |
| 7. | Encourage schools to join the TfL Sustainable Travel Active Responsible Safer (STARS) accredited travel planning programme by providing information on the benefits to schools and supporting the implementation of such a programme. | The Council has encouraged and engaged with all schools in the borough to join the STARS programme. The programme runs from October to August, and all targets were reached at the end of the project in August 2022. A 67% target was achieved for the programme. |  |
| 8. | Air quality at schools – Roll out the cleaner air for schools’ program that was previously run in 2 schools, to more schools in high pollution areas. | Contract with Loop labs for Bonner School Citizen Science programme complete June 2018. | Measure Completed – No further action |
| 9. | Pollution Audits in schools. Support the GLA in their program to provide air quality audits in 2 primary schools. | Two schools participated in GLA audit scheme: Marner Primary School, and Bonner Primary School. Reports received in 2018. The GLA also undertook air quality audits in 2 nurseries: Alice Model Nursery School and Columbia Market Nursery School. Subsequent baseline air quality monitoring undertaken and report outlining recommendations provided from audit findings. Reports received in 2019. Tower Hamlets Pollution Team audited 3 primary schools in 2021. The air quality school audits were undertaken at: Canon Barnett, Stephen Hawking school, and Bigland Green. | Measure Completed |
| 10. | Schools anti-idling project | (1) Anti-idling signs: In 2022, we identified 3 primary schools located in areas exceeding NO2 national objective: Canon Barnett Primary School, Woolmore Primary School, and Culloden Primary School. We have installed no-idling signs not only around these 3 schools, but also around several primary schools within the Borough. (2) Anti-Idling Campaign: In 2022, Tower Hamlets continued to engage with schools through the Idling Action campaign, which ended in Spring 2022: - Air quality workshops were conducted at 2 schools - 2 Idling Action events took place - 93 students in total took part at the workshops - 13 students in total took part at Idling Action events. (3) In 2022, the Pollution Team carried out weekly no-idling enforcement / events within the Borough. (4) See also Action 1. | https://www.towerhamlets.gov.uk/lgnl/environment_and_waste/environmental_health/pollution/air_quality/Air_quality_information_and_campaigns/Breathe_Clean.aspx https://www.towerhamlets.gov.uk/lgnl/environment_and_waste/environmental_health/pollution/air_quality/Breathe_Clean/Report_engine_idling.aspx https://www.towerhamlets.gov.uk/lgnl/environment_and_waste/environmental_health/pollution/air_quality/Air_quality_information_and_campaigns/Anti_idling.aspx |
| 11. | Schools Environmental Theatre Project | Big Wheel Theatre company ran events on 20 primary schools in 2018. | Measure Completed – No further action |
| 12. | Investigate and invest in new technology as it becomes available to reduce pollution levels at pollution hotspots & sensitive uses e.g. schools | No new effective technology has become available. |  |
| 13. | Citizen Science air quality monitoring project | Two projects were funded by the Tower Hamlets Mayor’s Air Quality Fund (2018/19): (1) Project Title – ‘Clear the Air’ Description of project: The project provided AQ monitoring training and community organising training to LBTH residents from disadvantaged groups to bring the community together and to raise awareness of pollution and exposure. Residents put up NO2 diffusion tubes to understand their exposure. Children from a local school created a video about air pollution which was launched on ‘Clean Air Day 2019’. (2) Project Title – ‘Air Aware at Bromley-by-Bow’ Description of project - This project engaged members of the Bromley-by-Bow community to raise awareness of air pollution, empowering them to reduce their exposure and safeguard their health while protecting the environment. By working with health practitioners at the Bromley-by-Bow Health Centre to identify patients with respiratory conditions, the project targeted those who are most vulnerable to air pollution. A series of air pollution workshops for BBBC patients and clients using a citizen science approach were delivered. A small team of Clean Air Champions were recruited and trained to use NO2 and particulate monitors to measure pollution in the local area. The group analysed the data, communicated their findings and co-designed effective solutions to reduce exposure. An action plan was developed to broaden the impact and share the knowledge with the wider community. | Measure Completed – No further action |
| 14. | Work with Residential Providers to develop and implement a strategy for disseminating air quality information to their tenants. | Presentation to LBTH Housing Forum used to agree that all housing providers will cascade our messages and opportunities for residents through their regular newsletters / social media and notice boards (the Council will coordinate and cascade all messages). Liaison with housing providers to schedule publicity. | Measure Completed – No further action |
| 15. | Use Health and Wellbeing Board to get existing and future public sector and RP partners to pledge to increase the number of, electric, hybrid, and cleaner vehicles in their fleets. | The Air Quality Partnership Board was amalgamated with the Health and Wellbeing Board in 2019. No actions in 2022. | In progress |
| 16. | Continue to run the 3 continuous monitoring stations, monitoring pollutants of concern to ensure air quality objectives are being met and to assess the effectiveness of local and regional policies. | 4 continuous monitoring stations are operating and maintained in the Borough: - Mile End (TH2): NO2, NO, NOx, PM2.5 - Blackwall (TH004): NO2, NO, NOx, PM2.5, PM10, O3 - Victoria Park (TH002): NO2, NO, NOx, PM2.5, PM10 - Millwall Park (TH001): NO2, NO, NOx, PM2.5, PM10, O3 | Data are available in this ASR and online: https://www.airqualityengland.co.uk/local-authority/?la_id=210 |
| 17. | Continue to implement the NOx Diffusion Tube Monitoring network across the borough. Investigate and implement further monitoring where necessary. E.g. at schools. | Borough-wide diffusion tube monitors maintained. . NOx tube raw data are loaded onto Council website monthly. | Data are available in the ASR and online: https://www.towerhamlets.gov.uk/lgnl/environment_and_waste/environmental_health/pollution/air_quality/Advanced_information_on_air_quality/Monitoring.aspx |
| 18. | Continue to ensure that all pollution monitoring data is available to the public and the website is regularly updated with the latest available data. | Ongoing work with Communications Team internally to disseminate information, arrange and agree street and school signs, updates to Breathe Clean message, etc. LBTH website updated with latest AQ monitoring results: both monthly diffusion tube data, and AQ continuous monitoring data are entered onto LBTH website. Data are available in the ASR and online, on Council website. | LBTH website with monitoring all results: https://www.towerhamlets.gov.uk/lgnl/environment_and_waste/environmental_health/pollution/air_quality/Advanced_information_on_air_quality/Monitoring.aspx |
| 19. | Fulfil the GLA’s criteria to retain our Cleaner Air Borough Status each year | Cleaner Air Borough (CAB) award retained in 2019. |  |
| 20. | Ensuring emissions from demolition and construction are minimised via planning applications reviews and conditions attached to planning permissions requiring Construction Environmental Management Plans, including dust mitigation and monitoring and Travel Plans encouraging sustainable travel for site workers | Ongoing. Major and minor planning applications reviewed by LBTH Environmental Health Department (Pollution Team) in respect to air pollution and air quality. Pollution Team providing air quality comments and recommending relevant conditions. Adopted Policy: Local Plan 2013: DM9 (Improving Air Quality)/ Requires an Air Quality Assessment to demonstrate how it will prevent and reduce air pollution during construction and demolition. New Local Plan: D.ES2 (Air Quality). Development is required to meet or exceed air quality neutral and consider impacts of pollution during construction and operation of the Proposed Development. New Local Plan: D.TR4 (Sustainable Delivery and Servicing). Development that generates a significant number of vehicle trips for goods or materials during its construction and/or operational phases is required to demonstrate sustainable transport). New Local Plan: S.TR1 (Sustainable Travel). Travel choice (including connectivity and affordability) and sustainable travel will be improved within the borough and to other parts of London, and beyond. Development will therefore be expected to prioritise the needs of pedestrians and cyclists as well as access to public transport, including river transport, before vehicular modes of transport. New Local Plan Policy D.TR2 (Impacts on the Transport Network): Major development and any development that is likely to have a significant impact on the transport network will be required to submit a transport assessment or transport statement as part of the planning application. | New Local Plan: https://www.towerhamlets.gov.uk/lgnl/planning_and_building_control/planning_policy_guidance/Local_plan/local_plan.aspx Air Quality and Planning: https://www.towerhamlets.gov.uk/lgnl/environment_and_waste/environmental_health/pollution/air_quality/Advanced_information_on_air_quality/Air-Quality-and-Planning.aspx See details on Table J ‘Planning requirements met by planning applications in Tower Hamlets in 2022’ of this ASR. |
| 21. | Ensuring all major developments adhere to the GLA’s Non Road Mobile Machinery Low Emission Zone. I.e. All NRMM used on site must meet the emissions standards stated in the GLA’s Control of Dust and Emissions during Demolition and Construction SPG 2014 (or subsequent updated guidance) | Major planning applications reviewed by LBTH Environmental Health Department (Pollution Team) in respect to air pollution and air quality. Pollution Team providing air quality comments and recommending relevant conditions. Relevant NRMM conditions recommended for each major site during planning consultations. Monthly planning decisions forwarded to LB Merton to feed into the London wide NRMM scheme for their inspections on follow up. CIL Tracker of when projects commence on site now also sent to the Pollution Team, from Planning. New Local Plan: D.SG4 (Planning and Construction of a New Development), require construction to comply with NRMM low emission zone requirements and minimize air quality and dust pollution. | LBTH is part of the MAQF funded pan London NRMM enforcement project led by LB Merton. LBTH officers contribute to the scheme by providing regular site information. See details on Table J ‘Planning requirements met by planning applications in Tower Hamlets in 2022’ of this ASR. |
| 22. | Ensuring Combined Heat and Power (CHP) and biomass air quality policies are met at all developments proposing to utilise CHP, including the NOx emission limits for heating plant as stated in the GLA’s Sustainable Design and Construction SPG (or subsequent updated guidance). | Major planning applications reviewed by LBTH Environmental Health Department (Pollution Team) in respect to air pollution and air quality. Pollution Team providing air quality comments and recommending relevant conditions. Planning applications reviewed for CHP and relevant conditions recommended. Comments provided on major planning applications as required by GLA SPG on Sustainable Design and Construction. | See details on Table J ‘Planning requirements met by planning applications in Tower Hamlets in 2022’ of this ASR. |
| 23. | Ensuring new developments have suitable energy efficiency measures installed to reduce the demand for onsite heat generation from boilers & CHP’s. | 100% of schemes were policy compliant through either onsite reduction or meeting carbon reduction requirements through utilising the carbon offsetting mechanism with a S106 contribution. The new AMR (Annual Monitoring Report) is now available. |  |
| 24. | Ensuring Air Quality Neutral policies are complied with at all developments and exceeded (improved) where possible. Ensure all larger developments (as defined by the GLA) will be air quality neutral | Ongoing. Major planning applications reviewed by LBTH Environmental Health Department (Pollution Team) in respect to air pollution and air quality. Pollution Team providing air quality comments and recommending relevant conditions. Air quality neutral policies adopted. Planning applications for major developments reviewed to ensure compliance with GLA air quality neutral policy. New Local Plan: D.ES2 (Air Quality). Development is required to meet or exceed air quality neutral and consider impacts of pollution during construction and operation of the Proposed Development. | New Local Plan: https://www.towerhamlets.gov.uk/lgnl/planning_and_building_control/planning_policy_guidance/Local_plan/local_plan.aspx |
| 25. | Reduce the use of private cars by residents by encouraging car free developments and limiting number of parking spaces in new developments. | Local Plan policy D.TR3 sets out a permit-free policy for new developments, encourages priority for cycle parking over car parking, and sets parking standards that minimise the amount of car parking spaces allowed in most parts of the borough. At most, a 3+ bedroom house in an area of PTAL 1-2 would be allowed 0.5 parking spaces – all other developments would be allowed fewer spaces than this. | New Local Plan: https://www.towerhamlets.gov.uk/lgnl/planning_and_building_control/planning_policy_guidance/Local_plan/local_plan.aspx |
| 26. | Ensure the layout of new developments considers air quality impacts, for example considering the locations of buildings with different proposed uses and locating the most sensitive use units in the least polluted areas | - Ongoing. All relevant major planning applications reviewed and assessed for compliance to the National Air Quality Strategy and the AQ objectives. AQ planning conditions reviewed and agreed between Planning and AQ Teams. Adopted in June 2021. |  |
| 27. | Ensuring adequate, appropriate, and well located green space and infrastructure, including for walking and cycling, is included in new developments with the Green Grid Strategy promoted and adhered to in all major planning applications and master planning to provide low emissions routes for walking and cycling. | The Green Grid Strategy formed part of evidence base for the new Local Plan. An SPD is currently being drafted to clarify the role and responsibilities towards the Green Grid within planning. The Green Grid Strategy identifies opportunities to improve connectivity between open spaces and community facilities to create a grid of walking routes and associated green infrastructure which provides a healthy and attractive environment for users. Strategic elements or projects for the Green Grid will be delivered through the Community Infrastructure Levy, including the healthy streets projects. The new Local Plan was adopted on 15th January 2020. Implementation of the Health Impact Assessment (HIA) policy within the Local Plan: HIA guidance now published and available on LBTH website. Improving HIA practice to aid consideration of adequate green spaces and low emission routes in planning applications: - Ongoing. - Further training delivered in August 2021. Development Management Crib Sheet sent to officers to aid implementation of HIA Policy, on the ending on the full time HIA Officer post. - Planning Department carried out an internal survey to gauge the understanding of our officers implementing the HIA policy, and identified any further training needs. - HIA Training took place in February 2021 to provide training to planning officers and across Place. A further training delivered in August 2021. - A one-day-a-week role for a HIA officer commenced in November 2022, and ended in March 2023?. A HIA drop-in session was arranged for planners, to build their capacity reviewing HIAs when the post comes to an end. | Green Grid Strategy: https://www.towerhamlets.gov.uk/Documents/Planning-and-building-control/Strategic-Planning/Local-Plan/Green_Grid_Update_2017.pdf Green Grid Strategy for City Fringe and other areas: https://www.towerhamlets.gov.uk/Documents/Planning-and-building-control/Strategic-Planning/Local-Plan/Green_Grid_Update_Appendicies_2017.pdf Local Plan and HIA available at Council website: https://www.towerhamlets.gov.uk/lgnl/planning_and_building_control/planning_applications/Making_a_planning_application/Local_validation_list/Health_Impact_Assessment.aspx |
| 28. | Encourage new developments to install alternative mass waste collection systems, such as ENVAC, to reduce collection vehicle emissions. | The new ‘Reuse, Recycling and Waste’ SPD was adopted in July 2021, after consultation and Cabinet approval. | Measure completed |
| 29. | Ensuring that the whole borough Smoke Control Zones is fully publicised and enforced. | Complaints/investigation records maintained and updated on APP database. Regulatory controls in place and investigated/enforced reactively through complaint investigation. Publicity and campaign about wood burning for smoke control area over winter months: 1) The Pollution Team updated the Council air quality webpage including new wood burning information. 2) The Pollution Team also liaised with Communications team to raise awareness on social media about health issues caused by wood burning and useful information (particularly Defra approved wood burners). A best practice leaflet aimed at canal boat owners has been produced and promoted via Council's webpage, social media, CRT and relevant boating associations and fortnightly engagement with boat owners during the winter months | Details available online at LBTH website: https://www.towerhamlets.gov.uk/lgnl/environment_and_waste/environmental_health/pollution/air_quality/Air_pollution/Smoke_control.aspx |
| 30. | Implement a Domestic boiler refit project using the GLA’s RE:FIT energy efficiency retrofit programme. | The Boiler Replacement Programme: Phase 1: - Delivered - Phase 1, which had a budget of £250,000, has supported more than 80 households to install new energy efficient boilers. Phase2: - Phase 2 had funding of £600,000. Phase 2 of the boiler replacement scheme ended in August 2022, and had 83 boiler installations. See Action 32. |  |
| 31. | Implement a Schools Carbon Emissions Reduction Programme providing funding towards boiler replacement and insulation schemes in schools. | Schools Carbon Emissions Reduction Programme organised in three phases. Phase I: completed in November 2019. Nine schools received grants for energy efficiency measures: Olga Primary, Osmani Primary, Cubitt Town Infants, Cubitt Town Juniors, Lansbury Lawrence, Lawdale Juniors, Wellington Primary, Oaklands, Arnhem Wharf. Phase II: This phase of the Programme has now had 8 of the 8 projects completed. The final school completed their project in May 2023 due to issues with the contractor. This phase of the programme is now complete. Phase 2021: All the projects are complete: 7 of them in 2022 and 1 in Feb 2023 |  |
| 32. | Implement a Carbon Emissions Reduction Programme for Council properties including boiler replacements and insulation projects. | As part of our net zero carbon Council by 2025 target we have developed an action plan which includes Council assets, we are preparing bids for external funding. We are already delivering the boiler replacement and insulation programme for residents. The Council was successful in receiving a £721K grant to replace gas boilers with air source heat pumps at 4 buildings - Overland's & Mowlem's Children Centres, Toby Lane Depot and Jack Dash House. All the works apart from at Jack Dash have been completed. The works at Jack Dash were completed in 2022. Phase 2 of the boiler replacement scheme ended in August 2022, and had 83 boiler installations. Although the budget was significantly larger than phase 1, we did not have access to additional funding which helped pay towards a large amount of applications from phase 1, so in reality phase 1 cost a lot more than the budget we had available. Also phase 2 was significantly affected by covid and the suspension to the programme. |  |
| 33. | Enderby Wharf – Ensure a thorough and robust evaluation of the Environmental statement, that methodologies used comply with current guidance and that the project will not lead to any significant adverse air quality impacts in the borough. | Application for cruise terminal withdrawn by developers. | Measure Completed – No further action |
| 34. | Ensure applications for new developments in neighbouring boroughs that have the potential to have impacts in Tower Hamlets are reviewed for air quality impacts and that no development will lead to any significant adverse air quality impacts in the borough. | Major and minor planning applications reviewed by LBTH Environmental Health Department (Pollution Team) in respect to air pollution and air quality. The Pollution Team reviews on a daily basis the APP database to monitor planning actions, providing information, air quality comments and recommending relevant conditions. |  |
| 35. | Lead by example by ensuring the Council’s new Civic Centre is a best practice example of a sustainable and low emissions development in regard to air pollution and CO2 with both air quality neutral and carbon zero policies being met. | New Council building planning application PA/17/02825/A1 approved, with conditions securing environmental information on: - Construction Environmental Management Plan (CEMP) - Full Travel Plan (to be approved in consultation with TfL) - BREEAM excellent target as a minimum, BREEAM outstanding to be targeted wherever possible - Demolition and Construction Dust monitoring procedure and reporting Approved details submitted with application: - AQ impact assessment submitted and approved, including AQ neutral assessment - Sustainability (carbon) and energy statement approved - Construction Logistics Plan approved - Transport Assessment approved ## The project timeline so far: - February 2015 – Tower Hamlets Council purchases Royal London Hospital. - November 2015 – Decision to transform the hospital into the new town hall formally announced - July 2016 – Allford Hall Monaghan Morris appointed as architects - November 2017 – Planning permission applied for - May 2018 – Planning permission and listing building consent granted. - December 2018 – Construction contract signed with Bouygues UK - February 2019 – Demolition and building works commenced. | Measure Completed – No further action New LBTH offices due to open 2022: https://www.towerhamlets.gov.uk/lgnl/council_and_democracy/New_town_hall/New_Town_Hall.aspx#:~:text=Opening%20in%202022%2C%20the%20new,accessible%20part%20of%20Tower%20Hamlets. |
| 36. | Improve the energy efficiency of John Onslow House as part of the upcoming refurbishment with the aim of becoming carbon zero and any new boilers to be ultra-low NOx. | New Remeha Quinta Pro and Vaillant Ecotec boilers installed. | Measure Completed – No further action |
| 37. | Ensure developments that will increase river traffic, in the operational phase of development, are thoroughly assessed for potential air quality impacts and will not have a significant negative impact on air quality. | Planning applications which may have an impact on air pollution on/from the River will be reviewed, including inter-borough applications.. |  |
| 38. | Ensure the Tideway Tunnel infrastructure project is sustainably delivered with the Construction Code of Practice adhered to and effective emissions mitigation in place during construction & operational phases. | LBTH Environmental Health Department (Pollution Team) reviews all Air Quality Monthly Monitoring Reports and provides comments and recommendations when necessary. |  |
| 39. | Silvertown Tunnel – Ensure a thorough and robust evaluation of the Environmental statement, that methodologies used comply with current guidance and that the project, during both the construction and operational phases, will not lead to any significant adverse air quality impacts in the borough and that adequate mitigation is provided for any potential impacts. Ensure traffic modelling on which the air quality statements are robust. | Development Consent Order granted May 2018. A review of the ES was carried out as part of the DCO planning process. Continuous air quality monitoring to be carried out during construction phase. |  |
| 40. | Ensure that all future major infrastructure projects are adequately reviewed and assessed through the planning process to ensure impacts on air quality are minimised. | No major infrastructure projects that are likely to effect LBTH are planned at present. Major planning applications reviewed by LBTH Environmental Health Department (Pollution Team) in respect to air pollution and air quality. Pollution Team providing air quality comments and recommending relevant conditions. New Code of Practice 2023 drafted as a joint teamwork by Planning, together with all other departments. |  |
| 41. | Ensure that Procurement policies to include a requirement for suppliers with large fleets to have attained, silver as a minimum or gold as a preference, Fleet Operator Recognition Scheme (FORS) accreditation or equivalent. | Actions 41, 42and 44 have been covered off in the single policy document. However, with the merging of Air Quality officer steering group and NZC (Net Zero Carbon), the policy has been reviewed again with a greater scope (more of a Social, Economic and Environmental an all-encompassing policy) to focus on assisting the Council with reaching the NZC status for 2025. Local authority Procurement policies have included (action completed in March 2022) a requirement for suppliers with large fleets (10 or more) to have attained silver FORS accreditation or equivalent. We have implemented changes to the procurement toolkit which dictates suitable accreditation for fleet operators along with a drive to reduce pollution. | Measure Completed – No further action |
| 42. | Investigate updating Procurement policies to ensure sustainable logistical measures are implemented (and include requirements for preferentially scoring bidders based on their sustainability criteria). | Actions 41, 42and 44 have been covered off in the single policy document. However, with the merging of Air Quality officer steering group and NZC (Net Zero Carbon), the policy has been reviewed again with a greater scope (more of a Social, Economic and Environmental an all-encompassing policy) to focus on assisting the Council with reaching the NZC status for 2025. Local authority Procurement policies have included (action completed in March 2022) a requirement for suppliers with large fleets (10 or more) to have attained silver FORS accreditation or equivalent. We have implemented changes to the procurement toolkit which dictates suitable accreditation for fleet operators along with a drive to reduce pollution. | Measure Completed – No further action |
| 43. | Investigate re-organisation of freight to support consolidation (or micro-consolidation) of deliveries, by setting up or participating in new logistics facilities, and/or requiring that Council suppliers participate in these. | With part-funding from GLA’s Business Low Emission Neighbourhood (BLEN) programme, the regeneration team are working on proposals to develop a market storage and consolidation site at Whitechapel. Feasibility and engagement work has been completed with a planning application anticipated during summer 2021. |  |
| 44. | Investigate implementing a local Eco Stars Fleet Recognition Scheme for Tower Hamlets. | Actions 41, 42and 44 have been covered off in the single policy document. However, with the merging of Air Quality officer steering group and NZC (Net Zero Carbon), the policy has been reviewed again with a greater scope (more of a Social, Economic and Environmental an all-encompassing policy) to focus on assisting the Council with reaching the NZC status for 2025. Local authority Procurement policies have included (action completed in March 2022) a requirement for suppliers with large fleets (10 or more) to have attained silver FORS accreditation or equivalent. We have implemented changes to the procurement toolkit which dictates suitable accreditation for fleet operators along with a drive to reduce pollution. | Measure Completed – No further action |
| 45. | Join a recognised appropriate driver award scheme, e.g. Fleet Operator Recognition Scheme (FORS) or Van Excellence & achieve certification. | We have joined FORS and have a working group forming to work to bronze accreditation. | Measure Completed – No further action |
| 46. | Increasing the number of, electric, hybrid, and cleaner vehicles in the boroughs’ fleet. | All waste vehicles are Euro VI diesel. All new vehicle leased are Euro VI, and the waste fleet has around 175 vehicles of cleaner fuels. Feasibility study carried out to understand infrastructure requirements for charging and associated cost. Currently attempting to secure funding. |  |
| 47. | Accelerate uptake of new Euro VI vehicles in borough fleet, ending the purchase of diesel vehicles where feasible. | All waste vehicles are Euro VI diesel. All new vehicle leased are Euro VI, and the waste fleet has around 175 vehicles of cleaner fuels. Feasibility study carried out to understand infrastructure requirements for charging and associated cost. Currently attempting to secure funding. |  |
| 48. | Real-time Telematics monitoring of fleet driver behaviour and subsequent driver training. | All waste vehicles are fitted with CMS Supatrak. Nearly all the fleet now has telematics fitted monitoring driver behaviour and fuel usage. Following the managerial restructure in Fleet and Waste Operations the development of telematics to monitor vehicle use and driver behaviour has been delayed. Work is required to develop and agree how we will implement this using the CMS Ecotrack vehicle telematics system we are using in 2022/2023. | https://www.supatrak.com/2021/08/06/discover-the-benefits-of-eco-coaching/ https://www.supatrak.com/driver-behaviour/ |
| 49. | Utilise round optimisation for Council fleet to reduce vehicle miles. | Route optimisation savings plan presented to Transport Demand Board: optimisation for waste operations, including waste, recycling and street cleansing, baseline modelling and options for change. Implementation of changes is expected by end quarter 3 - 2022/23. For all round efficient fleet optimisation: The Fleet and Depot Team will be reviewing plans to make use of vehicle tracking software to obtain base line data on vehicle usage and to review and develop plans for efficient driver training in 2022/23. Main focus of the Team is on roll out of electric vehicles from 2022 to March 2024. |  |
| 50. | Procure a cargo bike for regular delivery of literature to Councillors. | Parks Dept already have 2 cargo bikes in use in Victoria Park. Councillors now provided with tablets to facilitate delivery of literature. | Measure Completed – No further action |
| 51. | Project 2020: use the procurement process to ensure all waste & Recycling collection vehicles in the new contract are as low emission as possible by prioritising tenders with the highest proportion of low emission vehicles. | New recycling collection vehicles purchased in 2019 are all Euro VI. Waste management strategy approved Deliver vehicle replacement (Procurement comment; All vehicles produced and sold in the EU after September 2015 are euro 6 certified 'ULEZ compliant'). | Measure Completed – No further action |
| 52. | Project 2020: utilise round optimisation to reduce vehicle mileage for waste collections. | Round optimisation fleet movements for Refuse Collection Vehicles reviewed annually or whenever the service changes. Reduced mileage always sought by operations to save on fuel and services which directly influences fuel use. | Measure Completed – No further action |
| 53. | Reduce ‘Grey Fleet’ impacts by reviewing staff parking permits to reduce number or allocate shared team permits rather than individual. | Staff essential car user eligibility reviewed under terms and condition review. Commenced in 2020. | Measure Completed – No further action |
| 54. | Investigate installing Green Infrastructure, such as green walls, green screens or living roofs at schools/residential developments in polluted areas. Linking in with the Green Grid and Open Spaces Strategy. | Various installations of living green walls started in 2021 (Section 106 funds and LIF Project funds) at schools located in areas where nitrogen dioxide concentration levels exceed national objectives. Some installations completed in 2022, but then funding was paused: Cyril Jackson Primary school, Old Palace school, Bygrove Primary, and Stephen Hawking Primary School. Air Quality School Audits undertaken. | Measure completed |
| 55. | Low Emission Neighbourhoods (LENs) – implement the City Fringe LEN in partnership with Hackney and Islington. | Borough received funding for a business LEN in Whitechapel. | Measure Completed – No further action |
| 56. | Engagement with businesses – Continuation of the ZEN Project engaging businesses with advice and grants to enable them to reduce their air quality impact. | The ZEN Project has ended in March 2022. Tower Hamlets ZEN statistics for 2021: - 124 total members recruited (businesses and residents) - 70 measures completed - 23 grants awarded | https://www.towerhamlets.gov.uk/lgnl/environment_and_waste/environmental_health/pollution/air_quality/Air_quality_information_and_campaigns/Zero_Emissions_Network.aspx https://zeroemissionsnetwork.com/ |
| 57. | Discouraging unnecessary idling by taxis, coaches and other vehicles. Anti–Idling engagement project focusing on air pollution hotspots and high-risk locations such as hospitals and schools. | Weekly anti-idling enforcement visits in hotspot areas and idling patrol undertaken in 2022. For anti–idling engagement project focusing on schools, see Action 10 (Idling Action Campaign). Other no-idling events took place in 2022, such as Car Free events in September 2022, and idling awareness events with Lead Member. | https://www.towerhamlets.gov.uk/lgnl/environment_and_waste/environmental_health/pollution/air_quality/Breathe_Clean/Report_engine_idling.aspx https://www.towerhamlets.gov.uk/lgnl/environment_and_waste/environmental_health/pollution/air_quality/Air_quality_information_and_campaigns/Anti_idling.aspx https://www.towerhamlets.gov.uk/lgnl/environment_and_waste/environmental_health/pollution/air_quality/Air_quality_information_and_campaigns/Air_pollution_and_idling.aspx |
| 58. | Enforce anti-idling regulations by becoming a designated authority to issue Fixed Penalty Notices to idling drivers. | Fixed Penalty Notices enforcement mechanism now operational since 2018 for idling vehicles. Adopted powers to issue FPN’s for idling, under the Road Traffic Regulations 2002. Refer to action 57. | Measure Completed – No further action https://www.towerhamlets.gov.uk/lgnl/environment_and_waste/environmental_health/pollution/air_quality/Air_quality_information_and_campaigns/Anti_idling.aspx https://www.towerhamlets.gov.uk/lgnl/environment_and_waste/environmental_health/pollution/air_quality/Air_quality_information_and_campaigns/Air_pollution_and_idling.aspx |
| 59. | Increasing the proportion of electric, hydrogen and ultra-low emission vehicles in Car Clubs. | An initial report on the status of car clubs was considered by the Mayor and Lead Member in the Parking Policy Review Group. The proposal moving forward was to undertake a wider review of the changing need, operating models elsewhere and contract options to require certain things, including EV. The end result being a 'Tower Hamlets' model that could be put to market, focussed on the needs of TH residents and businesses, with standards and KPI monitoring by the Council. Unfortunately, we now only have two car clubs working in the borough with only one of them using the free-floating model. And the other one choosing to reduce its fleet by 20%. This is as a result of increasing market forces and the ability of car clubs to maintain profit margins. We have been instrumental in reducing the cost of a car club permit by 27% in order to support the business and encouraging move EV vehicles in to the borough. This is still a challenge due to the infrastructure of the EV charging points which continue to go in but at a pace but also the increased cost of an Electric vehicle over that of a Hybrid. We are working with the car clubs and our comms team to raise the profile of the car clubs with our residents and business of the borough. |  |
| 60. | Review parking permit fee banding to encourage lower emission vehicle choice or add an additional diesel surcharge to existing permit fees. | Review of parking permits completed; now includes banding which has lowest parking fee for low emissions vehicles, and also greatest fee chargeable for diesel vehicles: - Discount on Electric Vehicles introduced in April 2019 - Levy on Diesel vehicles introduced in October 2019 Review of parking permits completed on annual basis. In 2020, the Mayor and Lead Member in the Parking Policy Review Group agreed to take forward proposals to introduce a Levy on Diesel Vehicles using paid-for-parking. This was considered as part of the Annual Fees & Charges Review and approved. It was delayed due to Covid-19, but it has now been implemented. | Measure Completed – No further action |
| 61. | Installation of residential electric charge points. | We are working on a new EV strategy which will set out a new delivery plan for the borough from 2021-25. There are currently 201 electric vehicle chargers installed on street. The Council will install a further 150 by October 2023, bringing the total to 361. 148 have already been installed, so the figure is now 350. | https://talk.towerhamlets.gov.uk/evchargers |
| 62. | Installation of rapid chargers to help enable the take up of electric taxis, cabs and commercial vehicles (in partnership with TfL and/or OLEV) | TfL rapid charger funding is no longer available to boroughs, so we will seek to run a procurement exercise for new rapid chargers. The number of points will be determined by our new EV Strategy. No rapid chargers installed in 2021-22. Rapid chargers: We are identifying locations and will be seeking LEVI funding for new rapid chargers in autumn 2023. These should be delivered in the 2024/25 financial year. |  |
| 63. | Investigate reprioritisation of road space to smooth traffic flow, reduce congestion, improve bus journey times, cycling and pedestrian experience, and reduce emissions caused by congested traffic | In 2022, the implementation of 15 school streets were paused pending review of the School Streets programme by the new administration. The Low Traffic Neighbourhood in Bethnal Green, Bus Gate in Wapping and Pedestrianised Zones in Brick Lane are also under review . A modal filter installed in Antill Road in March 2022 has already been removed as part of the review process. Public consultations on the future of the above LTN schemes were undertaken in 2022 and results are being analysed. | https://www.towerhamlets.gov.uk/lgnl/transport_and_streets/roads,_highways_and_pavements/School_Streets.aspx |
| 64. | Continue to provide/ ensure provisions of infrastructure to support walking and cycling including on street residential secure parking lockers, cycle routes, cycle permeability schemes, traffic management area reviews. | No further measures were implemented in 2022 | Measure Completed – No further action |
| 65. | Reduce traffic in the borough through the development of a new Local Implementation Plan in line with the Mayors Transport Strategy. | Internal Transport Strategy workshop undertaken in January 2019, integrating Environmental Health, Transport, Highways and Public Health. Specific outcome and priority of the Transport Strategy is to Improve air quality and make our surroundings quieter, more inviting and more appealing. | Measure Completed – No further action |
| 66. | Continue to encourage staff sustainable travel by providing Dr Bike services and staff subscriptions to the TFL cycle hire scheme for site visits. Annual update of the Staff Travel Plan to ensure it remains relevant and proactive. | Ongoing. Staff Travel Survey was undertaken during Autumn 2021 to feed into travel plan for new Town Hall. However, the travel plan was paused due Covid-19 and the substantial impacts on travel to and during work. A staff travel survey was carried out during September/October 2022 before the move to new Town Hall, and we already had some baseline data for the staff working in the new Town Hall. The staff travel plan will need to be deferred until 2023, due to staff resources and also to allow time to analyse the results from the staff travel survey. Travel Plan will be published by March 2023. |  |
| 67. | Push for Tower Hamlets to be included in the ULEZ through partaking in the TFL Consultation process. | LBTH contributed to ULEZ consultations. ULEZ will include all LBTH when it is extended to North Circular in October 2021. | Measure Completed – No further action |
| 68. | Ensure responses to all government and regional consultations focus on reducing or eliminating emissions of Local air pollutants and CO2. | The Pollution Team provided input to the GLA consultation on air quality neutral and positive guidance. The GLA Air Quality Positive Guidance was published in February 2023. | Measure Completed – No further action |
| 69. | Lobby and work with TFL to reduce emissions from buses in the borough. e.g. through green bus corridors. Work with other statutory Services to reduce emissions – LFB, NHS etc. | Lobby continued to bring forward sooner TfL’s commitment to 100% buses by 2037. By summer 2022, all London buses met ULEZ standards in LBTH. A11 is a designated green bus corridor. Introduction of an electric bus on route 100: London bus route 100 was converted to electric traction on 18th January 2020, the 100 runs from near St Paul's Cathedral to Shadwell in London's East End via the edge of the City of London on London Wall, across Bishopsgate, on to Aldgate and Tower Gateway, near the Tower of London and Tower Bridge, on to Wapping the former dock area by the river Thames, and then turns away from the river to Cable Street, Shadwell, near the Shadwell Overground and Docklands Light Railway stations. The buses used by Go-Ahead London are 9.7m BYD/Alexander Enviro 200EV, they are the first buses fitted with the audible warning system for electric buses which cuts in at speeds lower than 12 mph, another innovation is the use of cameras instead of wing mirrors, there is a vertical oblong screen on the left hand side of the cab that shows both sides of the bus. Ultra-Low Emission Zone bus corridors to be implemented by Tfl: Discussions with TfL are continuing as TfL develop plans for practical implementation of electric buses in borough. |  |
| 70. | Lobby and work with TFL to reduce emissions from TfL controlled roads e.g. through reprioritisation of road space. | Ongoing. Officers are discussing TfL proposals for safety improvements on the A13 between New Street and Jubilee Street. This is a priority scheme for TfL despite current funding constraints. Service checking whether ULEZ bus corridors have been implemented by TfL, and if not, this action will be rolled into the next AQAP. |  |
| 71. | Lobby the GLA to strengthen their Air Quality Neutral Policy and lower the CHP emission limits in current guidance. | New Draft Environment Strategy and Draft London Plan seeks to have largest developments required to be air quality positive. Further guidance awaited from the GLA following adoption of the new London Plan. The new London Plan proposes a positive air quality approach. | Measure Completed – No further action |
| 72. | Delivery of the Tower Hamlets Mayor’s Air Quality fund over 2018/19 and 2019/20 | All grant funding from the Tower Hamlets Mayor’s air quality fund has now been allocated. All phase 1 work completed and some phase 2 work still ongoing. Final close out reports will be requested. 13 applications approved: - East End Homes - Swapping to battery operated gardening equipment - Coffe Afrique - Promoting AQ awareness through school assemblies and community events. - Women’s Environmental Network - Running participatory workshops and forum events focusing on raising awareness among women. Aiming to develop a toolkit for future replication - Citizens UK - Engage with locals to ascertain barriers to behaviour change; teach people how to engage with the LA to report and improve AQ - Bromley-by-Bow Centre - Target vulnerable people through links to the health centre. Recruit AQ champions, teach them how to monitor NOX and PM and develop a strategy based on findings - Trees for Cities - Planting trees and green screens - Barts Health - An anti-idling campaign focused on ambulance drivers and minicabs - Shapla Primary School - Installing a green wall around Shapla Primary - Barts Health - Training health professionals to teach their patients about air pollution and how to reduce their emissions andexposure - Poplar HARCA - Purchasing cargo bikes for Poplar Works fashion hub - Sustrans - Promoting sustainable transport at four schools on the cityfringe - CWC Environmental - Low pollution walking maps for Bow Schools - St Paul's Way Primary School -Promote modal shift at St Paul's Way Trust School | Measure Completed – No further action |
| 73. | The Mayor of Tower Hamlets to hold a meeting with The Royal Borough of Greenwich and Greater London Authority to discuss reducing the environmental impact of the proposed Enderby Wharf cruise terminal. Lobby for shore-side power to be provided for the ships. | Meeting held. Liaison between officers of LBTH & RBG to control pollution. Shore side power not feasible. Application for cruise terminal withdrawn by developers. Action deemed to be successful as proposal is now not going ahead as planned. | Measure Completed – No further action |
| 74. | Work with the Canal & River Trust, the GLA and other Boroughs with canals to devise a plan to best tackle issues with emissions from canal boats. Enforcement action to be taken where necessary. | Tower Hamlets initiated a canal engagement meeting involving boroughs with inland navigable waterways and the Canal & River Trust (CRT) about the need for a cohesive London-wide engagement campaign focused on sustainability and air quality on the canals. An awareness and engagement campaign for canal boat owners on pollution reduction (campaign jointly with LB Newham and LB Hackney) started early 2021. Joint good practice canal boat leaflet produced with LB Newham and LB Hackney and handed out to boat owners in the borough and promoted on CRT webpage. Also, leaflet was promoted on Twitter/Facebook. All marinas/moorings contacted: leaflets were sent, and it was asked to promote on their websites/newsletters etc and hopefully will be putting banners up when done on our waterways also as will Newham and Hackney Engagement work with Canal & River Trust (CRT) continuing. A canal engagement group consisting of London boroughs with canals has been formed with CRT represented. A London wide best practice leaflet developed for handing out to boat owners with consistent messaging. Further meetings suspended due to COVID. Joint bid with partners to review electric charging points for canal boat owners: - Engagement work with CRT continuing. - A canal engagement group consisting of London boroughs with canals has been formed with CRT represented. - A London wide best practice leaflet developed for handing out to boat owners with consistent messaging. - Further meetings suspended due to COVID. |  |
| 75. | Support the Port London Authority in the development and implementation of their Air Quality Strategy for the River Thames. | Presentation by PLA to Council’s Air Quality Partnership Board on their new strategy. Working with PLA to locate air quality monitors to assess the impact of existing Greenwich cruise liner terminal Monitors deployed by PLA in 2019. | Measure Completed – No further action |
| 76. | Support the GLA in Lobbying national Government to provide new powers and improved coordination for river and maritime vessels, including having a single regulatory authority for the Thames and London tributaries and introduce minimum emissions standards | Liaison with PLA Representative specifically on the production of guidance for developers and boroughs in 2018, as well as proposals for funds improving emissions from the marine sources within the borough. Attendance to the future workshops on the progress for the strategy. Maritime emissions addressed in latest DEFRA consultation. | Measure Completed – No further action |

# Planning Update and Other New Sources of Emissions

Table J.	Planning requirements met by planning applications in the London Borough of Tower Hamlets in 2022

The total number of planning applications in 2022 (minor and major applications, requests for Environmental Impact Assessments EIAs, full planning applications, and submissions of details) is 216.

| Condition | Number |
| --- | --- |
| Number of planning applications where an air quality impact assessment was reviewed for air quality impacts | 65 |
| Number of planning applications required to monitor for construction dust | 58 |
| Number of CHPs/Biomass boilers refused on air quality grounds | 1 |
| Number of CHPs/Biomass boilers subject to GLA emissions limits and/or other restrictions to reduce emissions | 20 |
| Number of developments required to install Ultra-Low NOx boilers | 5 |
| Number of developments where an AQ Neutral building and/or transport assessments undertaken | 51 |
| Number of developments where the AQ Neutral building and/or transport assessments not meeting the benchmark and so required to include additional mitigation | 2 |
| Number of planning applications with S106 agreements including other requirements to improve air quality | 0 |
| Number of planning applications with CIL payments that include a contribution to improve air quality | 0 |
| NRMM: Central Activity Zone, Canary Wharf and Opportunity Areas | - 37 conditions related to NRMM - 9 developments registered and compliant - 6 developments unregistered/uncompliant and being chased |
| NRMM: Greater London (excluding Central Activity Zone, Canary Wharf and Opportunity Areas) | - 21 conditions related to NRMM - 4 developments registered and compliant - 4 developments unregistered/uncompliant and being chased |

Commentary

All major planning applications are referred to the Pollution Team for comment. Each application is individually reviewed to ensure that the GLA SPGs on the ‘Sustainable Design and Construction’, as well as ‘The Control of Dust and Emissions During Construction and Demolition’, are in compliance.

Where there are compliance issues, Pollution Officers recommend either further information to be obtained from the applicant, or relevant conditions recommended.

It is at the discretion of the Development Control Team/Planning Committee as to what action is taken on recommendations made by the Pollution Team.

3.1	New or significantly changed industrial or other sources

No new sources were identified in 2022.

# Additional Activities to Improve Air Quality

4.1	London Borough of Tower Hamlets Fleet

At the end of December 2022, Tower Hamlets fleet consisted of 281 vehicles in total. Of these, 5 were Zero Emission vehicles, and 2 were non Zero Emission Capable vehicles (hybrids).

4.2	NRMM Enforcement Project

Tower Hamlets continue to support the project by:

- providing match funding,

- forwarding details of major applications determined by the Council to the project team,

- checking sites for completion,

- follow up action on non-compliance reported to us,

- Including NRMM condition on major planning applications consent,

4.3	Air Quality Alerts

Tower Hamlets support airTEXT (https://www.airtext.info/). Details can be found in Action 6 of Table I ‘Delivery of Air Quality Action Plan Measures’.

# Appendix A	Details of Monitoring Site Quality QA/QC

A.1	Automatic Monitoring Sites

Calibrations at Tower Hamlets Roadside, Millwall Park and Victoria Park are undertaken by Ricardo Energy and Environment. Millwall Park and Victoria Park are both urban background sites, so they calibrated every 4 weeks. Tower Hamlets Roadside is calibrated every 2 weeks. All sites are provided with ISO 17025 QC audits by Ricardo Energy and Environment every 6 months.

Note: the Blackwall site is operated by Transport for London, not LBTH.

PM10 Monitoring Adjustment

Millwall Park – 1020 Heated BAM, correction applied Victoria Park – TEOM, VCM correction applied Both VCM and BAM correction is applied automatically when data is downloaded from Air Quality England web site.

A.2	Diffusion Tubes

• Lab supplying and analysing the tubes:

SOCOTEC Unit 12, Moorbrook, Southmead Industrial Park Didcot OX11 7HP

• Preparation method used:

The tubes were prepared by spiking acetone: triethanolamine (50:50) onto the grids prior to the tubes being assembled. The tubes were desorbed with distilled water and the extract analysed using a segmented flow autoanalyser with ultraviolet detection

• Confirmation that the lab follows the procedures set out in the Practical Guidance:

The samples have been analysed in accordance with SOCOTEC’s standard operating procedure ANU/SOP/1015 Issue 1. This method meets the guidelines set out in DEFRA’s ‘Diffusion Tubes For Ambient NO2 Monitoring: Practical Guidance.’

• Results of laboratory precision results:

This analysis of diffusion tube samples to determine the amount of nitrogen dioxide present on the tube is within the scope of our UKAS schedule. Any further calculations and assessments requiring exposure details and conditions fall outside the scope of our accreditation. In the AIR PT intercomparison scheme for comparing spiked Nitrogen Dioxide diffusion tubes, SOCOTEC currently holds the highest rank of a ‘Satisfactory’ laboratory.

Factor from Local Co-location Studies

We have two diffusion tubes (the non-automatic monitoring sites of ID 42 Victoria Park co-location site, and ID 43 Victoria Park co-location site) co-located with the automatic monitoring site Victoria Park TH002.

As every year, a co-location study was undertaken with the use of the two diffusion tubes at Victoria Park automatic monitoring site, which has high quality chemiluminescence results (to national AURN standards).

However, the length of the monitoring study is not representative. The LLAQM.TG(19) states that the duration of the whole diffusion tube study is not representative if it is less than one year (especially, if it is less than nine monitoring periods).

As per the LLAQM Technical Guidance 19, it was not possible to calculate the local bias-adjustment factor from our co-location study (Table K). Therefore, the local bias-adjustment factor is not available.

Table K.	Local Bias Adjustment Factor in 2022

(Not possible to calculate the local bias-adjustment factor)

Discussion of Choice of Factor to Use

Since the local Bias Adjustment Factor is not available, we have used the national Bias Adjustment Factor 0.76 as per 03/2023 (SOCOTEC Didcot, 50% TEA in acetone).

The national Bias Adjustment Factor was used also for the previous years.

Table L.	Bias Adjustment Factors 2016 - 2022

| Year | Local or National | If National, Version of National Spreadsheet | Adjustment Factor |
| --- | --- | --- | --- |
| 2022 | National | 03/2023 (SOCOTEC Didcot, 50% TEA in acetone) | 0.76 |
| 2021 | National | 03/2022 (SOCOTEC Didcot, 50% TEA in acetone) | 0.78 |
| 2020 | National | 03/2021 (SOCOTEC Didcot, 50%TEA, 50% Acetone) | 0.77 |
| 2019 | National | 03/2020 | 0.75 |
| 2018 | National | 03/2019 | 0.77 |
| 2017 | National | 03/2018 | 0.77 |
| 2016 | National | Defra National Bias Adjustment Spreadsheet 2016 | 0.77 |

A.3	Adjustments to the Ratified Monitoring Data

Short-term to Long-term Data Adjustment

In 2022, data capture rate of 75% was not achieved at 6 non-automatic monitoring sites and at 3 automatic monitoring analysers.

Non-Automatic Monitoring Sites (diffusion tubes) NO2:

- Site ID 3 (Bethnal Green Rd/ Brick Lane), data capture: 67%

- Site ID 4 (Commercial St/Calvin St), data capture: 67%

- Site ID 7 (St Katherine's Way), data capture: 33%

- Site ID 10 (Whitechapel Rd/Adler St), data capture: 67%

- Site ID 25 (Cavell St/Stepney Way), data capture: 67%

- Site ID 66 (Millwall Park), data capture: 58%

Automatic Monitoring Site NO2:

- TH004 Blackwall, data capture 62%

Automatic Monitoring Site PM2.5:

- TH004 Blackwall, data capture 46%

Automatic Monitoring Site PM10:

- Millwall Park TH001, data capture 60%

Consequently, it has been necessary to annualise the results of these sites in accordance with the procedure described in LAQM TG (19). Calculations for each site are set out below.

As reference, Newham Wren Close (Newham Council) and Victoria Park (Tower Hamlets Council) sites were used. Both sites are background sites.

Millwall Park could not be used as reference for NO2, because data capture was 0% in both June and July.

Table M. Short-Term to Long-Term Monitoring Data Adjustment (9 sub-tables included):

Table M1. Short-Term to Long-Term Monitoring Data Adjustment - Diffusion tube Site ID 3 Bethnal Green Rd/ Brick Lane

Table M2. Short-Term to Long-Term Monitoring Data Adjustment - Diffusion tube Site ID 4 Commercial St/Calvin St

Table M3. Short-Term to Long-Term Monitoring Data Adjustment - Diffusion tube Site ID 7 St Katherine's Way

Table M4. Short-Term to Long-Term Monitoring Data Adjustment - Diffusion tube Site ID 10 Whitechapel Rd/Adler St

Table M5. Short-Term to Long-Term Monitoring Data Adjustment - Diffusion tube Site ID 25 Cavell St/Stepney Way

Table M6. Short-Term to Long-Term Monitoring Data Adjustment - Diffusion tube Site ID 66 Millwall Park

Table M7. Short-Term to Long-Term Monitoring Data Adjustment - TH004 Blackwall Automatic Monitoring Site NO2

Table M8. Short-Term to Long-Term Monitoring Data Adjustment - TH004 Blackwall Automatic Monitoring Site PM2.5

Table M9. Short-Term to Long-Term Monitoring Data Adjustment - Millwall Park TH001 Automatic Monitoring Site PM10

Distance Adjustment

All NO₂ diffusion tube results have been adjusted to estimate the concentrations at the nearest receptor, using the LAQM NO2 Fall-off with Distance Calculator (Version 4.2) in line with the procedure specified in LLAQM.TG (19).

The methodology consists of comparing the monitored NO₂ annual mean concentrations at a given point against known relationships between NO₂ concentrations and the distance from a road source.

As background concentration, the NO₂ annual mean concentration from the automatic monitoring site Victoria Park TH002 was used.

See Table N for the NO2 fall off with distance calculations.

Table N.	NO2 Fall off With Distance Calculations

Background concentration = NO₂ annual mean concentration from the automatic monitoring site Victoria Park TH002.

# Appendix B	Full Monthly Diffusion Tube Results for 2022

Table O.	NO2 Diffusion Tube Results

| Site ID | Location | Valid data capture for monitoring period % | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec | Annual Mean - Raw Data | Annual Mean - national bias-adjusted (as per 0.76 factor 03/2023) and annualised (LLAQM Technical Guidance) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Colombia Rd/Gossett Street | 92 | 42.6 | 31.4 | 32.9 | 18.6 | 19.7 | 17.3 | 17.4 | -- | 20.4 | 27.4 | 21.4 | 40.0 | 26.3 | 20.0 |
| 2 | Calvert Ave/Boundary Street | 92 | 45.8 | 33.5 | 30.4 | 24.4 | -- | 21.4 | 19.7 | 19.4 | 22.2 | 32.6 | 24.8 | 36.1 | 28.2 | 21.4 |
| 3 | Bethnal Green Rd/ Brick Lane | 67 | 48.9 | 40.0 | 35.0 | -- | 36.6 | 27.7 | 26.4 | 23.6 | -- | -- | 34.4 | -- | 34.1 | 26.4 |
| 4 | Commercial St/Calvin St | 67 | 53.2 | 43.7 | 47.6 | 39.4 | -- | 33.6 | 35.5 | 33.5 | -- | 31.7 | -- | -- | 39.8 | 30.4 |
| 5 | Whitechapel High St (KFC) | 100 | 66.7 | 57.2 | 49.8 | 46.6 | 46 | 41.9 | 42.1 | 36.6 | 45.9 | 53.8 | 42.3 | 51.9 | 48.4 | 36.8 |
| 6 | Mansell St | 100 | 53.0 | 44.6 | 45.9 | 38.8 | 36.9 | 35.6 | 33.5 | 32.9 | 39.3 | 49.5 | 47.2 | 49 | 42.2 | 32.1 |
| 7 | St Katherine's Way | 33 | 46.5 | 29.8 | -- | 22.5 | -- | -- | -- | -- | 23.2 | -- | -- | -- | 30.5 | 25.4 |
| 8 | Wapping High St/Sampson St | 100 | 46.4 | 29.3 | 33.9 | 25.1 | 19.4 | 20.5 | 19.9 | 20.5 | 25.7 | 29.4 | 24.3 | 39.9 | 27.9 | 21.2 |
| 9 | Cartwright Street | 75 | 47.4 | 33.5 | 34.2 | 27.5 | 25.3 | 22.5 | 20.2 | -- | -- | -- | 22.3 | 37.4 | 30.0 | 22.8 |
| 10 | Whitechapel Rd/Adler St | 67 | 50.7 | 37.0 | 41.6 | 34.7 | 28.5 | -- | 56 | -- | 33.5 | 38.4 | -- | -- | 40.1 | 30.5 |
| 11 | Brick Lane/Princelet St | 75 | 47.3 | 31.2 | 37.0 | 26.5 | 22 | 21.5 | -- | 21.9 | -- | 32.5 | -- | 42.0 | 31.3 | 23.8 |
| 12 | Buckfast St/Bethnal Green Rd | 100 | 38.5 | 32.3 | 34.4 | 28.9 | 25.4 | 21.5 | 27 | 24.3 | 26.8 | 30.5 | 25.8 | 36.6 | 29.3 | 22.3 |
| 13 | Squirries St/Gosset St | 92 | 48.1 | 31.1 | 38.4 | 29.0 | 25.8 | 24.7 | 23.5 | -- | 24.9 | 33.4 | 24 | 41.4 | 34.3 | 26.1 |
| 14 | Warner Place/Hackney Rd | 92 | 46.6 | 34.2 | 38.9 | 29.1 | 26.2 | 24.7 | 24.4 | -- | 39.2 | 39.6 | 31.5 | 43.4 | 34.3 | 26.1 |
| 15 | Parmiter St/ Cambridge Heath Road | 92 | 60.6 | 38.2 | 38.8 | 34.8 | 28.0 | 25.2 | 28.7 | 31.3 | -- | 36.2 | 27.6 | 47.5 | 36.1 | 27.4 |
| 16 | Paradise Row/Bethnal Green Rd | 83 | 50.4 | 38.9 | 41.8 | 32.2 | 28.8 | 27.2 | -- | 29.7 | -- | 38.4 | 37.7 | 48.0 | 37.3 | 28.4 |
| 17 | Finnis St/Three Colts Lane | 92 | 44.2 | 27.2 | -- | 24.9 | 22.0 | 18.8 | 19.1 | 20.1 | 23.9 | 27.8 | 29.2 | 39.7 | 27.0 | 20.5 |
| 18 | Sidney St/Mile End Rd | 100 | 54.8 | 38.5 | 38.9 | 32.9 | 28.9 | 30.2 | 29.6 | 27.3 | 34.8 | 39.2 | 26.4 | 50.2 | 34.3 | 26.1 |
| 19 | Philpot St/Commercial Road | 100 | 62.2 | 45.0 | 48.6 | 33.0 | 32.1 | 29.7 | 30.7 | 30.5 | 33.4 | 42.6 | 35.4 | 42.2 | 38.8 | 29.5 |
| 20 | Dellow St/The Highway | 100 | 64.9 | 58.9 | 59.9 | 43.0 | 45.8 | 44.3 | 45.1 | 43.6 | 42.3 | 53.0 | 50.6 | 45.1 | 49.7 | 37.8 |
| 21 | Queensbridge Rd/Hackney Rd | 100 | 48.4 | 35.6 | 34.8 | 28.0 | 26.9 | 25.3 | 25.3 | 27.3 | 27.8 | 35.4 | 27.7 | 42.2 | 32.1 | 24.4 |
| 22 | Wapping Wall/Garnet St | 100 | 51.8 | 29.8 | 43.1 | 31.3 | 24.4 | 21.8 | 22.9 | 25.1 | 29.8 | 30.1 | 24.7 | 40.8 | 31.3 | 23.8 |
| 23 | Brodlove Lane | 92 | 51.6 | 39.9 | 42.0 | -- | 35 | 34.7 | 34.3 | 31.5 | 35.0 | 42.1 | 35.8 | 40.9 | 38.4 | 29.2 |
| 24 | Jubilee Street/Commercial Rd | 100 | 62.3 | 46.5 | 52.8 | 44.0 | 36.3 | 37.8 | 43.1 | 44.7 | 50.4 | 43.5 | 39.1 | 49.2 | 45.8 | 34.8 |
| 25 | Cavell St/Stepney Way | 67 | 52.2 | 38.7 | 38.3 | 30.4 | 26.8 | -- | -- | -- | 33.5 | 44.5 | -- | 53.3 | 39.7 | 29.4 |
| 26 | Hannibal Rd/Mile End Rd | 100 | 52.8 | 38.8 | 34.4 | 30.1 | 29.6 | 29.3 | 29.4 | 27.3 | 32.5 | 34.9 | 27.1 | 41.3 | 34.0 | 25.8 |
| 27 | Roman Rd/Globe Road | 92 | 50.5 | 32.1 | 42.0 | 31.9 | 26.2 | 20.2 | 20.1 | 26.8 | 29.6 | -- | 31.6 | 39.8 | 31.9 | 24.2 |
| 28 | Bonner Road | 100 | 61.3 | 46.0 | 42.9 | 38.0 | 42.2 | 39.6 | 31.2 | 46.5 | 43.0 | 45.4 | 41 | 52.6 | 42.5 | 32.3 |
| 29 | Grove Rd/Old Ford Rd | 92 | 48.7 | 35.6 | 38.1 | 29.6 | 26.8 | 28.8 | 27.4 | 30.6 | 31.3 | 39.5 | -- | 47.2 | 34.9 | 26.5 |
| 30 | Fieldgate Street | 92 | 50.3 | 37.8 | 48.6 | 36.8 | 27.0 | 28.7 | -- | 33.5 | 32.4 | 41.2 | 37.8 | 42.0 | 37.8 | 28.7 |
| 31 | Whitechapel Market | 100 | 65.6 | 51.7 | 50.1 | 51.8 | 46.3 | 47.7 | 45.8 | 50.1 | 52.6 | 51.4 | 35.9 | 44.4 | 49.5 | 37.6 |
| 32 | Globe Rd/Mile End Rd | 100 | 56.4 | 40.8 | 45.9 | 40.3 | 33.4 | 34.2 | 33.6 | 38.5 | 39.8 | 41.2 | 38.2 | 45.1 | 40.6 | 30.9 |
| 33 | Stepney Green | 100 | 46.2 | 31.8 | 37.8 | 25.5 | 19.8 | 19.0 | 19.2 | 19.9 | 24.3 | 27.1 | 26.8 | 31.8 | 27.4 | 20.8 |
| 34 | Pitsea St/Commercial Rd | 92 | -- | 37.8 | 38.9 | 30.4 | 25.9 | 25.9 | 29.7 | 25.4 | 31.1 | 35.0 | 39.7 | 40.8 | 32.8 | 24.9 |
| 35 | Narrow St/Limehouse Link | 100 | 88.1 | 71.7 | 91.0 | 86.9 | 71.7 | 69.6 | 87.9 | 78.5 | 84.9 | 68.0 | 58.4 | 80.7 | 78.1 | 59.4 |
| 36 | Locksley St/St Paul's Way | 92 | 46.8 | 28.2 | 41.1 | 29.8 | 22.4 | 21.0 | 24.6 | 26.4 | 11.1 | -- | 18.3 | 38.9 | 28.1 | 21.3 |
| 37 | Rhodeswell Rd | 100 | 50.1 | 35.9 | 39.1 | 25.8 | 21.8 | 22.1 | 23.7 | 24.4 | 28.7 | 34.4 | 29.7 | 38.9 | 31.2 | 23.7 |
| 38 | Ben Johnson Road | 100 | 55.3 | 35.9 | 42.1 | 32.6 | 25.7 | 28.4 | 31.6 | 34.8 | 33.1 | 33.0 | 34.9 | 43.0 | 35.9 | 27.3 |
| 39 | Harford St/Mile End Rd | 100 | 43.7 | 35.2 | 49.5 | 39.2 | 26.7 | 27.0 | 30.7 | 34.2 | 33.4 | 33.1 | 34.8 | 40.3 | 35.7 | 27.1 |
| 40 | Thoydon Rd | 100 | 50.2 | 40.1 | 35.6 | 28.2 | 23.0 | 19.9 | 24.9 | 25.3 | 27.4 | 33.4 | 26.9 | 40.6 | 31.3 | 23.8 |
| 41 | Ford Close/Roman Rd | 100 | 51.8 | 35.4 | 43.7 | 38.3 | 29.8 | 26.8 | 32.5 | 35.7 | 35.2 | 41.2 | 26.2 | 45.3 | 36.8 | 28.0 |
| 42 | Victoria Park (Co-location site) | 92 | 32.5 | 22.0 | -- | 12.6 | 14.9 | 12.7 | 14.4 | 17.1 | 19.2 | 13.3 | 20.3 | 30.2 | 19.0 | 14.5 |
| 43 | Victoria Park (Co-location site) | 75 | 32.1 | 20.7 | -- | 12.6 | 15 | 14.6 | 16.9 | 20.5 | 17.8 | -- | 18.4 | -- | 18.7 | 14.2 |
| 44 | Parnell Rd/Old Ford Rd | 92 | 52.4 | 35.8 | 42.8 | 31.3 | 26.9 | -- | 28.8 | 30.4 | 34.6 | 39.0 | 38.4 | 42.2 | 36.6 | 27.8 |
| 45 | St Stephen's Rd/Tredegar Rd | 100 | 56.8 | 39.0 | 46.0 | 32.6 | 28.8 | 30.8 | 31.1 | 33.5 | 36.1 | 38.7 | 30.0 | 49.9 | 37.8 | 28.7 |
| 46 | Rhondda Grove/Mile End Rd | 83 | 39.3 | 32.6 | 37.5 | 28.2 | -- | 22.8 | 23.9 | 25.0 | -- | 34.6 | 35.0 | 38.8 | 31.8 | 24.1 |
| 47 | Wentworth Mews | 100 | 55.8 | 38.6 | 47.8 | 38.4 | 30.1 | 29.0 | 31.4 | 36.7 | 42.4 | 39.8 | 23.3 | 44.6 | 38.2 | 29.0 |
| 48 | Ackroyd Drive | 92 | 58.5 | 39.5 | 39.1 | -- | 29.1 | 30.6 | 33.3 | 30.3 | 38.7 | 42.4 | 26.7 | 44.3 | 37.5 | 28.5 |
| 49 | Dod St/Burdett Rd | 92 | 40.7 | 30.2 | 33.7 | 23.8 | 22.5 | 17.0 | 19.7 | -- | 21.9 | 30.1 | 28.1 | 35.6 | 27.6 | 21.0 |
| 50 | Rich Street | 92 | 51.9 | 34.7 | 43.1 | 42.3 | 30.0 | 27.5 | -- | 32.2 | 37.0 | 33.6 | 37.5 | 44.5 | 37.7 | 28.6 |
| 51 | Watney Market | 100 | 46.7 | 31.6 | 31.4 | 26.6 | 25.2 | 23.4 | 22.4 | 23.8 | 29.3 | 32.5 | 34.5 | 36.3 | 30.3 | 23.0 |
| 52 | Wick Lane/Autumn St | 92 | 48.4 | 34.6 | 44.2 | 32.8 | 28.6 | 29.9 | 34.9 | 34.9 | 40.8 | 41.2 | 43.9 | -- | 37.7 | 28.6 |
| 53 | Fairfield Road/Tredegar Road | 92 | 57.6 | 40.3 | 46.7 | 39.3 | -- | 34.8 | 37.7 | 42.2 | 39.6 | 43.1 | 34.0 | 47.4 | 42.1 | 32.0 |
| 54 | Bow Rd /Glebe Terrace | 100 | 66.5 | 49.0 | 61.0 | 49.9 | 44.6 | 49.1 | 51.3 | 53.8 | 52.5 | 62.0 | 47.3 | 63.3 | 54.2 | 41.2 |
| 55 | TH Cemetery Park | 100 | 36.1 | 26.8 | 27.2 | 17.8 | 16.1 | 15.4 | 16.2 | 17.4 | 18.5 | 21.2 | 19.5 | 28.3 | 21.7 | 16.5 |
| 56 | Bow Common Lane/St Paul's Way | 92 | 52.0 | -- | 41.7 | 26.7 | 27.2 | 27.7 | 25.8 | 28.3 | 31.7 | 37.2 | 34.3 | 34.2 | 33.3 | 25.3 |
| 57 | Augusta St/Giraud St | 100 | 43.6 | 26.2 | 40.0 | 23.9 | 20.0 | 19.6 | 20.7 | 18.4 | 26.7 | 28.9 | 27.0 | 36.5 | 27.6 | 21.0 |
| 58 | Dolphin Lane | 83 | 45.7 | 30.0 | 36.2 | 26.7 | 21.1 | 20.8 | 23.4 | -- | 23.9 | 31.00 | -- | 35.7 | 29.5 | 22.4 |
| 59 | Westferry Road/Limehouse Link jnct | 83 | 49.0 | 31.8 | 45.0 | 33.7 | 27.6 | 25.8 | 32.5 | -- | -- | 31.3 | 40.0 | 40.8 | 35.8 | 27.2 |
| 60 | Cascades, Westferry Road | 100 | 57.0 | 41.3 | 49.4 | 41.0 | 32.6 | 30.3 | 35.0 | 35.1 | 38.2 | 41.6 | 30.6 | 49.7 | 40.2 | 30.5 |
| 61 | Bow Rd/Alfred St | 100 | 52.8 | 35.4 | 35.0 | 27.2 | 25.4 | 21.7 | 24.7 | 25.6 | 28.4 | 33.8 | 34.5 | 41.6 | 32.2 | 24.5 |
| 62 | Mast House Terrace | 100 | 49.8 | 32.1 | 47.7 | 30.9 | 27.6 | 24.3 | 23.2 | 25.9 | 30.5 | 39.8 | 21.5 | 44.7 | 33.2 | 25.2 |
| 63 | Millwall Park | 83 | 42.3 | 27.5 | 29.4 | 21.7 | 18.6 | 16.9 | 15.3 | -- | 17.7 | -- | 22.5 | 33.3 | 24.5 | 18.6 |
| 64 | Limeharbour | 100 | 60.7 | 41.2 | 58.6 | 47.3 | 39.9 | 37.2 | 42.5 | 38.3 | 43.2 | 37.2 | 33.0 | 46.0 | 43.8 | 33.3 |
| 65 | Manchester Road/East Ferry Road | 75 | 40.5 | 25.1 | 42.1 | 28.5 | 22.4 | -- | -- | 23.3 | 24.4 | -- | 26.7 | 35.7 | 29.9 | 22.7 |
| 66 | Millwall Park | 58 | 44.8 | -- | 31.9 | 20.2 | -- | 18.5 | -- | -- | 17.4 | 25.6 | -- | 33.0 | 27.3 | 19.7 |
| 67 | Seyssel Street | 100 | 47.6 | 29.4 | 41.9 | 31.7 | 25.8 | 25.0 | 22.8 | 26.0 | 29.3 | 35.9 | 26.8 | 44.0 | 32.2 | 24.5 |
| 68 | Manchester Road/Ollife Street | 100 | 50.1 | 24.4 | 47.4 | 32.7 | 24.3 | 23.6 | 26.2 | 27.5 | 30.1 | 36.9 | 25.6 | 45.2 | 32.8 | 25.0 |
| 69 | Lawnhouse Close | 100 | 50.8 | 32.3 | 37.3 | 30.5 | 26.3 | 24.7 | 23.1 | 25.2 | 25.0 | 34.4 | 25.8 | 46.4 | 31.8 | 24.2 |
| 70 | Admirals Way | 92 | 41.4 | 27.0 | 39.0 | 24.4 | -- | 25.9 | 29.2 | 25.1 | 23.8 | 32.3 | 24.9 | 37.5 | 30.0 | 22.8 |
| 71 | Toynbee St/Commercial St | 100 | 63.5 | 46.8 | 39.8 | 36.2 | 33.6 | 32.9 | 31.9 | 35.8 | 40.4 | 44.5 | 43.9 | 51.8 | 41.8 | 31.7 |
| 72 | Prestons Road/ Coldharbour | 100 | 54.3 | 36.4 | 49.4 | 37.5 | 30.9 | 28.3 | 33.4 | 31.7 | 33.7 | 33.2 | 34.7 | 42.7 | 37.2 | 28.3 |
| 73 | John Smith Mews | 100 | 44.2 | 31.7 | 37.6 | 31.8 | 23.5 | 22.0 | 23.5 | 20.3 | 27.2 | 27.6 | 25.1 | 37.1 | 29.3 | 22.3 |
| 74 | Poplar High St/Cotton St | 100 | 82.4 | 66.1 | 88.3 | 65.9 | 66.5 | 75.7 | 71.5 | 69.2 | 78.6 | 82.4 | 62.1 | 71.1 | 73.3 | 55.7 |
| 75 | Hale Street | 100 | 44.8 | 28.2 | 43.2 | 25.0 | 23.9 | 24.3 | 20.7 | 19.5 | 23.2 | 32.2 | 30.6 | 36.1 | 29.3 | 22.3 |
| 76 | Chrisp Street/E India Dock Road | 92 | 53.0 | 38.1 | 54.9 | 39.8 | 33.1 | 32.8 | 35.3 | 32.5 | 40.9 | -- | 33.5 | 49.7 | 40.3 | 30.6 |
| 77 | Morris/Barchester Street | 100 | 42.4 | 26.8 | 40.3 | 24.4 | 20.0 | 20.2 | 23.0 | 20.4 | 28.7 | 31.7 | 28.5 | 35.5 | 28.5 | 21.7 |
| 78 | Devons Road / Campbell Road | 100 | 58.2 | 36.2 | 51.7 | 39.2 | 35.2 | 32.2 | 34.0 | 34.7 | 44.8 | 34.2 | 28.9 | 48.5 | 39.8 | 30.3 |
| 79 | Hatfield Terrace/Fairfield Road | 100 | 40.6 | 28.0 | 45.7 | 29.9 | 20.1 | 21.2 | 24.2 | 27.3 | 27.8 | 30.6 | 27.4 | 37.8 | 30.1 | 22.8 |
| 80 | Wrexham Road | 100 | 48.8 | 31.9 | 47.2 | 38.0 | 24.9 | 25.2 | 29.5 | 33.9 | 34.6 | 32.5 | 29.6 | 41.4 | 34.8 | 26.4 |
| 81 | Bromley High Street/ St Leonards | 100 | 50.8 | 35.9 | 43.0 | 34.1 | 28.8 | 28.1 | 30.0 | 32.0 | 36.3 | 36.2 | 32.0 | 44.7 | 36.0 | 27.4 |
| 82 | Devas Street /Devons road | 100 | 46.8 | 37.7 | 57.3 | 43.1 | 28.5 | 31.1 | 34.6 | 33.1 | 44.2 | 36.8 | 28.0 | 29.5 | 37.6 | 28.5 |
| 83 | Zetland Street/A12 | 100 | 57.9 | 42.5 | 69.4 | 54.6 | 47.3 | 48.4 | 58.5 | 52.5 | 66.0 | 47.7 | 47.9 | 53.8 | 53.9 | 40.9 |
| 84 | Blair Street (End of Street) | 100 | 55.6 | 44.7 | 44.9 | 31.1 | 32.6 | 37.4 | 31.9 | 27.9 | 37.4 | 45.8 | 34.5 | 48.9 | 39.4 | 29.9 |
| 85 | Portree Street | 92 | 52.3 | 36.5 | 53.2 | 36.7 | 34.7 | 35.2 | -- | 31.8 | 37.7 | 41.7 | 46.5 | 50.8 | 41.6 | 31.6 |
| 86 | Newport Avenue | 100 | 40.8 | 29.9 | 38.6 | 28.3 | 24.5 | 23.3 | 24.4 | 22.6 | 29.3 | 30.6 | 27.8 | 34.4 | 29.5 | 22.5 |
| 87 | Mile End Road Corner Bancroft Rd | 92 | 52.6 | 37.0 | 40.7 | 38.5 | 30.2 | 26.6 | -- | 33.1 | 39.3 | 34.2 | 32.8 | 38.7 | 36.7 | 27.9 |
| 88 | Shirbutt St o/s Holy Family School | 100 | 40.5 | 23.7 | 37.3 | 24.8 | 21.2 | 19.7 | 19.3 | 19.3 | 23.5 | 28.6 | 22.9 | 34.3 | 26.3 | 20.0 |
| 89 | Thames Path Storers Quay | 100 | 42.7 | 28.2 | 48.3 | 33.5 | 21.0 | 19.9 | 20.0 | 21.5 | 20.3 | 28.6 | 22.5 | 38.7 | 28.8 | 21.9 |
| 90 | Sextant Avenue | 100 | 41.7 | 24.4 | 39.3 | 23.5 | 16.4 | 18.0 | 18.0 | 21.2 | 19.7 | 26.6 | 19.5 | 36.0 | 25.4 | 19.3 |
| 91 | At the entrance of MOT station | 83 | -- | -- | 57.4 | 17.3 | 28.5 | 25.1 | 25.1 | 23.6 | 31.8 | 35.6 | 34.4 | 25.0 | 30.4 | 23.1 |
| 92 | At the exit of MOT station | 83 | -- | -- | 43.6 | 19.4 | 37.3 | 38.7 | 34.1 | 33.1 | 37.7 | 42.7 | 29.6 | 48.3 | 36.5 | 27.7 |

Notes

Concentrations are presented as μg m-3.

Exceedances of the NO2 annual mean AQO of 40 μg m-3 are shown in bold.

NO2 annual means in excess of 60 μg m-3, indicating a potential exceedance of the NO2 hourly mean AQS objective are shown in bold and underlined.

All means have been “annualised” in accordance with LLAQM Technical Guidance if valid data capture for the calendar year is less than 75% and greater than 25%.

(a) Data capture for the monitoring period, in cases where monitoring was only carried out for part of the year.

(b) data capture for the full calendar year (e.g. if monitoring was carried out for six months the maximum data capture for the full calendar year would be 50%).

# Appendix C	Monitoring sites

Figure 6. Location of automatic monitoring sites.

(Details of sites are on Table B)

Figure 7. Location of non-automatic monitoring sites.

(Details of sites are on Table C)

LLAQM Policy and Technical Guidance 2019 (LLAQM.TG(19))

New BAM PM 2.5 monitor installed in 2019

Site operated by Transport for London