# London Borough of Tower Hamlets

# Air Quality Annual Status Report for 2020

# Date of publication: 26 July 2021

This report provides a detailed overview of air quality in the London Borough of Tower Hamlets during 2020. It has been produced to meet the requirements of the London Local Air Quality Management (LLAQM) statutory process.

Contact details

Nicoletta Vianello

Pollution Team, Environmental Health and Trading Standards

Mulberry Place, London Borough of Tower Hamlets Town Hall, 5 Clove Cres

London E14 2BG

environmental.protection@towerhamlets.gov.uk

# Contents

Abbreviations	4

1. 	Air Quality Monitoring	6

1.1 	Locations	6

1.2	Comparison of Monitoring Results with AQOs	15

2.	Impact of COVID-19 upon LAQM Requirements	34

2.1	COVID-19 lockdown effects on air quality in the London Borough of Tower Hamlets 	34

2.2	   Impacts in the London Borough of Tower Hamlets ......................................41

3.	Action to Improve Air Quality	44

3.1	Air Quality Action Plan Progress	44

4. 	Planning Update and Other New Sources of Emissions	67

4.1	New or significantly changed industrial or other sources	68

Appendix A       Details of Monitoring Site Quality QA/QC	69

A.1	              Automatic Monitoring Sites	69

A.2	              Diffusion Tubes	69

A.3	              Adjustments to the Ratified Monitoring Data	72

Appendix B     Full Monthly Diffusion Tube Results for 2020	121

# Tables

Table A. Summary of National Air Quality Standards and Objectives	5

Table B. Details of Automatic Monitoring Sites for 2020	6

Table C. Details of Non-Automatic Monitoring Sites for 2020	7

Table D. Annual Mean NO2 Ratified and Bias-adjusted Monitoring Results	15

Table E. NO2 Automatic Monitoring Results: Comparison with 1-hour Mean Objective, Number of 1-Hour Means > 200μg m-3	27

Table F. Annual Mean PM10 Automatic Monitoring Results (μg m-3) 	29

Table G. PM10 Automatic Monitoring Results: Comparison with 24-Hour Mean Objective, Number of PM10 24-Hour Means > 50μg m-3	31

Table H. Annual Mean PM2.5 Automatic Monitoring Results (μg m-3) 	33

Table I.	Delivery of Air Quality Action Plan Measures	44

Table J. Planning Requirements Met by Planning Applications in the London Borough of Tower Hamlets in 2020	67

Table K. Local Bias-adjustment Factor	70

Table L. Short-Term to Long-Term Monitoring Data Adjustment for all 90 diffusion tubes 	72

Table M. NO2 Fall off With Distance Calculations	96

Table N. NO2 Diffusion Tube Results	121

# Figures

Figure 1. Annual Mean NO₂ Results at Automatic Monitoring Sites ….……………..22

Figure 2. NO₂ Diffusion Tube Annual Mean Ratified and Bias-adjusted Monitoring Results at Sites 1-30 from 2015 to 2020 ….………..……………….............24

Figure 3. NO₂ Diffusion Tube Annual Mean Ratified and Bias-adjusted Monitoring Results at Sites 31-60 from 2015 to 2020 ….………..………………...........25

Figure 4. NO₂ Diffusion Tube Annual Mean Ratified and Bias-adjusted Monitoring Results at Sites 61-90 from 2015 to 2020 ….………..……………..............24

Figure 5. Annual Mean PM10 Automatic Monitoring Results (μg m-3) ….……..……..30

# Abbreviations

| Abbreviation | Description |
| --- | --- |
| AQAP | Air Quality Action Plan |
| AQMA | Air Quality Management Area |
| AQO | Air Quality Objective |
| BEB | Buildings Emission Benchmark |
| CAB | Cleaner Air Borough |
| EV | Electric Vehicle |
| GLA | Greater London Authority |
| LAEI | London Atmospheric Emissions Inventory |
| LAQM | Local Air Quality Management |
| LLAQM | London Local Air Quality Management |
| NRMM | Non-Road Mobile Machinery |
| PM10 | Particulate matter less than 10 micron in diameter |
| PM2.5 | Particulate matter less than 2.5 micron in diameter |
| TEB | Transport Emissions Benchmark |
| TfL | Transport for London |

## Table A.	Summary of National Air Quality Standards and Objectives

| Pollutant | Standard / Objective (UK) | Averaging Period | Date(1) |
| --- | --- | --- | --- |
| Nitrogen dioxide (NO2) | 200μg m-3 not to be exceeded more than 18 times a year | 1-hour mean | 31 Dec 2005 |
| Nitrogen dioxide (NO2) | 40μg m-3 | Annual mean | 31 Dec 2005 |
| Particles (PM10) | 50μg m-3 not to be exceeded more than 35 times a year | 24-hour mean | 31 Dec 2004 |
| Particles (PM10) | 40μg m-3 | Annual mean | 31 Dec 2004 |
| Particles (PM2.5) | 25μg m-3 | Annual mean | 2020 |
| Particles (PM2.5) | Target of 15% reduction in concentration at urban background locations | 3-year mean | Between 2010 and 2020 |
| Sulphur dioxide (SO2) | 266μg m-3 not to be exceeded more than 35 times a year | 15-minute mean | 31 Dec 2005 |
| Sulphur dioxide (SO2) | 350μg m-3 not to be exceeded more than 24 times a year | 1-hour mean | 31 Dec 2004 |
| Sulphur dioxide (SO2) | 125μg m-3 mot to be exceeded more than 3 times a year | 24-hour mean | 31 Dec 2004 |

Notes:

(1) Date by which to be achieved by and maintained thereafter

# 1. 	Air Quality Monitoring

1.1 	Locations

Table B.	Details of Automatic Monitoring Sites for 2020

| Site ID | Site Name | X (m) | Y (m) | Site Type | In AQMA? | Distance from monitoring site to relevant exposure(m) | Distance to kerb of nearest road (N/A if not applicable) (m) | Inlet height (m) | Pollutants monitored | Monitoring technique |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TH2 and TH2P | Mile End | 535927 | 182221 | Roadside | Y | 1m (offices) (40m residential) | 3 | 3 | NO2, NO, NOx, PM2.5 | Chemiluminescence; BAM for PM2.5 |
| TH004 | Blackwall | 538290 | 181452 | Roadside | Y | 29m (residential) | 3 | 3 | NO2, NO, NOx, PM2.5, PM10, O3 | Chemiluminescence; UV photometric; FDMS TEOM (for PM) |
| TH002 | Victoria Park | 536487 | 184238 | Background | Y | 290m (residential) | 300 | 2 | NO2, NO, NOx, PM2.5, PM10 | Chemiluminescence; BAM for PM2.5andPM10 |
| TH001 | Millwall Park | 538052 | 178559 | Background | Y | 60m (residential) | 60 | 1.5 | NO2, NO, NOx, PM10, O3 | Chemiluminescence; BAM UV absorption |

## Table C.	Details of Non-Automatic Monitoring Sites for 2020

| Site ID | Site Name | X (m) | Y (m) | Site Type | In AQMA? | Distance from monitoring site to relevant exposure(m) | Distance to kerb of nearest road (N/A if not applicable)(m) | Inlet height (m) | Pollutants monitored | Tube co-located with an automatic monitor? (Y/N) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Colombia Rd/Gossett Street | 533883 | 182815 | Kerbside | Y | 5 | 0.5 | 2.4 | NO2 | N |
| 2 | Calvert Ave/Boundary Street | 533507 | 182569 | Kerbside | Y | 4 | 0.5 | 2.3 | NO2 | N |
| 3 | Bethnal Green Rd/ Brick Lane | 533860 | 182442 | Kerbside | Y | 3 | 0.5 | 2.3 | NO2 | N |
| 4 | Commercial St/Calvin St | 533611 | 182037 | Kerbside | Y | 7 | 0.5 | 2.4 | NO2 | N |
| 5 | Whitechapel High St (KFC) | 533985 | 181426 | Kerbside | Y | 3 | 0.5 | 2.3 | NO2 | N |
| 6 | Mansell St | 533800 | 181021 | Kerbside | Y | 6 | 0.5 | 2.2 | NO2 | N |
| 7 | St Katherine's Way | 533992 | 180376 | Roadside | Y | 10 | 10 | 2.3 | NO2 | N |
| 8 | Wapping High St/Sampson St | 534444 | 180122 | Kerbside | Y | 3 | 0.5 | 2.4 | NO2 | N |
| 9 | Cartwright Street | 533955 | 180805 | Kerbside | Y | 5 | 0.5 | 2.4 | NO2 | N |
| 10 | Whitechapel Rd/Adler St | 534133 | 181509 | Kerbside | Y | 6 | 0.5 | 2.3 | NO2 | N |
| 11 | Brick Lane/Princelet St | 533866 | 181860 | Kerbside | Y | 5 | 0.5 | 2.3 | NO2 | N |
| 12 | Buckfast St/Bethnal Green Rd | 534259 | 182580 | Kerbside | Y | 4 | 0.5 | 2.5 | NO2 | N |
| 13 | Squirries St/Gosset St | 534313 | 182810 | Kerbside | Y | 4 | 0.5 | 2.3 | NO2 | N |
| 14 | Warner Place/Hackney Rd | 534255 | 183130 | Kerbside | Y | 17 | 0.5 | 2.4 | NO2 | N |
| 15 | Parmiter St/ Cambridge Heath Road | 534881 | 183240 | Kerbside | Y | 4 | 0.5 | 2.2 | NO2 | N |
| 16 | Paradise Row/Bethnal Green Rd | 534959 | 182757 | Kerbside | Y | 3 | 0.5 | 2.3 | NO2 | N |
| 17 | Finnis St/Three Colts Lane | 534783 | 182385 | Kerbside | Y | 2 | 0.5 | 2.2 | NO2 | N |
| 18 | Sidney St/Mile End Rd | 534968 | 181878 | Roadside | Y | 6 | 2 | 2.3 | NO2 | N |
| 19 | Philpot St/Commercial Road | 534816 | 181321 | Kerbside | Y | 8 | 0.5 | 2.3 | NO2 | N |
| 20 | Dellow St/The Highway | 534951 | 180779 | Roadside | Y | 4 | 2 | 2.2 | NO2 | N |
| 21 | Queensbridge Rd/Hackney Rd | 533985 | 183122 | Kerbside | Y | 4 | 0.5 | 2.2 | NO2 | N |
| 22 | Wapping Wall/Garnet St | 535133 | 180376 | Kerbside | Y | 3 | 0.5 | 2.4 | NO2 | N |
| 23 | Brodlove Lane | 535598 | 180816 | Kerbside | Y | 3 | 0.5 | 2.2 | NO2 | N |
| 24 | Jubilee Street/Commercial Rd | 535174 | 181290 | Kerbside | Y | 5 | 0.5 | 2.3 | NO2 | N |
| 25 | Cavell St/Stepney Way | 534884 | 181667 | Kerbside | Y | 20 | 1 | 2.3 | NO2 | N |
| 26 | Hannibal Rd/Mile End Rd | 535386 | 182021 | Kerbside | Y | 3 | 0.5 | 2.2 | NO2 | N |
| 27 | Roman Rd/Globe Road | 535296 | 182793 | Kerbside | Y | 12 | 0.5 | 2.2 | NO2 | N |
| 28 | Bonner Road | 535356 | 183223 | Kerbside | Y | 7 | 0.5 | 2.7 | NO2 | N |
| 29 | Grove Rd/Old Ford Rd | 535930 | 183385 | Kerbside | Y | 12 | 0.5 | 2.4 | NO2 | N |
| 30 | Fieldgate Street | 534239 | 181565 | Kerbside | Y | 8 | 0.5 | 2.3 | NO2 | N |
| 31 | Whitechapel Market | 534516 | 181744 | Roadside | Y | 15 | 1.5 | 2.2 | NO2 | N |
| 32 | Globe Rd/Mile End Rd | 535634 | 182148 | Kerbside | Y | 4 | 0.5 | 2.3 | NO2 | N |
| 33 | Stepney Green | 535545 | 181604 | Urban background | Y | 30 | 15 | 2.4 | NO2 | N |
| 34 | Pitsea St/Commercial Rd | 535797 | 181164 | Kerbside | Y | 4 | 0.5 | 2.3 | NO2 | N |
| 35 | Narrow St/Limehouse Link | 535977 | 180879 | Roadside | Y | 15 | 1.5 | 2.6 | NO2 | N |
| 36 | Locksley St/St Paul's Way | 536704 | 181647 | Kerbside | Y | 40 | 0.5 | 2.9 | NO2 | N |
| 37 | Rhodeswell Rd | 536577 | 181379 | Kerbside | Y | 40 | 1 | 2.4 | NO2 | N |
| 38 | Ben Johnson Road | 536080 | 181721 | Kerbside | Y | 4 | 0.5 | 2.6 | NO2 | N |
| 39 | Harford St/Mile End Rd | 536089 | 182258 | Roadside | Y | 3 | 1.5 | 2.2 | NO2 | N |
| 40 | Thoydon Rd | 536105 | 183049 | Kerbside | Y | 7 | 0.5 | 2.4 | NO2 | N |
| 41 | Ford Close/Roman Rd | 536457 | 183301 | Roadside | Y | 2 | 1.5 | 2.3 | NO2 | N |
| 42 | Victoria Park (Co-location site) | 536494 | 184170 | Urban background | Y | 330 | 320 | 2.15 | NO2 | Y |
| 43 | Victoria Park (Co-location site) | 536494 | 184170 | Urban background | Y | 330 | 320 | 2.1 | NO2 | Y |
| 44 | Parnell Rd/Old Ford Rd | 536875 | 183740 | Kerbside | Y | 4 | 0.5 | 2.4 | NO2 | N |
| 45 | St Stephen's Rd/Tredegar Rd | 536713 | 183070 | Kerbside | Y | 3 | 0.5 | 2.3 | NO2 | N |
| 46 | Rhondda Grove/Mile End Rd | 536542 | 182589 | Kerbside | Y | 5 | 0.5 | 2.5 | NO2 | N |
| 47 | Wentworth Mews | 536452 | 182454 | Kerbside | Y | 15 | 0.5 | 2.5 | NO2 | N |
| 48 | Ackroyd Drive | 536768 | 181772 | Kerbside | Y | 40 | 0.5 | 2.5 | NO2 | N |
| 49 | Dod St/Burdett Rd | 537049 | 181292 | Kerbside | Y | 5 | 0.5 | 2.5 | NO2 | N |
| 50 | Rich Street | 536937 | 180987 | Roadside | Y | 3 | 1.5 | 2.2 | NO2 | N |
| 51 | Watney Market | 534938 | 181257 | Roadside | Y | 10 | 15 | 2.2 | NO2 | N |
| 52 | Wick Lane/Autumn St | 537304 | 183619 | Kerbside | Y | 3 | 0.5 | 2.4 | NO2 | N |
| 53 | Fairfield Road/Tredegar Road | 537159 | 183415 | Kerbside | Y | 4 | 0.5 | 2.4 | NO2 | N |
| 54 | Bow Rd /Glebe Terrace | 537525 | 182887 | Kerbside | Y | 5 | 0.5 | 2.4 | NO2 | N |
| 55 | TH Cemetery Park | 536732 | 182361 | Roadside | Y | 15 | 5 | 2.5 | NO2 | N |
| 56 | Bow Common Lane/St Paul's Way | 537248 | 181820 | Kerbside | Y | 30 | 0.5 | 2.3 | NO2 | N |
| 57 | Augusta St/Giraud St | 537516 | 181392 | Kerbside | Y | 15 | 1 | 2.4 | NO2 | N |
| 58 | Dolphin Lane | 537539 | 180688 | Kerbside | Y | 7 | 1 | 2.9 | NO2 | N |
| 59 | Westferry Road/Limehouse Link jnct | 537100 | 180791 | Kerbside | Y | 7 | 1 | 2.2 | NO2 | N |
| 60 | Cascades, Westferry Road | 537115 | 180074 | Kerbside | Y | 18 | 0.5 | 2.4 | NO2 | N |
| 61 | Bow Rd/Alfred St | 537056 | 182773 | Kerbside | Y | 6 | 0.5 | 2.4 | NO2 | N |
| 62 | Mast House Terrace | 537348 | 178690 | Kerbside | Y | 5 | 0.5 | 2.7 | NO2 | N |
| 63 | Millwall Park | 538246 | 178689 | Urban background | Y | 300 | 250 | 2.3 | NO2 | N |
| 64 | Limeharbour | 537953 | 179357 | Kerbside | Y | 10 | 0.5 | 2.2 | NO2 | N |
| 65 | Manchester Road/East Ferry Road | 538032 | 178360 | Kerbside | Y | 2 | 0.5 | 2.3 | NO2 | N |
| 66 | Millwall Park | 538258 | 178689 | Urban background | Y | 300 | 250 | 2.3 | NO2 | N |
| 67 | Seyssel Street | 538544 | 178767 | Kerbside | Y | 15 | 0.5 | 2.3 | NO2 | N |
| 68 | Manchester Road/Ollife Street | 538431 | 179044 | Kerbside | Y | 3 | 0.5 | 2.3 | NO2 | N |
| 69 | Lawnhouse Close | 538190 | 179750 | Kerbside | Y | 30 | 0.5 | 2.3 | NO2 | N |
| 70 | Admirals Way | 537424 | 179910 | Kerbside | Y | 15 | 0.5 | 2.3 | NO2 | N |
| 71 | Toynbee St/Commercial St | 533689 | 181705 | Roadside | Y | 10 | 2 | 2.5 | NO2 | N |
| 72 | Prestons Road/ Coldharbour | 538364 | 180188 | Kerbside | Y | 4 | 0.5 | 2.2 | NO2 | N |
| 73 | John Smith Mews | 538742 | 180756 | Kerbside | Y | 10 | 0.5 | 2.3 | NO2 | N |
| 74 | Poplar High St/Cotton St | 538244 | 180761 | Kerbside | Y | 10 | 0.5 | 2.2 | NO2 | N |
| 75 | Hale Street | 537661 | 180768 | Kerbside | Y | 7 | 0.5 | 2.3 | NO2 | N |
| 76 | Chrisp Street/E India Dock Road | 537940 | 181021 | Kerbside | Y | 20 | 0.5 | 2.7 | NO2 | N |
| 77 | Morris/Barchester Street | 537731 | 181761 | Kerbside | Y | 4 | 0.5 | 2.5 | NO2 | N |
| 78 | Devons Road / Campbell Road | 537577 | 182232 | Kerbside | Y | 10 | 0.5 | 2.4 | NO2 | N |
| 79 | Hatfield Terrace/Fairfield Road | 537355 | 183059 | Kerbside | Y | 3 | 0.5 | 2.4 | NO2 | N |
| 80 | Wrexham Road | 537581 | 183209 | Kerbside | Y | 3 | 0.5 | 2.4 | NO2 | N |
| 81 | Bromley High Street/ St Leonards | 537868 | 182912 | Kerbside | Y | 5 | 0.5 | 2.4 | NO2 | N |
| 82 | Devas Street /Devons road | 537821 | 182332 | Kerbside | Y | 7 | 0.5 | 2.4 | NO2 | N |
| 83 | Zetland Street/A12 | 538178 | 181747 | Kerbside | Y | 50 | 0.5 | 2.3 | NO2 | N |
| 84 | Blair Street (End of Street) | 538365 | 181180 | Roadside | Y | 15 | 5 | 2.5 | NO2 | N |
| 85 | Portree Street | 538895 | 181296 | Kerbside | Y | 4 | 0.5 | 2.3 | NO2 | N |
| 86 | Newport Avenue | 538954 | 180872 | Kerbside | Y | 15 | 0.5 | 2.6 | NO2 | N |
| 87 | Mile End Road Corner Bancroft Rd | 535929 | 182220 | Kerbside | Y | 30 | 0.5 | 2.3 | NO2 | N |
| 88 | Shirbutt St o/s Holy Family School | 537555 | 180892 | Kerbside | Y | 10 | 0.5 | 2.3 | NO2 | N |
| 89 | Thames Path Storers Quay | 538730 | 178733 | Roadside | Y | 4 | 10 | 2.3 | NO2 | N |
| 90 | Sextant Avenue | 538674 | 178888 | Kerbside | Y | 4 | 1 | 2.3 | NO2 | N |

1.2	Comparison of Monitoring Results with AQOs

The results presented are after adjustments for “annualisation” and for distance to a location of relevant public exposure (if required), the details of which are described in Appendix A.

## Table D.	Annual Mean NO2 Ratified and Bias-adjusted Monitoring Results

| Site ID | Site type | Valid data capture for monitoring period % | Valid data capture 2020 % b | 2014 Annual Mean Concentration (μg m-3) | 2015 Annual Mean Concentration (μg m-3) | 2016 Annual Mean Concentration (μg m-3) | 2017 Annual Mean Concentration (μg m-3) | 2018 Annual Mean Concentration (μg m-3) | 2019 Annual Mean Concentration (μg m-3) | 2020 Annual Mean Concentration (μg m-3) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TH2 Mile End | Automatic | N/A | 99.54 | 62 | 53 | 51.7 | 48 | 47 | 35 | 25 |
| TH004 Blackwall | Automatic | N/A | 99 | 58 | 58 | 59 | 56 | 51 | 47 | 39 |
| TH002 Victoria Park | Automatic | N/A | 96.56 | 44 | 33 | 32 | 32 | 26 | 24 | 17 |
| TH001 Millwall Park | Automatic | N/A | 98.09 | - | 26 | 25.3 | 26 | 23 | 24 | 17 |

| Site ID | Site type | Valid data capture for monitoring period % | Valid data capture 2020 % b | 2014 Annual Mean Concentration (μg m-3) | 2015 Annual Mean Concentration (μg m-3) | 2016 Annual Mean Concentration (μg m-3) | 2017 Annual Mean Concentration (μg m-3) | 2018 Annual Mean Concentration (μg m-3) | 2019 Annual Mean Concentration (μg m-3) | 2020 Annual Mean Concentration (μg m-3) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Colombia Rd/Gossett Street | N/A | 58 | - | 38 | 37 | 39 | 34 | 33 | 24.9 |
| 2 | Calvert Ave/Boundary Street | N/A | 50 | - | 42 | 41 | 40 | 37 | 35 | 26.0 |
| 3 | Bethnal Green Rd/ Brick Lane | N/A | 58 | - | 47 | 46 | 45 | 36 | 37 | 27.4 |
| 4 | Commercial St/Calvin St | N/A | 58 | - | 66 | 60 | 60 | 53 | 48 | 34.2 |
| 5 | Whitechapel High St (KFC) | N/A | 50 | - | 72 | 64 | 62 | 61 | 48 | 33.9 |
| 6 | Mansell St | N/A | 58 | - | 84 | 71 | 75 | 50 | 45 | 35.2 |
| 7 | St Katherine's Way | N/A | 58 | - | 33 | 34 | 30 | 28 | 28 | 19.9 |
| 8 | Wapping High St/Sampson St | N/A | 58 | - | 35 | 36 | 33 | 31 | 30 | 21.6 |
| 9 | Cartwright Street | N/A | 58 | - | - | - | - | 33 | 34 | 24.7 |
| 10 | Whitechapel Rd/Adler St | N/A | 58 | - | - | - | - | 46 | 40 | 28.9 |
| 11 | Brick Lane/Princelet St | N/A | 50 | - | 42 | 44 | 40 | 35 | 32 | 24.4 |
| 12 | Buckfast St/Bethnal Green Rd | N/A | 58 | - | 42 | 42 | 39 | 35 | 32 | 24.2 |
| 13 | Squirries St/Gosset St | N/A | 58 | - | - | - | - | 38 | 38 | 27.1 |
| 14 | Warner Place/Hackney Rd | N/A | 58 | - | 42 | 42 | 41 | 38 | 35 | 25.5 |
| 15 | Parmiter St/ Cambridge Heath Road | N/A | 58 | - | - | - | - | 45 | 41 | 30.3 |
| 16 | Paradise Row/Bethnal Green Rd | N/A | 58 | - | 50 | 50 | 42 | 41 | 36 | 28.1 |
| 17 | Finnis St/Three Colts Lane | N/A | 58 | - | 35 | 35 | 35 | 29 | 31 | 21.2 |
| 18 | Sidney St/Mile End Rd | N/A | 58 | - | 47 | 47 | 46 | 40 | 37 | 29.3 |
| 19 | Philpot St/Commercial Road | N/A | 58 | - | 54 | 54 | 51 | 44 | 41 | 31.0 |
| 20 | Dellow St/The Highway | N/A | 58 | - | 70 | 69 | 59 | 52 | 49 | 34.3 |
| 21 | Queensbridge Rd/Hackney Rd | N/A | 58 | - | - | - | - | 55 | 35 | 26.2 |
| 22 | Wapping Wall/Garnet St | N/A | 58 | - | 34 | 37 | 34 | 32 | 30 | 23.3 |
| 23 | Brodlove Lane | N/A | 50 | - | 47 | 45 | 46 | 43 | 40 | 29.7 |
| 24 | Jubilee Street/Commercial Rd | N/A | 58 | - | 68 | 65 | 62 | 64 | 47 | 33.2 |
| 25 | Cavell St/Stepney Way | N/A | 58 | - | 44 | 45 | 45 | 40 | 38 | 29.9 |
| 26 | Hannibal Rd/Mile End Rd | N/A | 33 | - | 72 | 50 | 50 | 44 | 42 | 29.2 |
| 27 | Roman Rd/Globe Rd | N/A | 58 | - | - | - | - | 36 | 34 | 25.8 |
| 28 | Bonner Road | N/A | 58 | - | 39 | 41 | 40 | 37 | 35 | 25.7 |
| 29 | Grove Rd/Old Ford Rd | N/A | 50 | - | 47 | 48 | 46 | 43 | 40 | 27.5 |
| 30 | Fieldgate Street | N/A | 58 | - | 53 | 48 | 42 | 46 | 38 | 27.7 |
| 31 | Whitechapel Market | N/A | 58 | - | 71 | 68 | 69 | 63 | 54 | 40.4 |
| 32 | Globe Rd/Mile End Rd | N/A | 58 | - | 55 | 54 | 52 | 48 | 42 | 29.7 |
| 33 | Stepney Green | N/A | 58 | - | 34 | 34 | 37 | 39 | 28 | 23.5 |
| 34 | Pitsea St/Commercial Rd | N/A | 58 | - | - | - | - | 37 | 35 | 26.3 |
| 35 | Narrow St Limehouse Link | N/A | 58 | - | - | - | - | 86 | 77 | 54.4 |
| 36 | Locksley St/St Paul's Way | N/A | 50 | - | 31 | 38 | 36 | 35 | 32 | 26.0 |
| 37 | Rhodeswell Rd | N/A | 58 | - | 35 | 39 | 36 | 34 | 30 | 26.5 |
| 38 | Ben Johnson Road | N/A | 58 | - | 41 | 45 | 44 | 36 | 36 | 28.7 |
| 39 | Harford St/Mile End Rd | N/A | 58 | - | 43 | 41 | 41 | 42 | 36 | 26.4 |
| 40 | Thoydon Rd | N/A | 42 | - | - | - | - | 36 | 33 | 24.2 |
| 41 | Ford Close/Roman Rd | N/A | 50 | - | 41 | 41 | 40 | 38 | 34 | 25.9 |
| 42 | Victoria Park Co-location site | N/A | 58 | - | 23 | 24 | 24 | 22 | 21 | 16.9 |
| 43 | Victoria Park Co-location site | N/A | 58 | - | 23 | 25 | 23 | 22 | 21 | 16.4 |
| 44 | Parnell Rd/Old Ford Rd | N/A | 58 | - | 39 | 41 | 42 | 35 | 34 | 27.8 |
| 45 | St Stephen's Rd/Tredegar Rd | N/A | 58 | - | 44 | 47 | 45 | 56 | 39 | 31.0 |
| 46 | Rhondda Grove/Mile End Rd | N/A | 42 | - | 35 | 41 | 37 | 48 | 33 | 26.2 |
| 47 | Wentworth Mews | N/A | 58 | - | 50 | 51 | 46 | 48 | 41 | 32.2 |
| 48 | Ackroyd Drive | N/A | 58 | - | 45 | 44 | 44 | 38 | 37 | 32.3 |
| 49 | Dod St/Burdett Rd | N/A | 58 | - | 37 | 38 | 38 | 33 | 30 | 24.9 |
| 50 | Rich Street | N/A | 58 | - | 42 | 45 | 42 | 42 | 35 | 28.5 |
| 51 | Watney Market | N/A | 33 | - | 38 | 37 | 34 | 33 | 29 | 23.2 |
| 52 | Wick Lane/Autumn St | N/A | 50 | - | 44 | 45 | 42 | 40 | 37 | 29.8 |
| 53 | Fairfield Road/Tredegar Road | N/A | 50 | - | 52 | 52 | 50 | 42 | 43 | 34.5 |
| 54 | Bow Rd /Glebe Terrace | N/A | 42 | - | 57 | 49 | 57 | 60 | 50 | 37.0 |
| 55 | TH Cemetery Park | N/A | 58 | - | 25 | 26 | 25 | 23 | 22 | 16.7 |
| 56 | Bow Common Lane/St Paul's Way | N/A | 58 | - | 41 | 43 | 40 | 37 | 32 | 25.9 |
| 57 | Augusta St/Girauld St | N/A | 58 | - | - | - | - | 28 | 27 | 22.7 |
| 58 | Dolphin Lane | N/A | 58 | - | 33 | 36 | 32 | 29 | 28 | 23.1 |
| 59 | Westferry Road/Limehouse Link jnct | N/A | 50 | - | 40 | 39 | 40 | 37 | 31 | 28.1 |
| 60 | Cascades, Westferry Road | N/A | 58 | - | 44 | 45 | 41 | 39 | 36 | 33.5 |
| 61 | Bow Rd/Alfred St | N/A | 58 | - | 42 | 44 | 41 | 35 | 35 | 27.6 |
| 62 | Mast House Terrace | N/A | 58 | - | 32 | 35 | 34 | 29 | 32 | 26.7 |
| 63 | Millwall Park | N/A | 50 | - | 27 | 29 | 26 | 22 | 24 | 21.1 |
| 64 | Limeharbour | N/A | 58 | - | 42 | 42 | 40 | 38 | 37 | 36.2 |
| 65 | Manchester Road/East Ferry Road | N/A | 50 | - | 31 | 34 | 32 | 28 | 29 | 24.6 |
| 66 | Millwall Park | N/A | 58 | - | 27 | 30 | 29 | 25 | 22 | 18.2 |
| 67 | Seyssel Street | N/A | 58 | - | 33 | 34 | 34 | 30 | 31 | 26.6 |
| 68 | Manchester Road/Ollife Street | N/A | 42 | - | 29 | 34 | 33 | 32 | 34 | 26.3 |
| 69 | Lawnhouse Close | N/A | 58 | - | 44 | 41 | 41 | 34 | 31 | 27.4 |
| 70 | Admirals Way | N/A | 58 | - | - | - | - | 27 | 29 | 23.7 |
| 71 | Toynbee St/Commercial St | N/A | 42 | - | - | - | - | 54 | 45 | 35.2 |
| 72 | Prestons Road/ Coldharbour | N/A | 58 | - | 41 | 39 | 40 | 39 | 38 | 30.4 |
| 73 | John Smith Mews | N/A | 58 | - | 36 | 38 | 40 | 32 | 31 | 24.6 |
| 74 | Poplar High St/Cotton St | N/A | 58 | - | - | - | - | 64 | 71 | 58.6 |
| 75 | Hale Street | N/A | 58 | - | 31 | 33 | 34 | 34 | 29 | 23.5 |
| 76 | Chrisp Street/E India Dock Road | N/A | 58 | - | 51 | 48 | 49 | 45 | 39 | 32.8 |
| 77 | Morris/Barchester Street | N/A | 50 | - | 35 | 39 | 40 | 37 | 31 | 26.0 |
| 78 | Devons Road / Campbell Road | N/A | 50 | - | 47 | 48 | 47 | 43 | 36 | 32.0 |
| 79 | Hatfield Terrace/Fairfield Road | N/A | 58 | - | 31 | 31 | 33 | 32 | 28 | 23.6 |
| 80 | Wrexham Road | N/A | 58 | - | 43 | 41 | 40 | 38 | 35 | 27.6 |
| 81 | Bromley High Street/ St Leonards | N/A | 58 | - | 37 | 39 | 38 | 38 | 34 | 28.5 |
| 82 | Devas Street /Devons road | N/A | 50 | - | 47 | 50 | 48 | 45 | 37 | 28.7 |
| 83 | Zetland Street/A12 | N/A | 58 | - | 66 | 63 | 62 | 63 | 52 | 41.2 |
| 84 | Blair Street (End of Street) | N/A | 58 | - | 52 | 48 | 52 | 44 | 39 | 35.8 |
| 85 | Portree Street | N/A | 58 | - | 48 | 48 | 48 | 45 | 38 | 34.3 |
| 86 | Newport Avenue | N/A | 58 | - | 33 | 34 | 33 | 30 | 28 | 21.7 |
| 87 | Mile End Road Corner Bancroft Rd | N/A | 58 | - | - | - | - | 49 | 37 | 30.8 |
| 88 | Shirbutt St o/s Holy Family School | N/A | 58 | - | - | - | - | 28 | 26 | 21.3 |
| 89 | Thames Path Storers Quay | N/A | 42 | - | 24 | 30 | 29 | 26 | 26 | 22.7 |
| 90 | Sextant Avenue | N/A | 50 | - | 16 | 28 | 28 | 25 | 24 | 19.7 |

Notes:

The annual mean concentrations are presented as μg m-3.

Exceedances of the NO2 annual mean AQO of 40μgm-3 are shown in bold.

NO2 annual means in excess of 60μg m-3, indicating a potential exceedance of the NO2 hourly mean AQS objective are shown in bold and underlined.

Means for diffusion tubes have been corrected for bias.

All means have been “annualised” in accordance with LLAQM Technical Guidance if valid data capture for the calendar year is less than 75% and greater than 33%.

Results have been distance corrected where applicable.

(a) Data capture for the monitoring period, in cases where monitoring was only carried out for part of the year.

(b) Data capture for the full calendar year (e.g. if monitoring was carried out for six months the maximum data capture for the full calendar year would be 50%).

## Commentary: Automatic Monitoring Sites

All data from the automatic monitoring stations have been fully ratified.

Data capture rate of 75% was achieved at all nitrogen dioxide automatic monitoring stations in 2020.

All automatic monitoring sites in 2020 demonstrated a reduction in annual mean NO2 concentration from the previous years.

Annual mean NO2 concentrations at all monitoring sites are below the National Objective of 40µg/m³, even on roadside monitors, which show a decline in levels of NO2. There has been a significant reduction at the Mile End and Blackwall roadside sites since 2014.

Concentrations at background locations are relatively low.

Figure 1 shows annual mean NO2 results at the automatic monitoring sites.

Figure 1: Annual mean NO2 results at automatic monitoring sites.

## Commentary: Non-Automatic Monitoring Sites

For all NO2 annual mean concentrations of the diffusion tube monitoring sites, national bias-adjustment factor has been chosen versus the local bias-adjustment factor from our co-location study (see Section ‘Discussion of Choice of Factor to Use’ in this ASR).

Raw monthly data for all diffusion tube sites can be found in Table N at the end of this report.  Most of the sites are showing a downward trend compared to the previous year. Month by month analysis of data suggests this may be due to the introduction of the central London ULEZ in April 2019, or to the three Covid-19 lockdowns in 2020.

Data capture rate of 75% was not achieved at any nitrogen dioxide diffusion tube sites in 2020 due to Covid-19. Consequently, it has been necessary to annualise the results of all sites in accordance with the procedure described in LAQM TG (19) (see A3 ‘Adjustments to the Ratified Monitoring Data’ in this ASR).

Figure 2: NO2 diffusion tube annual mean ratified and bias-adjusted monitoring results at sites 1- 30 from 2015 to 2020.

Figure 3: NO2 diffusion tube annual mean ratified and bias-adjusted monitoring results at sites 31- 60 from 2015 to 2020.

Figure 4: NO2 diffusion tube annual mean ratified and bias-adjusted monitoring results at sites 61- 90 from 2015 to 2020.

## Table E.	NO2 Automatic Monitoring Results: Comparison with 1-hour Mean Objective, Number of 1-Hour Means > 200μgm-3

| Site ID | Valid data capture for monitoring period % a | Valid data capture 2020 % b | 2014 Number of Hourly Means > 200μg m-3 | 2015 Number of Hourly Means > 200μg m-3 | 2016 Number of Hourly Means > 200μg m-3 | 2017 Number of Hourly Means > 200μg m-3 | 2018 Number of Hourly Means > 200μg m-3 | 2019 Number of Hourly Means > 200μg m-3 | 2020 Number of Hourly Means > 200μg m-3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TH2 Mile End | N/A | 99.54 | 1 | 0 | 0 | 2 | 0 | 1 | 0 |
| TH004 Blackwall | N/A | 99.49 | 0 | 0 | 9 | 0 | 0 | 0 | 0 |
| TH002 Victoria Park | N/A | 96.56 | 0 | 0 | 0 | 24 | 1 | 0 | 0 |
| TH001 Millwall Park | N/A | 98.09 | - | 0 | 0 | 0 | 0 | 0 | 0 |

Notes

Results are presented as the number of 1-hour periods where concentrations greater than 200μg m-3 have been recorded.

Exceedance of the NO2 short term AQO of 200μgm-3 over the permitted 18 hours per year are shown in bold.

If the period of valid data is less than 85%, the 99.8th percentile of 1-hour means is provided in brackets.

(a) Data capture for the monitoring period, in cases where monitoring was only carried out for part of the year

(b) Data capture for the full calendar year (e.g. if monitoring was carried out for six months the maximum data capture for the full calendar year would be 50%)

## Commentary

All data from the automatic monitoring stations have been fully ratified.

For 2020, the hourly objective for NO2 concentrations are significantly below the permitted 18 days per year: the number of hourly means exceeding 200μg m-3 is zero at all monitoring sites.

## Table F.	Annual Mean PM10 Automatic Monitoring Results (μg m-3)

| Site ID | Valid data capture for monitoring period % a | Valid data capture 2020 %b | 2014 Annual Mean Concentration (μg m-3) | 2015 Annual Mean Concentration (μg m-3) | 2016 Annual Mean Concentration (μg m-3) | 2017 Annual Mean Concentration (μg m-3) | 2018 Annual Mean Concentration (μg m-3) | 2019 Annual Mean Concentration (μg m-3) | 2020 Annual Mean Concentration (μg m-3) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TH004 Blackwall | N/A | 94.97 | 29 | 22 | 23 | 25 | 20 | 20 | 17 |
| TH001 Millwall Park | N/A | 89.22 | - | 15 | 17 | 20 | 18 | 18 | 18 |
| TH002 Victoria Park | N/A | 97.09 | 17 | 17 | 16 | 17 | 18 | 18 | 17 |

Notes

The annual mean concentrations are presented as μgm-3.

Exceedances of the PM10 annual mean AQO of 40μgm-3 are shown in bold.

All means have been “annualised” in accordance with LLAQM Technical Guidance, if valid data capture is less than 75% and more than 33%.

(a) Data capture for the monitoring period, in cases where monitoring was only carried out for part of the year.

(b) Data capture for the full calendar year (e.g. if monitoring was carried out for six months the maximum data capture for the full calendar year would be 50%).

## Commentary

All data from the automatic monitoring stations have been fully ratified.

PM10 at the road-side monitor Blackwall has shown a slow downward trend from 2015, but a slight increase at the background site Millwall Park.

Annual mean PM10 concentrations at all monitoring sites are below the National Objective of 40µg/m³.

Figure 5: Annual Mean PM10 Automatic Monitoring Results (μg m-3).

## Table G.	PM10 Automatic Monitoring Results: Comparison with 24-Hour Mean Objective, Number of PM10 24-Hour Means > 50μg m-3

| Site ID | Valid data capture for monitoring period % a | Valid data capture 2020 % b | 2014 Number of Daily Means > 50μg m-3 | 2015 Number of Daily Means > 50μg m-3 | 2016 Number of Daily Means > 50μg m-3 | 2017 Number of Daily Means > 50μg m-3 | 2018 Number of Daily Means > 50μg m-3 | 2019 Number of Daily Means > 50μg m-3 | 2020 Number of Daily Means > 50μg m-3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TH004 Blackwall | N/A | 94.97 | 16 | 8 | 10 | 10 | 10 | 8(35) | 4 |
| TH001 Millwall Park | N/A | 89.22 | - | 0(22.04) | 0(27.9) | 8 | 1 | 7 (30) | 7 |
| TH002 Victoria Park | N/A | 97.09 | 0 | 2(32.36) | 3(28.6) | 2 | 1 | 7 | 5 |

Notes

Exceedances of the PM10 24-hour mean objective (50μgm-3 over the permitted 35 days per year) are shown in bold.

Where the period of valid data is less than 85% of a full year, the 90.4th percentile is provided in brackets.

(a) Data capture for the monitoring period, in cases where monitoring was only carried out for part of the year

(b) Data capture for the full calendar year (e.g. if monitoring was carried out for six months the maximum data capture for the full calendar year would be 50%).

## Commentary

24-hour values of PM10 concentrations over 50μg/m³ are significantly below the national air quality objective level at all monitoring sites.

In Blackwall (TH004), the 2020 data suggest that there has been a significant decrease in the number of PM10 daily mean values > 50μg/m³ since 2014.

In Victoria Park (TH002), an opposite trend happened: there has been an increase in the number of PM10 daily mean values > 50μg/m³ since 2014. However, following an increase in 2019, in 2020 there has been a decrease compared to the previous years.

Millwall Park (TH001), data show a slight increase in the number of daily mean exceeding 50μg/m³ since 2014. However, the situation in 2020 is the same as 2019.

## Table H.	Annual Mean PM2.5 Automatic Monitoring Results (μg m-3)

| Site ID | Valid data capture for monitoring period % a | Valid data capture 2020 % b | 2014 Annual Mean Concentration (μg m-3) | 2015 Annual Mean Concentration (μg m-3) | 2016 Annual Mean Concentration (μg m-3) | Annual Mean Concentration (μg m-3) | 2018 Annual Mean Concentration (μg m-3) | 2019 Annual Mean Concentration (μg m-3) | 2020 Annual Mean Concentration (μg m-3) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TH2P Mile End | N/A | 97.73 | - | - | - | - | - | 9.8 | 12 |
| TH004 Blackwall | N/A | 93 | 16 | 14 | 20 | 13 | 13 | 12.4 | 9 |
| TH002 Victoria Park | N/A | 99.02 | - | - | - | - | - | 10 | 12 |

Notes

The annual mean concentrations are presented as μgm-3.

Exceedances of the PM2.5 annual mean AQO of 25μgm-3 are shown in bold.

All means have been “annualised” in accordance with LLAQM Technical Guidance, if valid data capture is less than 75% and more than 33%.

(a) Data capture for the monitoring period, in cases where monitoring was only carried out for part of the year.

(b) Data capture for the full calendar year (e.g. if monitoring was carried out for six months the maximum data capture for the full calendar year would be 50%).

Commentary

PM2.5 concentrations at all monitoring sites are below the National Air Quality Objective of 25μg/m3.

In 2019, two PM2.5 monitors were installed into the existing monitoring site locations of Victoria Park (TH002), and Mile End Road (TH2P).

# 2.	Impact of COVID-19 upon LAQM

https://www.airqualityengland.co.uk/assets/reports/210/TowerHamlets_report_covid_analysis.html#time_series

David Madle, Ricardo Energy and Environment

2.1	COVID19 lockdown effects on air quality in the London Borough of Tower Hamlets

A report has been produced for London Borough of Tower Hamlets by Ricardo Energy and Environment to examine the impact of lockdown measures on evolving ambient air quality data. This analysis focuses on NOx, NO2 (and ozone data where measured) from January 2020 through January 2021, and uses proven modelling techniques to discount the influence of weather on ambient pollutant concentrations.

At roadside locations NOx concentrations are closely linked to primary emissions and should show the direct impact of reduced local traffic on air pollution. NO2 is from a mixture of primary emissions and secondary chemical reactions, but should again be closely linked to local traffic reduction.

With a reduction in NOx, we may expect to see an increase in ozone at roadside locations, as there is limited freshly emitted NO available to scavenge ozone.

Additionally, during the lockdown period, there were a number of regional ozone pollution episodes which may also have an effect on any changes observed. High ozone episodes typically occur during the summer season, when weather conditions are warm and sunny.

Both measured and modelled data reported here are provisional pending full QA/QC processes.

## Road Traffic Data

Between January 2020 and January 2021 there was a change in road traffic volumes due to Covid-19 restrictions and lockdowns.

The Department for Transport (DfT) publishes statistics (~ weekly) on estimated transport use during the COVID-19 pandemic. Details on the methodology and full statistics can be found here: (https://www.gov.uk/government/statistics/transport-use-during-the-coronavirus-covid-19-pandemic).

The blue line in the figure below shows the estimated daily percentage of all motor vehicles, with respect to the equivalent day in the first week of February 2020, across Great Britain, since 1st March 2020. The red line indicates the start of lockdown (23rd March 2020), and shaded grey areas represent weekends.

Figure 5: Daily percentage of all motor vehicles, across

Great Britain, from February 2020 to 1st March 2020.

## Simulated Normal Concentrations

A perennial problem when comparing air pollution data before and after certain interventions is the effect of the weather. To counteract the effect of weather, a model was used to simulate pollutant concentrations using wind speed/direction, temperature, hour of the day, weekday and Julian day as predictors. This model was then used to predict concentrations from March 2020, which can be seen as the normal concentrations expected if no intervention had taken place. The model also takes into some account the long-term reduction in NOx concentrations due to the ongoing decrease in NOx emissions.

Before 23rd March (before the lockdown was enforced), the measured and modelled are similar, suggesting that the measured concentrations are comparable to the usual levels at this time of the year and under normal business activities. The modelled (i.e. ‘business as usual’) NO2 and NOx concentrations are predominately higher than the measured concentrations from 23rd March to July, which suggests that reduced emissions from traffic and industry are being seen in the measurements.

## Monthly Average Pollutant Concentrations

The plots below are box and whisker plots to show the distribution in monthly concentrations for each pollutant measured. The boxes represent the interquartile range (IQR) ranging from the lower quartile (25th percentile) to the upper quartile (75th percentile). The horizontal line within each box represents the median (50th percentile). The whiskers extend to the maximum and minimum values within the median ± (1.5 x IQR). Values outside the median ± (1.5 x IQR) are not shown here.

## Annual Average Pollutant Concentrations

The annual average concentrations for 2018 and 2019 are shown below for each site and pollutant, along with the BAU and measured averages for 2020.

2.2	Impacts in the London Borough of Tower Hamlets

Some projects / programmes continued in 2020 despite Covid-19 restrictions and lockdowns, while others were temporarily suspended and therefore affected and / or delayed.

More details can be found in each actions of the Air Quality Action Plan (Chapter 3 “Action to Improve Air Quality”). Here below are some examples.

Continuous monitoring continued, together with maintenance and monthly data uploading onto Council website. However, diffusion tube monitoring at all diffusion tube sites was temporarily suspended for 5 months, from March 2020 until July 2020 including.

LBTH website updated with latest AQ monitoring results: both monthly diffusion tube data, and AQ continuous monitoring data are entered onto LBTH website.

Air Quality inspections, complaints, NRMM scheme inspections, and anti-idling enforcement were physically suspended during lockdown, but they continued to be investigated/enforced remotely through regulatory controls in place. All complaints/investigation records were maintained and updated on APP database.

Publicity for smoke control area done as part of the Clan Air Day 2020 (8/10/2020) and continued over winter months.

Raising awareness of air pollution on social media was temporarily suspended to give priority to Covid-19 information and updates. However, work with Communication’s Team continued internally to disseminate information about air pollution, arrange and agree projects, update to Breathe Clean message, etc.

airTEXT awareness continued and provided air quality details during the lockdown to all residents registered.

The Anti-Idling Campaign in March 2020 was really impacted by Covid. It stopped in March 2020, and restarted in spring 2021. Workshop sessions to students were offered only online.

New anti-signs were procured in 2020 for primary schools and nurseries located in high pollution areas. Installation will occur in 2021. Schools and nurseries have already been contacted with offer of signs.

All major and minor planning applications continued to be reviewed by LBTH Environmental Health Department (Pollution Team) in respect to National Air Quality Strategy and the AQ objectives. The Pollution Team provides air quality comments and recommending relevant conditions.

The new Tower Hamlets Local Plan was adopted on 15th January 2020.

The Pandemic delayed the completion of Schools Carbon Emissions Reduction Programme. However, there is a regular communication with the schools to see how and when these projects can be delivered.

LEN (Low Emission Neighbourhood) Project, ZEN (Zero Emissions Network), Liveable Street programme, etc, were temporarily suspended during lockdown.

Lobbying and working with TFL to reduce emissions from buses in the borough, and to bring forward sooner TfL’s commitment to 100% buses by 2037 continued despite Covid.

# 3.	Action to Improve Air Quality

3.1	Air Quality Action Plan Progress

Table I provides a brief summary of the London Borough of Tower Hamlets progress against the Air Quality Action Plan, showing progress made this year.

Table I.	Delivery of Air Quality Action Plan Measures

| Measure | Action | Progress - Emissions/Concentration data - Benefits - Negative impacts / Complaints | Further information |
| --- | --- | --- | --- |
| 1. | Develop and implement a communications strategy for disseminating air quality information in the borough to raise awareness of the impacts of poor air quality and encourage behaviour change. | Ongoing. (1) Strategy was proposed and developed. AQ Partnership Board Meeting in Dec 2017: a presentation was made by the Comms Team. March 2020 meeting: the ‘Breath Clean’ Strategy’ was outlined. ‘Breathe Clean’ Strategy’s Objectives: - To define the School Streets project committed in the Mayor’s Manifesto, and to improve air quality at the school gate, particularly at entry /exit times, in order to improve the health of the pupils. - To improve road safety for vulnerable pedestrians, reducing congestion, encouraging walking and cycling and thus facilitating behavioural change in travel choices and increased physical activity. - To introduce school streets at 50 primary schools in the borough, outlining a proposed prioritisation methodology and identifying the first schools to be included in the programme. - To lobby Mayor of London and/or government for funding/changes to policy that will help Tower Hamlets improve local air quality. - The “Breathe Clean” Strategy will be implemented over 3 phases. - Phases 1 and 2 will raise awareness about air pollution and educate residents about what can be done to help reduce it locally and across London. - Phase 3 will lobby the government and the London Mayor for better support to reduce air pollution, and the negative health impacts it brings in Tower Hamlets (2) We also have the ongoing awareness raising campaign. The campaign needs to address two groups: (i) vulnerable people, and (ii) those who are causing air pollution – educating this second group is about bringing about behaviour change which is effective. (3) An anti-idling campaign in March 2020, really impacted by Covid: - March 2020, TH Council launched an anti-idling campaign - It included an anti-idling pledge that generated over 100 sign ups - As part of activity idling action days were organised at local schools, of which two took place - Information leaflets on idling were created and content about idling and harmful impacts were shared across key council channels including social media, resident e-newsletters, Our East End and the council website - The campaign was stopped towards the end of March due to the start of the Covid-19 pandemic which impacted ability to conduct activity, evaluate and review impact See also Action 10 and 63. | https://www.towerhamlets.gov.uk/lgnl/environment_and_waste/environmental_health/pollution/air_quality/Air_quality_information_and_campaigns/Breathe_Clean.aspx https://www.towerhamlets.gov.uk/lgnl/environment_and_waste/environmental_health/pollution/air_quality/Air_quality_information_and_campaigns/Anti_idling.aspx |
| 2 | Director of Public Health to have responsibility for ensuring their Joint Strategic Needs Assessment (JSNA) has up to date information on air quality impacts on the population – Air Quality officer to be consulted on JSNA. | JSNA specific reference to air quality published in 2016. | Measure Completed – No further action See: https://www.towerhamlets.gov.uk/lgnl/health__social_care/joint_strategic_needs_assessme/joint_strategic_needs_assessme.aspx |
| 3. | Strengthening co-ordination with Public Health by ensuring that at least one public health specialist within the borough has air quality responsibilities outlined in their job profile. | Public Health embedded into Local Air Quality. Three officers in Public Health including the Director of Public Health have a specific remit to work on Air Quality and they sit on all steering groups and boards that discuss air pollution. The Air Quality Steering Board has been integrated within the Public Health driven Health and Wellbeing Board. | Measure Completed – No further action |
| 4. | Director of Public Health to sign off all new Air Quality Action Plans. | Air quality action plan signed off by Public Health in 2017. | Measure Completed – No further action |
| 5. | Support patients with heart and lung conditions by providing air quality advice to discharged patients, particularly vulnerable & those with heart/lung conditions. This would be a continuation of the ‘Protecting Patient’ work stream from the Barts Project. | Public Health funded Barts to conduct a Clean Air Hospital Framework in 2019, in order for them to develop an action plan to better consider opportunities to reduce air pollution and improve understanding of the harms of poor air pollution on health. Action plan is completed. Barts to present action plan to HWB. Public Health also sponsored the Tower Hamlets CCG to develop a work programme in partnership with Global Action Plan to improve awareness and understanding of the health harms of air pollution for children with asthma. An awareness campaign was launched in 2020 by the CCG. Barts ran Protecting Patients as a Pilot and decided not to continue the programme post evaluation. It did demonstrate some success in engagement clinicians on the health harms of air pollution. Revised Action: “Public Health to work with NHS Partners to support vulnerable patients, specifically, those with lung and heart conditions” Barts decided to not continue with the ‘Protecting Patient’ workstream in 2017. Public Health have since worked with Barts to develop a Clean Air Hospital Framework, to help them develop an action plan to reduce air pollution inside and outside the Trust as well as support and training for health professionals to improve health outcomes for vulnerable patients. Furthermore, Public Health have worked with the NHS to develop an innovative programme of work to raise awareness of the harms of air pollution for children with asthma, specifically, to influence how health professionals can assist children in management of their underlying health conditions. |  |
| 6. | Support and Promotion of air quality awareness programmes such as AirTEXT | Annual airTEXT statistics for Tower Hamlets at the end of 2020: Number of Subscriber - Number of subscribers 306 - Number of new subscribers 21 - Number of airText Tower Hamlets alert days 36 - Number of emails 1,203 - Number of sms 4,461 - Number of voicemail 72 | AirText services is promoted on the Council's air quality webpage: https://www.towerhamlets.gov.uk/lgnl/environment_and_waste/environmental_health/pollution/air_quality/air_text_-_air_pollution_forec.aspx |
| 7. | Encourage schools to join the TfL Sustainable Travel Active Responsible Safer (STARS) accredited travel planning programme by providing information on the benefits to schools and supporting the implementation of such a programme. | Ongoing. Rolling out the school travel plan to 70% of primary schools within the Borough by 2021. Current proportion of schools is 50%. |  |
| 8. | Air quality at schools – Roll out the cleaner air for schools’ program that was previously run in 2 schools, to more schools in high pollution areas. | Contract with Loop labs for Bonner School Citizen Science programme complete June 2018. | Measure Completed – No further action |
| 9. | Pollution Audits in schools. Support the GLA in their program to provide air quality audits in 2 primary schools. | Two schools participated in GLA audit scheme: Marner Primary School, and Bonner Primary School. Reports received in 2018. GLA undertook air quality audits also in nurseries: Alice Model Nursery School and Columbia Market Nursery School. Subsequent baseline air quality monitoring undertaken and report outlining recommendations provided from audit findings. Reports received in 2019. | Measure Completed – LBTH also looking into potential school air quality audits. |
| 10. | Schools anti-idling project | (1) Anti-idling signs: 50 anti-idling signs were delivered to schools in 2018 for the schools to put them up. Further signs procured in 2020 and Pollution team will install signs at a suitable location outside interested schools in high pollution areas. More than 50 primary schools and nurseries have been contacted directly with offer of signs. (2) Vehicle Idling Action phase 4 – End of year 1 (October 19 – March 20): - 3 school air quality workshops were conducted to 79 KS2 children to create anti-idling banners - 1 Idling Action event took place at one school At Idling Action event: - 45% of drivers switched off when asked, 55% did not switch off - 100% of idling vehicles were cars - 25% of drivers spoken to already had their engines switched off - 67% pledged to switch off in the future - No of school workshops delivered 3 - No of students engaged 79 - Number of volunteers trained this year 5 - Number of volunteers who took part in events 5 (TH staff, parent, local XR members) - Number of Idling Action Events 1 - Number of interactions 2 - % of drivers who switched off 100% We were unable to meet our target of 2 Idling Action events for Year 1, due to Covid-19 restrictions. We continue to engage schools and deliver workshop sessions to students. We are currently producing virtual learning resources so that the workshops can be offered to schools whilst closures are preventing physical workshops taking place. In addition, resources will be made available so that when schools are back, they can carry out their own anti-idling campaign if it is not appropriate for us to run a larger scale event. (3) See also Action 1. | https://www.towerhamlets.gov.uk/lgnl/environment_and_waste/environmental_health/pollution/air_quality/Air_quality_information_and_campaigns/Breathe_Clean.aspx https://www.towerhamlets.gov.uk/lgnl/environment_and_waste/environmental_health/pollution/air_quality/Breathe_Clean/Report_engine_idling.aspx https://www.towerhamlets.gov.uk/lgnl/environment_and_waste/environmental_health/pollution/air_quality/Air_quality_information_and_campaigns/Anti_idling.aspx |
| 11. | Schools Environmental Theatre Project | Big Wheel Theatre company ran events on 20 primary schools in 2018. | Measure Completed – No further action |
| 12. | Investigate and invest in new technology as it becomes available to reduce pollution levels at pollution hotspots & sensitive uses e.g. schools | No new effective technology has become available. New funding available to install living green screens/walls at schools to reduce pollution for 2021/22 and 2022/23. Further new funding available in 2021/22 to implement small-scale green infrastructure within the borough to tackle air pollution. |  |
| 13. | Citizen Science air quality monitoring project | Two projects were funded by the Tower Hamlets Mayor’s Air Quality Fund (2018/19): (1) Project Title – ‘Clear the Air’ Description of project: The project provided AQ monitoring training and community organising training to LBTH residents from disadvantaged groups to bring the community together and to raise awareness of pollution and exposure. Residents put up NO2 diffusion tubes to understand their exposure. Children from a local school created a video about air pollution which was launched on ‘Clean Air Day 2019’. (2) Project Title – ‘Air Aware at Bromley-by-Bow’ Description of project - This project engaged members of the Bromley-by-Bow community to raise awareness of air pollution, empowering them to reduce their exposure and safeguard their health while protecting the environment. By working with health practitioners at the Bromley-by-Bow Health Centre to identify patients with respiratory conditions, the project targeted those who are most vulnerable to air pollution. A series of air pollution workshops for BBBC patients and clients using a citizen science approach were delivered. A small team of Clean Air Champions were recruited and trained to use NO2 and particulate monitors to measure pollution in the local area. The group analysed the data, communicated their findings and co-designed effective solutions to reduce exposure. An action plan was developed to broaden the impact and share the knowledge with the wider community. | Measure Completed – No further action |
| 14. | Work with Residential Providers to develop and implement a strategy for disseminating air quality information to their tenants. | Presentation to LBTH Housing Forum used to agree that all housing providers will cascade our messages and opportunities for residents through their regular newsletters / social media and notice boards. (The council will coordinate and cascade all messages). Liaison with housing providers to schedule publicity. | Measure Completed – No further action |
| 15. | Use Health and Wellbeing Board to get existing and future public sector and RP partners to pledge to increase the number of, electric, hybrid, and cleaner vehicles in their fleets. | The Air Quality Partnership Board was amalgamated with the Health and Wellbeing Board in 2019. The next update is due June 2021 when a report to the health & wellbeing board is expected. | In progress |
| 16. | Continue to run the 3 continuous monitoring stations, monitoring pollutants of concern to ensure air quality objectives are being met and to assess the effectiveness of local and regional policies. | 4 continuous monitoring stations are operating and maintained in the Borough: - Mile End (TH2): NO2, NO, NOx, PM2.5 - Blackwall (TH004): NO2, NO, NOx, PM2.5, PM10, O3 - Victoria Park (TH002): NO2, NO, NOx, PM2.5, PM10 - Millwall Park (TH001): NO2, NO, NOx, PM2.5, PM10, O3 | Data are available in this ASR and online: https://www.airqualityengland.co.uk/local-authority/?la_id=210 |
| 17. | Continue to implement the NOx Diffusion Tube Monitoring network across the borough. Investigate and implement further monitoring where necessary. E.g. at schools. | Borough-wide diffusion tube monitors maintained. Diffusion tube monitoring was temporarily suspended due to COVID lockdown but then resumed. Real time monitoring continuing. NOx tube raw data are loaded onto Council website monthly. | Data are available in the ASR and online: https://www.towerhamlets.gov.uk/lgnl/environment_and_waste/environmental_health/pollution/air_quality/Monitoring.aspx |
| 18. | Continue to ensure that all pollution monitoring data is available to the public and the website is regularly updated with the latest available data. | Ongoing work with Communication’s team internally to disseminate information, arrange and agree street and school signs, updates to Breathe Clean message, etc. LBTH website updated with latest AQ monitoring results: both monthly diffusion tube data, and AQ continuous monitoring data are entered onto LBTH website. Data are available in the ASR and online, on Council website. | LBTH website with monitoring all results: https://www.towerhamlets.gov.uk/lgnl/environment_and_waste/environmental_health/pollution/air_quality/Monitoring.aspx |
| 19. | Fulfil the GLA’s criteria to retain our Cleaner Air Borough Status each year | Cleaner Air Borough award retained in 2019. CAB for 2020 suspended due to COVID. |  |
| 20. | Ensuring emissions from demolition and construction are minimised via planning applications reviews and conditions attached to planning permissions requiring Construction Environmental Management Plans, including dust mitigation and monitoring and Travel Plans encouraging sustainable travel for site workers | Ongoing. Major and minor planning applications reviewed by LBTH Environmental Health Department (Pollution Team) in respect to air pollution and air quality. Pollution Team providing air quality comments and recommending relevant conditions. Adopted Policy: Local Plan 2013: DM9 (Improving Air Quality)/ Requires an Air Quality Assessment to demonstrate how it will prevent and reduce air pollution during construction and demolition. New Local Plan: D.ES2 (Air Quality). Development is required to meet or exceed air quality neutral and consider impacts of pollution during construction and operation of the Proposed Development. New Local Plan: D.TR4 (Sustainable Delivery and Servicing). Development that generates a significant number of vehicle trips for goods or materials during its construction and/or operational phases is required to demonstrate sustainable transport). New Local Plan: S.TR1 (Sustainable Travel). Travel choice (including connectivity and affordability) and sustainable travel will be improved within the borough and to other parts of London, and beyond. Development will therefore be expected to prioritise the needs of pedestrians and cyclists as well as access to public transport, including river transport, before vehicular modes of transport. New Local Plan Policy D.TR2 (Impacts on the Transport Network): Major development and any development that is likely to have a significant impact on the transport network will be required to submit a transport assessment or transport statement as part of the planning application. | New Local Plan: https://www.towerhamlets.gov.uk/lgnl/planning_and_building_control/planning_policy_guidance/Local_plan/local_plan.aspx See details on Table J ‘Planning requirements met by planning applications in Tower Hamlets in 2020’ of this ASR. |
| 21. | Ensuring all major developments adhere to the GLA’s Non Road Mobile Machinery Low Emission Zone. I.e. All NRMM used on site must meet the emissions standards stated in the GLA’s Control of Dust and Emissions during Demolition and Construction SPG 2014 (or subsequent updated guidance) | Ongoing. Major planning applications reviewed by LBTH Environmental Health Department (Pollution Team) in respect to air pollution and air quality. Pollution Team providing air quality comments and recommending relevant conditions. Relevant NRMM conditions recommended for each major site during planning consultations. Monthly planning decisions forwarded to LB Merton to feed into the London wide NRMM scheme for their inspections on follow up. CIL Tracker of when projects commence on site now also sent to the AQ Team, from Planning. New Local Plan: D.SG4 (Planning and Construction of a New Development), require construction to comply with NRMM low emission zone requirements and minimize air quality and dust pollution. | LBTH is part of the MAQF funded pan London NRMM enforcement project led by LB Merton. LBTH officers contribute to the scheme by providing regular site information. See details on Table J ‘Planning requirements met by planning applications in Tower Hamlets in 2020’ of this ASR. |
| 22. | Ensuring Combined Heat and Power (CHP) and biomass air quality policies are met at all developments proposing to utilise CHP, including the NOx emission limits for heating plant as stated in the GLA’s Sustainable Design and Construction SPG (or subsequent updated guidance). | Ongoing. Major planning applications reviewed by LBTH Environmental Health Department (Pollution Team) in respect to air pollution and air quality. Pollution Team providing air quality comments and recommending relevant conditions. Planning applications reviewed for CHP and relevant conditions recommended. Comments provided on major planning applications as required by GLA SPG on Sustainable Design and Construction. | See details on Table J ‘Planning requirements met by planning applications in Tower Hamlets in 2020’ of this ASR. |
| 23. | Ensuring new developments have suitable energy efficiency measures installed to reduce the demand for onsite heat generation from boilers & CHP’s. | 100% of schemes were policy compliant through either onsite reduction or meeting carbon reduction requirements through utilising the carbon offsetting mechanism with a S106 contribution. |  |
| 24. | Ensuring Air Quality Neutral policies are complied with at all developments and exceeded (improved) where possible. Ensure all larger developments (as defined by the GLA) will be air quality neutral | Ongoing. Major planning applications reviewed by LBTH Environmental Health Department (Pollution Team) in respect to air pollution and air quality. Pollution Team providing air quality comments and recommending relevant conditions. Air quality neutral policies adopted. Planning applications for major developments reviewed to ensure compliance with GLA air quality neutral policy. New Local Plan: D.ES2 (Air Quality). Development is required to meet or exceed air quality neutral and consider impacts of pollution during construction and operation of the Proposed Development. | New Local Plan: https://www.towerhamlets.gov.uk/lgnl/planning_and_building_control/planning_policy_guidance/Local_plan/local_plan.aspx |
| 25. | Reduce the use of private cars by residents by encouraging car free developments and limiting number of parking spaces in new developments. | Local Plan policy D.TR3 sets out a permit-free policy for new developments, encourages priority for cycle parking over car parking, and sets parking standards that minimise the amount of car parking spaces allowed in most parts of the borough. At most, a 3+ bedroom house in an area of PTAL 1-2 would be allowed 0.5 parking spaces – all other developments would be allowed fewer spaces than this. | New Local Plan: https://www.towerhamlets.gov.uk/lgnl/planning_and_building_control/planning_policy_guidance/Local_plan/local_plan.aspx |
| 26. | Ensure the layout of new developments considers air quality impacts, for example considering the locations of buildings with different proposed uses and locating the most sensitive use units in the least polluted areas | - Ongoing. All relevant major planning applications reviewed and assessed for compliance to the National Air Quality Strategy and the AQ objectives. AQ planning conditions have been reviewed and agreed between Planning and AQ Teams. Expected to be adopted mid-2021. |  |
| 27. | Ensuring adequate, appropriate, and well located green space and infrastructure, including for walking and cycling, is included in new developments with the Green Grid Strategy promoted and adhered to inall major planning applications and master planning to provide low emissions routes for walking and cycling. | The Green Grid Strategy formed part of evidence base for the new Local Plan. An SPD is currently being drafted to clarify the role and responsibilities towards the Green Grid within planning. The Green Grid Strategy identifies opportunities to improve connectivity between open spaces and community facilities to create a grid of walking routes and associated green infrastructure which provides a healthy and attractive environment for users. Strategic elements or projects for the Green Grid will be delivered through the Community Infrastructure Levy, including the healthy streets projects. The new Local Plan was adopted on 15th January 2020. Implementation of the Health Impact Assessment (HIA) policy within the Local Plan: HIA guidance now published and available on LBTH website. Improving HIA practice to aid consideration of adequate green spaces and low emission routes in planning applications: - Ongoing. - December 2020 - HIA Training planned for February 2021 to provide training to planning officers and across Place. | Green Grid Strategy: https://www.towerhamlets.gov.uk/Documents/Planning-and-building-control/Strategic-Planning/Local-Plan/Green_Grid_Update_2017.pdf Green Grid Strategy for City Fringe and other areas: https://www.towerhamlets.gov.uk/Documents/Planning-and-building-control/Strategic-Planning/Local-Plan/Green_Grid_Update_Appendicies_2017.pdf Local Plan and HIA available at Council website: https://www.towerhamlets.gov.uk/lgnl/planning_and_building_control/planning_applications/Making_a_planning_application/Local_validation_list/Health_Impact_Assessment.aspx |
| 28. | Encourage new developments to install alternative mass waste collection systems, such as ENVAC, to reduce collection vehicle emissions. | A new waste SPD is going to cabinet in April/May 2021. |  |
| 29. | Ensuring that the whole borough Smoke Control Zones is fully publicised and enforced. | Complaints/investigation records maintained and updated on APP database. Regulatory controls in place and investigated/enforced reactively through complaint investigation. Publicity for smoke control area done as part of the Clan Air Day 2020 (8/10/2020) and continued over winter months. A best practice leaflet aimed at canal boat owners has been produced and promoted via Council's webpage, social media, CRT and relevant boating associations. | Details available online at LBTH website: https://www.towerhamlets.gov.uk/lgnl/environment_and_waste/environmental_health/pollution/air_quality/Air_pollution/Smoke_control.aspx |
| 30. | Implement a Domestic boiler refit project using the GLA’s RE:FIT energy efficiency retrofit programme. | The Boiler Replacement Programme: Phase 1 - Delivered - Phase 1, which had a budget of £250,000, has supported more than 80 households to install new energy efficient boilers. Phase2 - Phase 2 has just begun and has a funding of £600,000 |  |
| 31. | Implement a Schools Carbon Emissions Reduction Programme providing funding towards boiler replacement and insulation schemes in schools. | Schools Carbon Emissions Reduction Programme organised in three phases. Phase I: completed in November 2019. 9 schools received grants for energy efficiency measures, namely; Olga Primary, Osmani Primary, Cubitt Town Infants, Cubitt Town Juniors, Lansbury Lawrence, Lawdale Juniors, Wellington Primary, Oaklands, Arnhem Wharf. Phase II: This phase of the Programme has now had four of the eight projects completed with the remaining four still being supported to get their projects completed. These projects have been delayed due to the pandemic so were unable to be completed over the summer holidays as originally intended. Jan 2021.The Pandemic continues to delay the completion of these projects. The aim is to have projects completed by Easter. However, with schools now closed again this may be delayed further. There is regular communication with the schools to see how and when these projects can be delivered. |  |
| 32. | Implement a Carbon Emissions Reduction Programme for council properties including boiler replacements and insulation projects. | As part of our net zero carbon Council by 2025 target we have developed an action plan which includes Council assets, we are preparing bids for external funding. We are already delivering the boiler replacement and insulation programme for residents. The Council has secured £721,000 from the Public Sector Decarbonisation Fund, this money will be matched funded by the Council and a programme of works total £1.4M will be delivered to four council buildings, works will include decarbonisation of the heating system and lighting upgrades. This project is be completed by September 2021. We have also surveyed four of our council offices for Solar PV and a programme of installation will begin in 2021 and 2022. |  |
| 33. | Enderby Wharf – Ensure a thorough and robust evaluation of the Environmental statement, that methodologies used comply with current guidance and that the project will not lead to any significant adverse air quality impacts in the borough. | Application for cruise terminal withdrawn by developers. | Measure Completed – No further action |
| 34. | Ensure applications for new developments in neighbouring boroughs that have the potential to have impacts in Tower Hamlets are reviewed for air quality impacts and that no development will lead to any significant adverse air quality impacts in the borough. | Ongoing. Major and minor planning applications reviewed by LBTH Environmental Health Department (Pollution Team) in respect to air pollution and air quality. Pollution Team providing air quality comments and recommending relevant conditions. In October 2020, LBTH was consulted on a Scoping Opinion request (PA/20/02195) for a proposed development to replace Enerby Wharf (15/0973/F), which LBTH previously objected to on air quality ground. |  |
| 35. | Lead by example by ensuring the councils new Civic Centre is a best practice example of a sustainable and low emissions development in regard to air pollution and CO2 with both air quality neutral and carbon zero policies being met. | New council building planning application PA/17/02825/A1 approved, with conditions securing environmental information on: - Construction Environmental Management Plan (CEMP) - Full Travel Plan (to be approved in consultation with TfL) - BREEAM excellent target as a minimum, BREEAM outstanding to be targeted wherever possible - Demolition and Construction Dust monitoring procedure and reporting Approved details submitted with application: - AQ impact assessment submitted and approved, including AQ neutral assessment - Sustainability (carbon) and energy statement approved - Construction Logistics Plan approved - Transport Assessment appr | Measure Completed – No further action New LBTH offices due to open 2022: https://www.towerhamlets.gov.uk/lgnl/council_and_democracy/council_departments_and_offic/A_new_civic_centre.aspx |
| 36. | Improve the energy efficiency of John Onslow House as part of the upcoming refurbishment with the aim of becoming carbon zero and any new boilers to be ultra-low NOx. | New Remeha Quinta Pro and Vaillant Ecotec boilers installed. | Measure Completed – No further action |
| 37. | Ensure developments that will increase river traffic, in the operational phase of development, are thoroughly assessed for potential air quality impacts and will not have a significant negative impact on air quality. | Ongoing. Major and minor planning applications reviewed by LBTH Environmental Health Department (Pollution Team) in respect to air pollution and air quality. Pollution Team providing air quality comments and recommending relevant conditions. Planning applications which may have an impact on air pollution on/from the River will be reviewed, even inter-borough applications. No application in 2020 that would increase river traffic. |  |
| 38. | Ensure the Tideway Tunnel infrastructure project is sustainably delivered with the Construction Code of Practice adhered to and effective emissions mitigation in place during construction & operational phases. | Ongoing. LBTH did not attend forum meetings in 2020 as it was deemed unnecessary. No reported/complaints of air quality issues. CoCP is still maintained and air quality/dust is monitored during construction. LBTH Environmental Health Department (Pollution Team) reviews the AQ monitoring data. Monitoring results provided for Thames Tideway Tunnel are significantly below actionable levels. Testing and commissioning of tunnel due 2022. |  |
| 39. | Silvertown Tunnel – Ensure a thorough and robust evaluation of the Environmental statement, that methodologies used comply with current guidance and that the project, during both the construction and operational phases, will not lead to any significant adverse air quality impacts in the borough and that adequate mitigation is provided for any potential impacts. Ensure traffic modelling on which the air quality statements are robust. | Development Consent Order granted May 2018. A review of the ES was carried out as part of the DCO planning process Virtual meetings have been held with TfL to discuss monitoring locations for air quality monitors during construction phase. Some diffusion tube monitors are to be installed in LBTH. In November 2020, final locations for monitoring in LBTH for 8 diffusion tubes agreed with TfL. |  |
| 40. | Ensure that all future major infrastructure projects are adequately reviewed and assessed through the planning process to ensure impacts on air quality are minimised. | Ongoing December 2020 - No NSIPs or TWOA projects likely to effect LBTH have come forward https://infrastructure.planninginspectorate.gov.uk has been reviewed and no projects that are likely to effect LBTH are planned at present. Major planning applications reviewed by LBTH Environmental Health Department (Pollution Team) in respect to air pollution and air quality. Pollution Team providing air quality comments and recommending relevant conditions. |  |
| 41. | Ensure that Procurement policies to include a requirement for suppliers with large fleets to have attained, silver as a minimum or gold as a preference, Fleet Operator Recognition Scheme (FORS) accreditation or equivalent. | Both Action 41 and Action 42 have been covered off in the single policy document. However, with the merging of Air Quality officer steering group and NZC (Net Zero Carbon), the policy is currently being reviewed again with a greater scope (more of a Social, Economic and Environmental an all-encompassing policy) with an intended roll out of September 2021 which will focus on assisting the council with reaching the NZC status for 2025. Local authority Procurement policies to include a requirement for suppliers with large fleets (10 or more) to have attained silver FORS accreditation or equivalent. | Measure Completed – No further action |
| 42. | Investigate updating Procurement policies to ensure sustainable logistical measures are implemented (and include requirements for preferentially scoring bidders based on their sustainability criteria). | Both Action 41 and Action 42 have been covered off in the single policy document. However, with the merging of Air Quality officer steering group and NZC (Net Zero Carbon), the policy is currently being reviewed again with a greater scope (more of a Social, Economic and Environmental an all-encompassing policy) with an intended roll out of September 2021 which will focus on assisting the council with reaching the NZC status for 2025. | Measure Completed – No further action |
| 43. | Investigate re-organisation of freight to support consolidation (or micro-consolidation) of deliveries, by setting up or participating in new logistics facilities, and/or requiring that council suppliers participate in these. | With part-funding from GLA’s Business Low Emission Neighbourhood (BLEN) programme, the regeneration team are working on proposals to develop a market storage and consolidation site at Whitechapel. Feasibility and engagement work has been completed with a planning application anticipated during summer 2021. |  |
| 44. | Investigate implementing a local Eco Stars Fleet Recognition Scheme for Tower Hamlets. | Actions 41, 42 and 44 have been covered off in the single policy document. However, with the merging of Air Quality Board and NZC, the policy is currently being reviewed again with a greater scope (more of a Social, Economic and Environmental an all-encompassing policy) with an intended role out of September 2021 which will focus on assisting the council with reaching the NZC status for 2025. Local authority Procurement policies to include a requirement for suppliers with large fleets (10 or more) to have attained silver FORS accreditation or equivalent. | Measure Completed – No further action |
| 45. | Join a recognised appropriate driver award scheme, e.g. Fleet Operator Recognition Scheme (FORS) or Van Excellence & achieve certification. | We have joined FORS and have a working group forming to work to bronze accreditation. | Measure Completed – No further action |
| 46. | Increasing the number of, electric, hybrid, and cleaner vehicles in the boroughs’ fleet. | All waste vehicles are Euro VI diesel (circa 100). 39 passenger vehicles are being replaced this year which are Euro VI, ten temporary passenger vehicles are also Euro VI. Circa 40 vehicles (cars/vans) have had replacements ordered which are currently being delivered, the majority are Euro VI diesel but some are Euro VI petrol and one is electric. |  |
| 47. | Accelerate uptake of new Euro VI vehicles in borough fleet, ending the purchase of diesel vehicles where feasible. | All waste vehicles are Euro VI diesel (circa 100). 39 passenger vehicles are being replaced this year which are Euro VI, ten temporary passenger vehicles are also Euro VI. Circa 40 vehicles (cars/vans) have had replacements ordered which are currently being delivered, the majority are Euro VI diesel but some are Euro VI petrol and one is electric |  |
| 48. | Real-time Telematics monitoring of fleet driver behaviour and subsequent driver training. | All waste vehicles are fitted with CMS Supatrak, new vehicles for other departments are arriving with Supatrak fitted training is still to be carried out. Nearly all the fleet now has telematics fitted monitoring driver behaviour and fuel usage. Training has not progressed as planned due to COVID, but anti idling training is planned to take place, as well as training related to FORS in 2021. | https://www.supatrak.com/driver-eco-coaching/ https://www.supatrak.com/driver-behaviour/ |
| 49. | Utilise round optimisation for council fleet to reduce vehicle miles. | Project and action plan for waste and cleansing route optimization being drafted for sign off by end of March 2021 Route optimisation savings plan presented to Transport Demand Board. |  |
| 50. | Procure a cargo bike for regular delivery of literature to councillors. | Parks Dept already have 2 cargo bikes in use in Victoria Park. Councillors now provided with tablets to facilitate delivery of literature. | Measure Completed – No further action |
| 51. | Project 2020: use the procurement process to ensure all waste & Recycling collection vehicles in the new contract are as low emission as possible by prioritising tenders with the highest proportion of low emission vehicles. | New recycling collection vehicles purchased in 2019 are all Euro VI. Waste management strategy approved Deliver vehicle replacement (Procurement comment; All vehicles produced and sold in the EU after September 2015 are euro 6 certified 'ULEZ compliant'). | Measure Completed – No further action |
| 52. | Project 2020: utilise round optimisation to reduce vehicle mileage for waste collections. | Round optimisation fleet movements for Refuse Collection Vehicles reviewed annually or whenever the service changes. Reduced mileage always sought by operations to save on fuel and services which directly influences fuel use. | Measure Completed – No further action |
| 53. | Reduce ‘Grey Fleet’ impacts by reviewing staff parking permits to reduce number or allocate shared team permits rather than individual. | Staff essential car user eligibility reviewed under terms and condition review. Commenced in 2020. | Measure Completed – No further action |
| 54. | Investigate installing Green Infrastructure, such as green walls, green screens or living roofs at schools/residential developments in polluted areas. Linking in with the Green Grid and Open Spaces Strategy. | Various installations of living green walls will start in 2021-2022 (Section 106 funds), and in 2022-2023 (LIF Project funds) at schools located in areas where nitrogen dioxide concentration levels exceed national objectives. LIF funding secured to deliver living green walls/screens at schools exceeding legal limit for nitrogen dioxide. LIF Project BID prepared. |  |
| 55. | Low Emission Neighbourhoods (LENs) – implement the City Fringe LEN in partnership with Hackney and Islington. | Borough received funding for a business LEN in Whitechapel and officers are currently developing the Project Action Plan. LENs replaced with Liveable Street programme. | https://www.towerhamlets.gov.uk/lgnl/community_and_living/Liveable_Streets.aspx Here are the final designsand consultation results for Bethnal Green: https://www.pclconsult.co.uk/liveablestreetsbethnalgreen/consultation |
| 56. | Engagement with businesses – Continuation of the ZEN Project engaging businesses with advice and grants to enable them to reduce their air quality impact. | - ZEN held an online event “The Green Economy in Action” on 24 September. The international digital webinar was attended by organisations from 5 different countries. - 2020 figures of new business and residents signed up to ZEN, include - 190 total members recruited in Tower Hamlets in 2020 - 58 measures completed - 2 grants awarded | https://www.towerhamlets.gov.uk/lgnl/environment_and_waste/environmental_health/pollution/air_quality/Air_quality_information_and_campaigns/What_are_we_doing/Zero_Emissions_Network.aspx https://zeroemissionsnetwork.com/ |
| 57. | Discouraging unnecessary idling by taxis, coaches and other vehicles. Anti–Idling engagement project focusing on air pollution hotspots and high-risk locations such as hospitals and schools. | Anti-idling enforcement visits suspended due to COVID. Enforcement visits undertaken until 12/03/2020. For anti–idling engagement project focusing on schools, see Action 10 for Idling Action Campaign (vehicle and schools). | https://www.towerhamlets.gov.uk/lgnl/environment_and_waste/environmental_health/pollution/air_quality/Breathe_Clean/Report_engine_idling.aspx https://www.towerhamlets.gov.uk/lgnl/environment_and_waste/environmental_health/pollution/air_quality/Air_quality_information_and_campaigns/Anti_idling.aspx https://www.towerhamlets.gov.uk/lgnl/environment_and_waste/environmental_health/pollution/air_quality/Air_quality_information_and_campaigns/Air_pollution_and_idling.aspx |
| 58. | Enforce anti-idling regulations by becoming a designated authority to issue Fixed Penalty Notices to idling drivers. | Fixed Penalty Notices enforcement mechanism now operational since 2018 for idling vehicles. Adopted powers to issue FPN’s for idling, under the Road Traffic Regulations 2002. Enforcement visits undertaken by Council staff weekly in 2020, when Covid-19 restriction permitted. Refer to action 57. | Measure Completed – No further action https://www.towerhamlets.gov.uk/lgnl/environment_and_waste/environmental_health/pollution/air_quality/Air_quality_information_and_campaigns/Anti_idling.aspx https://www.towerhamlets.gov.uk/lgnl/environment_and_waste/environmental_health/pollution/air_quality/Air_quality_information_and_campaigns/Air_pollution_and_idling.aspx |
| 59. | Increasing the proportion of electric, hydrogen and ultra-low emission vehicles in Car Clubs. | An initial report on the status of car clubs has been considered by the Mayor and Lead Member in the Parking Policy Review Group. The proposal moving forward is to undertake a wider review of the changing need, operating models elsewhere and contract options to require certain things, including EV. The end result being a 'Tower Hamlets' model that could be put to market, focussed on the needs of TH residents and businesses, with standards and KPI monitoring by the Council. Currently unable to move forward due to limited resources and infrastructure of EV charging points. However, whilst one car club has stopped trading, we have a new one signed up to start working in the borough in 2021. All car clubs are operating with hybrid vehicles. |  |
| 60. | Review parking permit fee banding to encourage lower emission vehicle choice or add an additional diesel surcharge to existing permit fees. | Review of parking permits completed; now includes banding which has lowest parking fee for low emissions vehicles, and also greatest fee chargeable for diesel vehicles: - Discount on Electric Vehicles introduced in April 2019 - Levy on Diesel vehicles introduced in October 2019 Review of parking permits completed on annual basis. In 2020, the Mayor and Lead Member in the Parking Policy Review Group agreed to take forward proposals to introduce a Levy on Diesel Vehicles using paid-for-parking. This was considered as part of the Annual Fees & Charges Review and approved. It was delayed due to Covid-19, but it has now been implemented. | Measure Completed – No further action |
| 61. | Installation of residential electric charge points. | In March and April 2021, 94 new lamp post charging points added to an existing network of 42 charging points (20 lampposts and 22 fast charging). The total network is now 136 and an additional 78 are due to be installed by the ned of April taking the total to 204. We are working on a new EV strategy which will set out a new delivery plan for the borough from 2021-25. This is due to be launched in Spring/Summer 2021. |  |
| 62. | Installation of rapid chargers to help enable the take up of electric taxis, cabs and commercial vehicles (in partnership with TfL and/or OLEV) | TfL Rapid charger funding is no longer available to boroughs so we will seek to run a procurement exercise in 2021 for new rapid chargers. The number of points will be determined by our new EV Strategy. |  |
| 63. | Investigate reprioritisation of road space to smooth traffic flow, reduce congestion, improve bus journey times, cycling and pedestrian experience, and reduce emissions caused by congested traffic | - School Streets worked on since May 2019 - 50 Schools Streets in total in 2021 and 2022 - 21 Schools Streets to be provided in 2020: - 5 School Streets in place by Sept 2020 - 9 delayed due to schools’ closures in Lockdown - 20 Schools Streets to be provided in 2021: 35 School Streets to be added by Sept 2021 (40 in total - all commenced work, and 4 of these completed by Nov 2020) - 10 Schools Streets to be provided in 2022 See also Action1. | https://www.towerhamlets.gov.uk/lgnl/transport_and_streets/roads,_highways_and_pavements/School_Streets.aspx |
| 64. | Continue to provide/ ensure provisions of infrastructure to support walking and cycling including on street residential secure parking lockers, cycle routes, cycle permeability schemes, traffic management area reviews. | - Seventeen Liveable Street Areas, covering half of the borough. Three started in April 2019, and two Started in October 2019 - 3 Liveable Streets, with works started onsite, due to be completed June 2021 - 5 Liveable Streets in engagement stages - 4 due for completion of build by March 22 and 1 by July 22 |  |
| 65. | Reduce traffic in the borough through the development of a new Local Implementation Plan in line with the Mayors Transport Strategy. | Internal Transport Strategy workshop undertaken in January 2019, integrating Environmental Health, Transport, Highways and Public Health. Specific outcome and priority of the Transport Strategy is to Improve air quality and make our surroundings quieter, more inviting and more appealing. | Measure Completed – No further action |
| 66. | Continue to encourage staff sustainable travel by providing Dr Bike services and staff subscriptions to the TFL cycle hire scheme for site visits. Annual update of the Staff Travel Plan to ensure it remains relevant and proactive. | Ongoing. This year's Staff Travel Plan is postponed due to the impact of Covid 19 of staff travel patterns and ongoing uncertainty regarding the Council's staff accommodation strategy. |  |
| 67. | Push for Tower Hamlets to be included in the ULEZ through partaking in the TFL Consultation process. | LBTH contributed to ULEZ consultations. ULEZ will include all LBTH when it is extended to North Circular in October 2021. | Measure Completed – No further action |
| 68. | Ensure responses to all government and regional consultations focus on reducing or eliminating emissions of Local air pollutants and CO2. | On-going. Responses to consultations undertaken where relevant No national/regional consultation was undertaken in 2020. |  |
| 69. | Lobby and work with TFL to reduce emissions from buses in the borough. e.g. through green bus corridors. Work with other statutory Services to reduce emissions – LFB, NHS etc. | Lobby continues to bring forward sooner TfL’s commitment to 100% buses by 2037. A11 is a designated green bus corridor. Introduction of an electric bus on route 100: London bus route 100 was converted to electric traction on 18th January 2020, the 100 runs from near St Paul's Cathedral to Shadwell in London's East End via the edge of the City of London on London Wall, across Bishopsgate, on to Aldgate and Tower Gateway, near the Tower of London and Tower Bridge, on to Wapping the former dock area by the river Thames, and then turns away from the river to Cable Street, Shadwell, near the Shadwell Overground and Docklands Light Railway stations. The buses used by Go-Ahead London are 9.7m BYD/Alexander Enviro 200EV, they are the first buses fitted with the audible warning system for electric buses which cuts in at speeds lower than 12 mph, another innovation is the use of cameras instead of wing mirrors, there is a vertical oblong screen on the left hand side of the cab that shows both sides of the bus. Ultra-Low Emission Zone bus corridors to be implemented by Tfl: Discussions with TfL are continuing as TfL develop plans for practical implementation of electric buses in borough. |  |
| 70. | Lobby and work with TFL to reduce emissions from TfL controlled roads e.g. through reprioritisation of road space. | Ongoing. Officers are discussing TfL proposals for safety improvements on the A13 between New Street and Jubilee Street. This is a priority scheme for TfL despite current funding constraints. |  |
| 71. | Lobby the GLA to strengthen their Air Quality Neutral Policy and lower the CHP emission limits in current guidance. | New Draft Environment Strategy and Draft London Plan seeks to have largest developments required to be air quality positive. Further guidance awaited from the GLA following adoption of the new London Plan. The new London Plan proposes a positive air quality approach. | Measure Completed – No further action |
| 72. | Delivery of the Tower Hamlets Mayor’s Air Quality fund over 2018/19 and 2019/20 | All grant funding from the Tower Hamlets Mayor’s air quality fund has now been allocated. All phase 1 work completed and some phase 2 work still ongoing. Final close out reports will be requested. 13 applications approved: - East End Homes - Swapping to battery operated gardening equipment - Coffe Afrique - Promoting AQ awareness through school assemblies and community events. - Women’s Environmental Network - Running participatory workshops and forum events focusing on raising awareness among women. Aiming to develop a toolkit for future replication - Citizens UK - Engage with locals to ascertain barriers to behaviour change; teach people how to engage with the LA to report and improve AQ - Bromley-by-Bow Centre - Target vulnerable people through links to the health centre. Recruit AQ champions, teach them how to monitor NOX and PM and develop a strategy based on findings - Trees for Cities - Planting trees and green screens - Barts Health - An anti-idling campaign focused on ambulance drivers and minicabs - Shapla Primary School - Installing a green wall around Shapla Primary - Barts Health - Training health professionals to teach their patients about air pollution and how to reduce their emissions andexposure - Poplar HARCA - Purchasing cargo bikes for Poplar Works fashion hub - Sustrans - Promoting sustainable transport at four schools on the cityfringe - CWC Environmental - Low pollution walking maps for Bow Schools - St Paul's Way Primary School -Promote modal shift at St Paul's Way Trust School | Measure Completed – No further action |
| 73. | The Mayor of Tower Hamlets to hold a meeting with The Royal Borough of Greenwich and Greater London Authority to discuss reducing the environmental impact of the proposed Enderby Wharf cruise terminal. Lobby for shore-side power to be provided for the ships. | Meeting held. Liaison between officers of LBTH & RBG to control pollution. Shore side power not feasible. Application for cruise terminal withdrawn by developers. Action deemed to be successful as proposal is now not going ahead as planned. | Measure Completed – No further action |
| 74. | Work with the Canal & River Trust, the GLA and other Boroughs with canals to devise a plan to best tackle issues with emissions from canal boats. Enforcement action to be taken where necessary. | Tower Hamlets initiated a canal engagement meeting involving boroughs with inland navigable waterways and the Canal & River Trust about the need for a cohesive London-wide engagement campaign focused on sustainability and air quality on the canals. An awareness and engagement campaign for canal boat owners on pollution reduction, as planned, could not be run in 2020. The awareness campaign jointly with LB Newham and LB Hackney started early 2021. Joint bid with partners to review electric charging points for canal boat owners: - Engagement work with CRT continuing. - A canal engagement group consisting of London boroughs with canals has been formed with CRT represented. - A London wide best practice leaflet developed for handing out to boat owners with consistent messaging. - Further meetings suspended due to COVID. - In November 2020, a single LBTH grant bid submitted to DEFRA for charging points in LBTH. |  |
| 75. | Support the Port London Authority in the development and implementation of their Air Quality Strategy for the River Thames. | Presentation by PLA to Council’s Air Quality Partnership Board on their new strategy. Working with PLA to locate air quality monitors to assess the impact of existing Greenwich cruise liner terminal Monitors deployed by PLA in 2019. | Measure Completed – No further action |
| 76. | Support the GLA in Lobbying national Government to provide new powers and improved coordination for river and maritime vessels, including having a single regulatory authority for the Thames and London tributaries and introduce minimum emissions standards | Liaison with PLA Representative specifically on the production of guidance for developers and boroughs in 2018, as well as proposals for funds improving emissions from the marine sources within the borough. Attendance to the future workshops on the progress for the strategy. Maritime emissions addressed in latest DEFRA consultation. | Measure Completed – No further action |

# 4. 	Planning Update and Other New Sources of Emissions

Table J.	Planning Requirements Met by Planning Applications in the London Borough of Tower Hamlets in 2020

| Condition | Number |
| --- | --- |
| Number of planning applications where an air quality impact assessment was reviewed for air quality impacts | 19 |
| Number of planning applications required to monitor for construction dust | 15 |
| Number of CHPs/Biomass boilers refused on air quality grounds | 0 |
| Number of CHPs/Biomass boilers subject to GLA emissions limits and/or other restrictions to reduce emissions | 0 |
| Number of developments required to install Ultra-Low NOx boilers | 1 |
| Number of developments where an AQ Neutral building and/or transport assessments undertaken | 6 |
| Number of developments where the AQ Neutral building and/or transport assessments not meeting the benchmark and so required to include additional mitigation | 2 |
| Number of planning applications with S106 agreements including other requirements to improve air quality | 0 |
| Number of planning applications with CIL payments that include a contribution to improve air quality | 0 |
| NRMM: Central Activity Zone and Canary Wharf Number of conditions related to NRMM included. Number of developments registered and compliant. Please include confirmation that you have checked that the development has been registered with the GLA through the relevant NRMM website and that all NRMM used on-site is compliant with Stage IIIB of the Directive and/or exemptions to the policy. | 4 conditions included 10 registered and compliant 0 unregistered/uncompliant and being chased |
| NRMM: Greater London (excluding Central Activity Zone and Canary Wharf) Number of conditions related to NRMM included. Number of developments registered and compliant. Please include confirmation that you have checked that the development has been registered at www.nrmm.london and that all NRMM used on-site is compliant with Stage IIIA of the Directive and/or exemptions to the policy. | 16 conditions included 4 registered and compliant 0 unregistered/uncompliant and being chased |

Commentary

All major planning applications are referred to the Pollution Team for comment. Each application is individually reviewed to ensure that the GLA SPGs on the ‘Sustainable Design and Construction’, as well as ‘The Control of Dust and Emissions During Construction and Demolition’, are in compliance.

Where there are compliance issues, Pollution Officers recommend either further information to be obtained from the applicant, or relevant conditions recommended.

It is at the discretion of the Development Control Team/Planning Committee as to what action is taken on recommendations made by the Pollution Team.

4.1	New or significantly changed industrial or other sources

No new sources identified in 2020.

# Appendix A	Details of Monitoring Site Quality QA/QC

A.1	Automatic Monitoring Sites

Calibrations at Tower Hamlets Roadside, Millwall Park and Victoria Park are undertaken by Ricardo Energy and Environment. Millwall Park and Victoria Park are both urban background sites, so they calibrated every 4 weeks. Tower Hamlets Roadside is calibrated every 2 weeks. All sites are provided with ISO 17025 QC audits by Ricardo Energy and Environment every 6 months.

Note: the Blackwall site is operated by Transport for London, not LBTH.

PM10 Monitoring Adjustment

Millwall Park – 1020 Heated BAM, correction applied Victoria Park – TEOM, VCM correction applied Both VCM and BAM correction is applied automatically when data is downloaded from Air Quality England web site.

A.2	Diffusion Tubes

• Lab supplying and analysing the tubes:

SOCOTEC Unit 12, Moorbrook, Southmead Industrial Park Didcot OX11 7HP

• Preparation method used:

The tubes were prepared by spiking acetone:triethanolamine (50:50) onto the grids prior to the tubes being assembled. The tubes were desorbed with distilled water and the extract analysed using a segmented flow autoanalyser with ultraviolet detection

• Confirmation that the lab follows the procedures set out in the Practical Guidance:

The samples have been analysed in accordance with SOCOTEC’s standard operating procedure ANU/SOP/1015 Issue 1. This method meets the guidelines set out in DEFRA’s ‘Diffusion Tubes For Ambient NO2 Monitoring: Practical Guidance.’

• Results of laboratory precision results:

This analysis of diffusion tube samples to determine the amount of nitrogen dioxide present on the tube is within the scope of our UKAS schedule. Any further calculations and assessments requiring exposure details and conditions fall outside the scope of our accreditation. In the AIR PT intercomparison scheme for comparing spiked Nitrogen Dioxide diffusion tubes, SOCOTEC currently holds the highest rank of a ‘Satisfactory’ laboratory.

Factor from Local Co-location Study

We have two diffusion tubes (42 Victoria Park co-location site, and 43 Victoria Park co-location site) co-located with the automatic monitoring site Victoria Park TH002.

As per the LLAQM Technical Guidance 19, we have calculated the local bias-adjustment factor from our co-location study (Table K).

Table K.	Local Bias-adjustment Factor

Discussion of Choice of Factor to Use

Using diffusion tubes as part of our review and assessment, we are reporting in this ASR both the local bias-adjustment factor from our co-location study, and the bias-adjustment factor from the national database:

- The local bias-adjustment factor is 0.82(see previous paragraph for calculations)

- The national bias-adjustment factor is 0.77as per 03/2021(SOCOTEC Didcot, 50% TEA in acetone)

The decision of which of the two factors to use depend upon a number of factors that will need to be considered.

Ultimately, we have chosen the national bias-adjustment factor versus the local bias-adjustment factor for the following reasons:

- The length of the monitoring study: Due to Covid-19 lockdown, in 2020 the co-location study consists of just 7 monitoring periods. Therefore, we think that the co-location study does not have valid diffusion tube measurements, and that the local bias-adjustment factor found is not representative. It is stated in the LLAQM.TG(19) that the duration of the whole diffusion tube study is not representative if it is less than one year (especially, if it is less than nine monitoring periods).

- Less than 75% data capture from the analyser: In one of the 7 monitoring periods, data capture from the analyser is 65.60%, thus making a poor data capture.

- The diffusion tube exposure periods are not weekly or fortnightly (they are monthly, as the national database of co-location results).

A.3	Adjustments to the Ratified Monitoring Data

Short-term to Long-term Data Adjustment

Data capture rate of 75% was not achieved at any nitrogen dioxide diffusion tube sites in 2020 due to Covid-19. Consequently, it has been necessary to annualise the results of all sites in accordance with the procedure described in LAQM TG (19). Calculations for each site are as follows.

Table L.	Short-Term to Long-Term Monitoring Data Adjustment for all 90 diffusion tubes

Distance Adjustment

All NO₂ diffusion tube results have been adjusted to estimate the concentrations at the nearest receptor, using the LAQM NO2 Fall-off with Distance Calculator (Version 4.2) in line with the procedure specified in LLAQM.TG (19).

The methodology consists of comparing the monitored NO₂ annual mean concentrations at a given point against known relationships between NO₂ concentrations and the distance from a road source.

As background concentration, the NO₂ annual mean concentration from the automatic monitoring site Victoria Park TH002 was used.

See Table M for the NO2 fall off with distance calculations.

Table M.	NO2 Fall off With Distance Calculations

| Site Name/ID | Monitoring Site to Kerb Distance (m) | Receptor to Kerb Distance (m) | Background NO2 Annual Mean Concentration (µg/m3) | Monitored at Site NO2 Annual Mean Concentration (µg/m3) | Predicted at Receptor NO2 Annual Mean Concentration (µg/m3) | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| 1 Colombia Rd / Gossett Street | 0.5 | 5.5 | 17.0 | 24.9 | 21.6 |  |
| 2 Calvert Ave / Boundary Street | 0.5 | 4.5 | 17.0 | 26.0 | 22.5 |  |
| 3 Bethnal Green Rd / Brick Lane | 0.5 | 3.5 | 17.0 | 27.4 | 23.8 |  |
| 4 Commercial St / Calvin St | 0.5 | 7.5 | 17.0 | 34.2 | 26.0 |  |
| 5 Whitechapel High St (KFC) | 0.5 | 3.5 | 17.0 | 33.9 | 28.1 |  |
| 6 Mansell St | 0.5 | 6.5 | 17.0 | 35.2 | 26.9 |  |
| 7 St Katherine's Way | 10.0 | 20.0 | 17.0 | 19.9 | 19.1 |  |
| 8 Wapping High St / Sampson St | 0.5 | 3.5 | 17.0 | 21.6 | 20.0 |  |
| 9 Cartwright Street | 0.5 | 5.5 | 17.0 | 24.7 | 21.4 |  |
| 10 Whitechapel Rd / Adler St | 0.5 | 6.5 | 17.0 | 28.9 | 23.5 |  |
| 11 Brick Lane / Princelet St | 0.5 | 5.5 | 17.0 | 24.4 | 21.3 |  |
| 12 Buckfast St / Bethnal Green Rd | 0.5 | 4.5 | 17.0 | 24.2 | 21.4 |  |
| 13 Squirries St / Gosset St | 0.5 | 4.5 | 17.0 | 27.1 | 23.2 |  |
| 14 Warner Place / Hackney Rd | 0.5 | 17.5 | 17.0 | 25.5 | 20.2 |  |
| 15 Parmiter St / Cambridge Heath Road | 0.5 | 4.5 | 17.0 | 30.3 | 25.1 |  |
| 16 Paradise Row / Bethnal Green Rd | 0.5 | 3.5 | 17.0 | 28.1 | 24.3 |  |
| 17 Finnis St / Three Colts Lane | 0.5 | 2.5 | 17.0 | 21.2 | 20.0 |  |
| 18 Sidney St / Mile End Rd | 2.0 | 8.0 | 17.0 | 29.3 | 25.3 |  |
| 19 Philpot St / Commercial Road | 0.5 | 8.5 | 17.0 | 31.0 | 24.0 |  |
| 20 Dellow St / The Highway | 2.0 | 6.0 | 17.0 | 34.3 | 29.8 |  |
| 21 Queensbridge Rd / Hackney Rd | 0.5 | 4.5 | 17.0 | 26.2 | 22.6 |  |
| 22 Wapping Wall / Garnet St | 0.5 | 3.5 | 17.0 | 23.3 | 21.1 |  |
| 23 Brodlove Lane | 0.5 | 3.5 | 17.0 | 29.7 | 25.3 |  |
| 24 Jubilee Street / Commercial Rd | 0.5 | 5.5 | 17.0 | 33.2 | 26.3 |  |
| 25 Cavell St / Stepney Way | 1.0 | 21.0 | 17.0 | 29.9 | 22.0 | Warning: your receptor is more than 20m further from the kerb than your monitor - treat result with caution. |
| 26 Hannibal Rd / Mile End Rd | 0.5 | 3.5 | 17.0 | 29.2 | 25.0 |  |
| 27 Roman Rd / Globe Road | 0.5 | 12.5 | 17.0 | 25.8 | 20.8 |  |
| 28 Bonner Road | 0.5 | 7.5 | 17.0 | 25.7 | 21.5 |  |
| 29 Grove Rd / Old Ford Rd | 0.5 | 12.5 | 17.0 | 27.5 | 21.5 |  |
| 30 Fieldgate Street | 0.5 | 8.5 | 17.0 | 27.7 | 22.3 |  |
| 31 Whitechapel Market | 1.5 | 16.5 | 17.0 | 40.4 | 28.1 |  |
| 32 Globe Rd / Mile End Rd | 0.5 | 4.5 | 17.0 | 29.7 | 24.8 |  |
| 33 Stepney Green | 15.0 | 45.0 | 17.0 | 23.5 | 20.3 | Warning: your receptor is more than 20m further from the kerb than your monitor - treat result with caution. Warning: your monitor is more than 10m further from the kerb than your receptor - treat result with caution. |
| 34 Pitsea St / Commercial Rd | 0.5 | 4.5 | 17.0 | 26.3 | 22.7 |  |
| 35 Narrow St / Limehouse Link | 1.5 | 16.5 | 17.0 | 54.4 | 34.7 |  |
| 36 Locksley St / St Paul's Way | 0.5 | 40.5 | 17.0 | 26.0 | 19.0 | Warning: your receptor is more than 20m further from the kerb than your monitor - treat result with caution. |
| 37 Rhodeswell Rd | 1.0 | 41.0 | 17.0 | 26.5 | 19.4 | Warning: your receptor is more than 20m further from the kerb than your monitor - treat result with caution. |
| 38 Ben Johnson Road | 0.5 | 4.5 | 17.0 | 28.7 | 24.2 |  |
| 39 Harford St / Mile End Rd | 1.5 | 4.5 | 17.0 | 26.4 | 24.1 |  |
| 40 Thoydon Rd | 0.5 | 7.5 | 17.0 | 24.2 | 20.8 |  |
| 41 Ford Close / Roman Rd | 1.5 | 3.5 | 17.0 | 25.9 | 24.2 |  |
| 42 Victoria Park (Co-location site) |  | N/A - distances must be less than 50m |  |  |  |  |
| 43 Victoria Park (Co-location site) | N/A - distances must be less than 50m |  |  |  |  |  |
| 44 Parnell Rd / Old Ford Rd | 0.5 | 4.5 | 17.0 | 27.8 | 23.6 |  |
| 45 St Stephen's Rd / Tredegar Rd | 0.5 | 3.5 | 17.0 | 31.0 | 26.2 |  |
| 46 Rhondda Grove / Mile End Rd | 0.5 | 5.5 | 17.0 | 26.2 | 22.3 |  |
| 47 Wentworth Mews | 0.5 | 15.5 | 17.0 | 32.2 | 23.0 |  |
| 48 Ackroyd Drive | 0.5 | 40.5 | 17.0 | 32.3 | 20.4 | Warning: your receptor is more than 20m further from the kerb than your monitor - treat result with caution. |
| 49 Dod St / Burdett Rd | 0.5 | 5.5 | 17.0 | 24.9 | 21.6 |  |
| 50 Rich Street | 1.5 | 4.5 | 17.0 | 28.5 | 25.7 |  |
| 51 Watney Market | 15.0 | 25.0 | 17.0 | 23.2 | 21.8 | Warning: your receptor is more than 20m further from the kerb than your monitor - treat result with caution. Warning: your monitor is more than 10m further from the kerb than your receptor - treat result with caution. |
| 52 Wick Lane / Autumn St | 0.5 | 3.5 | 17.0 | 29.8 | 25.4 |  |
| 53 Fairfield Road / Tredegar Road | 0.5 | 4.5 | 17.0 | 34.5 | 27.7 |  |
| 54 Bow Rd / Glebe Terrace | 0.5 | 5.5 | 17.0 | 37.0 | 28.5 |  |
| 55 TH Cemetery Park | 5.0 | 20.0 | 17.0 | 16.7 |  | Error: Measured concentration must be above background concentration. |
| 56 Bow Common Lane / St Paul's Way | 0.5 | 30.5 | 17.0 | 25.9 | 19.4 | Warning: your receptor is more than 20m further from the kerb than your monitor - treat result with caution. |
| 57 Augusta St / Giraud St | 1.0 | 16.0 | 17.0 | 22.7 | 19.5 |  |
| 58 Dolphin Lane | 1.0 | 8.0 | 17.0 | 23.1 | 20.5 |  |
| 59 Westferry Road / Limehouse Link jnct | 1.0 | 8.0 | 17.0 | 28.1 | 23.4 |  |
| 60 Cascades, Westferry Road | 0.5 | 18.5 | 17.0 | 33.5 | 23.0 |  |
| 61 Bow Rd / Alfred St | 0.5 | 6.5 | 17.0 | 27.6 | 22.8 |  |
| 62 Mast House Terrace | 0.5 | 5.5 | 17.0 | 26.7 | 22.6 |  |
| 63 Millwall Park |  |  | N/A - distances must be less than 50m |  |  |  |
| 64 Limeharbour | 0.5 | 10.5 | 17.0 | 36.2 | 25.9 |  |
| 65 Manchester Road / East Ferry Road | 0.5 | 2.5 | 17.0 | 24.6 | 22.4 |  |
| 66 Millwall Park |  | N/A - distances must be less than 50m |  |  |  |  |
| 67 Seyssel Street | 0.5 | 15.5 | 17.0 | 26.6 | 20.8 |  |
| 68 Manchester Road / Ollife Street | 0.5 | 3.5 | 17.0 | 26.3 | 23.1 |  |
| 69 Lawnhouse Close | 0.5 | 30.5 | 17.0 | 27.4 | 19.8 | Warning: your receptor is more than 20m further from the kerb than your monitor - treat result with caution. |
| 70 Admirals Way | 0.5 | 15.5 | 17.0 | 23.7 | 19.6 |  |
| 71 Toynbee St / Commercial St | 2.0 | 12.0 | 17.0 | 35.2 | 27.6 |  |
| 72 Prestons Road / Coldharbour | 0.5 | 4.5 | 17.0 | 30.4 | 25.2 |  |
| 73 John Smith Mews | 0.5 | 10.5 | 17.0 | 24.6 | 20.5 |  |
| 74 Poplar High St / Cotton St | 0.5 | 10.5 | 17.0 | 58.6 | 36.2 | Predicted concentration at Receptor within 10% the AQS objective. |
| 75 Hale Street | 0.5 | 7.5 | 17.0 | 23.5 | 20.4 |  |
| 76 Chrisp Street / E India Dock Road | 0.5 | 20.5 | 17.0 | 32.8 | 22.4 | Warning: your receptor is more than 20m further from the kerb than your monitor - treat result with caution. |
| 77 Morris / Barchester Street | 0.5 | 4.5 | 17.0 | 26.0 | 22.5 |  |
| 78 Devons Road / Campbell Road | 0.5 | 10.5 | 17.0 | 32.0 | 23.9 |  |
| 79 Hatfield Terrace / Fairfield Road | 0.5 | 3.5 | 17.0 | 23.6 | 21.3 |  |
| 80 Wrexham Road | 0.5 | 3.5 | 17.0 | 27.6 | 24.0 |  |
| 81 Bromley High Street / St Leonards | 0.5 | 5.5 | 17.0 | 28.5 | 23.6 |  |
| 82 Devas Street / Devons road | 0.5 | 7.5 | 17.0 | 28.7 | 23.1 |  |
| 83 Zetland Street / A12 |  |  | N/A - distances must be less than 50m |  |  |  |
| 84 Blair Street (End of Street) | 5.0 | 20.0 | 17.0 | 35.8 | 28.0 |  |
| 85 Portree Street | 0.5 | 4.5 | 17.0 | 34.3 | 27.6 |  |
| 86 Newport Avenue | 0.5 | 15.5 | 17.0 | 21.7 | 18.8 |  |
| 87 Mile End Road Corner Bancroft Rd | 0.5 | 30.5 | 17.0 | 30.8 | 20.8 | Warning: your receptor is more than 20m further from the kerb than your monitor - treat result with caution. |
| 88 Shirbutt St o/s Holy Family School | 0.5 | 10.5 | 17.0 | 21.3 | 19.0 |  |
| 89 Thames Path Storers Quay | 10.0 | 14.0 | 17.0 | 22.7 | 22.0 |  |
| 90 Sextant Avenue | 1.0 | 5.0 | 17.0 | 19.7 | 18.8 |  |

# Appendix B	Full Monthly Diffusion Tube Results for 2020

Table N.	NO2 Diffusion Tube Results (µgm-3)

| Site ID | Location | Valid data capture for monitoring period % | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec | Annual Mean - Raw Data | Annual Mean - Annualised (LLAQM Technical Guidance) | Annual Mean - National Bias-Adjustment Factor 0.77 as per 03/2021 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Colombia Rd/Gossett Street | 58 | 46.2 | 41.6 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 26.3 | 30.6 | 35.9 | 41.8 | 38.3 | 37.2 | 32.3 | 24.9 |
| 2 | Calvert Ave/Boundary Street | 50 | 50.3 | 44.2 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 29.2 | 31.8 | missing | 38.7 | 38.3 | 38.8 | 33.8 | 26.0 |
| 3 | Bethnal Green Rd/ Brick Lane | 58 | 48.0 | 47.2 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 31.8 | 33.3 | 38.8 | 41.6 | 39.0 | 40.0 | 35.6 | 27.4 |
| 4 | Commercial St/Calvin St | 58 | 57.8 | 47.7 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 45.2 | 47.1 | 45.5 | 50.5 | 54.9 | 49.8 | 44.4 | 34.2 |
| 5 | Whitechapel High St (KFC) | 50 | 63.3 | 56.2 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | missing | 49.0 | 50.6 | 48.5 | 39.3 | 51.2 | 44.0 | 33.9 |
| 6 | Mansell St | 58 | 66.5 | 61.8 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 47.5 | 46.1 | 44.8 | 45.9 | 46.4 | 51.3 | 45.7 | 35.2 |
| 7 | St Katherine's Way | 58 | 37.8 | 31.7 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 20.2 | 23.2 | 29.6 | 35.2 | 25.6 | 29.0 | 25.9 | 19.9 |
| 8 | Wapping High St/Sampson St | 58 | 34.2 | 31.7 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 22.5 | 25.6 | 30.6 | 37.2 | 38.1 | 31.4 | 28.0 | 21.6 |
| 9 | Cartwright Street | 58 | 44.3 | 40.4 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 25.8 | 27.9 | 33.5 | 42.6 | 37.7 | 36.0 | 32.1 | 24.7 |
| 10 | Whitechapel Rd/Adler St | 58 | 48.0 | 42.8 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 36.9 | 38.9 | 41.8 | 44 | 41.9 | 42.0 | 37.5 | 28.9 |
| 11 | Brick Lane/Princelet St | 50 | 41.9 | 37.8 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 30 | missing | 31.3 | 38.1 | 37.2 | 36.1 | 31.7 | 24.4 |
| 12 | Buckfast St/Bethnal Green Rd | 58 | 44.2 | 40.6 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 28.7 | 31.1 | 28.3 | 36.8 | 37.5 | 35.3 | 31.5 | 24.2 |
| 13 | Squirries St/Gosset St | 58 | 45.5 | 38.3 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 33.8 | 38.1 | 35.2 | 41.9 | 43.7 | 39.5 | 35.2 | 27.1 |
| 14 | Warner Place/Hackney Rd | 58 | 55.0 | 44.0 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 27.1 | 29.9 | 32.8 | 37.3 | 34.0 | 37.2 | 33.1 | 25.5 |
| 15 | Parmiter St/ Cambridge Heath Road | 58 | 55.9 | 45.7 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 36.1 | 39.6 | 41.3 | 44.8 | 45.5 | 44.1 | 39.4 | 30.3 |
| 16 | Paradise Row/Bethnal Green Rd | 58 | 52.7 | 48.3 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 33.3 | 32.6 | 38.4 | 37.7 | 43.4 | 40.9 | 36.5 | 28.1 |
| 17 | Finnis St/Three Colts Lane | 58 | 34.0 | 30.0 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 24.1 | 28.2 | 31.4 | 31.5 | 37.3 | 30.9 | 27.6 | 21.2 |
| 18 | Sidney St/Mile End Rd | 58 | 47.3 | 46.8 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 37.4 | 38.2 | 40 | 42.4 | 46.9 | 42.7 | 38.1 | 29.3 |
| 19 | Philpot St/Commercial Road | 58 | 54.6 | 54.0 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 39.4 | 35.5 | 38.9 | 43.6 | 50.2 | 45.2 | 40.3 | 31.0 |
| 20 | Dellow St/The Highway | 58 | 47.8 | 47.6 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 46.5 | 47.5 | 53.9 | 55 | 51.1 | 49.9 | 44.5 | 34.3 |
| 21 | Queensbridge Rd/Hackney Rd | 58 | 46.1 | 43.9 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 28.4 | 32.4 | 36.3 | 40.6 | 39.3 | 38.1 | 34.0 | 26.2 |
| 22 | Wapping Wall/Garnet St | 58 | 42.1 | 32.8 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 25.6 | 30.0 | 33 | 36 | 38.5 | 34.0 | 30.3 | 23.3 |
| 23 | Brodlove Lane | 50 | 49.3 | 46.7 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 37 | missing | 43.7 | 43.5 | 43.3 | 43.9 | 38.6 | 29.7 |
| 24 | Jubilee Street/Commercial Rd | 58 | 47.2 | 36.8 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 51.2 | 55.3 | 45.3 | 51.7 | 51.0 | 48.4 | 43.1 | 33.2 |
| 25 | Cavell St/Stepney Way | 58 | 60.0 | 47.0 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 30.7 | 33.8 | 41.3 | 47.5 | 44.8 | 43.6 | 38.9 | 29.9 |
| 26 | Hannibal Rd/Mile End Rd | 33 | 53.9 | 49.0 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | missing | missing | missing | 43.2 | 42.5 | 47.2 | 37.9 | 29.2 |
| 27 | Roman Rd/Globe Road | 58 | 42.6 | 35.2 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 33.5 | 30.0 | 34.8 | 44.3 | 43.1 | 37.6 | 33.6 | 25.8 |
| 28 | Bonner Road | 58 | 36.8 | 35.6 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 32 | 38.0 | 34.6 | 40.7 | 44.0 | 37.4 | 33.3 | 25.7 |
| 29 | Grove Rd/Old Ford Rd | 50 | 51.8 | 44.8 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 32.3 | 38.7 | 28.9 | 41.9 | missing | 39.7 | 35.7 | 27.5 |
| 30 | Fieldgate Street | 58 | 41.4 | 41.8 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 35.6 | 36.4 | 39.3 | 43.1 | 44.3 | 40.3 | 35.9 | 27.7 |
| 31 | Whitechapel Market | 58 | 65.7 | 58.5 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 57.5 | 59.4 | 59.1 | 55.6 | 56.1 | 58.8 | 52.5 | 40.4 |
| 32 | Globe Rd/Mile End Rd | 58 | 47.6 | 40.3 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 42.6 | 43.0 | 40.8 | 42.9 | 45.1 | 43.2 | 38.5 | 29.7 |
| 33 | Stepney Green | 58 | 40.3 | 34.7 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 24.9 | 30.1 | 32.7 | 38.5 | 38.0 | 34.2 | 30.5 | 23.5 |
| 34 | Pitsea St/Commercial Rd | 58 | 46.0 | 37.7 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 31.5 | 34.8 | 32.9 | 41 | 44.0 | 38.3 | 34.1 | 26.3 |
| 35 | Narrow St/Limehouse Link | 58 | 87.1 | 75.6 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 86.3 | 86.6 | 70.2 | 79.8 | 68.8 | 79.2 | 70.6 | 54.4 |
| 36 | Locksley St/St Paul's Way | 50 | 44.7 | missing | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 32.1 | 33.5 | 35.6 | 42 | 39.2 | 37.9 | 33.7 | 26.0 |
| 37 | Rhodeswell Rd | 58 | 42.0 | 29.3 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 32.0 | 30.1 | 31.6 | 36.6 | 37.5 | 34.2 | 34.5 | 26.5 |
| 38 | Ben Johnson Road | 58 | 43.5 | 37.9 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 37.3 | 41.7 | 41.8 | 45.6 | 45.0 | 41.8 | 37.3 | 28.7 |
| 39 | Harford St/Mile End Rd | 58 | 44.2 | 31.8 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 34.5 | 40.5 | 38.2 | 40.4 | 39.3 | 38.4 | 34.3 | 26.4 |
| 40 | Thoydon Rd | 50 | missing | null | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 30.8 | 29.4 | 30.1 | 41.8 | 36.8 | 33.8 | 31.4 | 24.2 |
| 41 | Ford Close/Roman Rd | 50 | 44.4 | 35.1 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | missing | 39.3 | 36.7 | 39 | 40.0 | 39.1 | 33.6 | 25.9 |
| 42 | Victoria Park (Co-location site) | 58 | 32.4 | 23.1 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 18.7 | 19.5 | 24 | 28.5 | 26.6 | 24.7 | 22.0 | 16.9 |
| 43 | Victoria Park (Co-location site) | 58 | 30.6 | 22.9 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 17.5 | 18.4 | 21.6 | 27.3 | 28.6 | 23.8 | 21.3 | 16.4 |
| 44 | Parnell Rd/Old Ford Rd | 58 | 51.3 | 39.3 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 30.2 | 36.9 | 38.6 | 44.9 | 41.9 | 40.4 | 36.1 | 27.8 |
| 45 | St Stephen's Rd/Tredegar Rd | 58 | 54.8 | 40.0 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 41.2 | 45.0 | 41 | 48.5 | 45.2 | 45.1 | 40.2 | 31.0 |
| 46 | Rhondda Grove/Mile End Rd | 42 | 47.7 | 37.8 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 30 | missing | 39.6 | 38 | missing | 38.6 | 34.1 | 26.2 |
| 47 | Wentworth Mews | 58 | 57.3 | 43.7 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 47.9 | 40.8 | 40 | 49.2 | 49.2 | 46.9 | 41.8 | 32.2 |
| 48 | Ackroyd Drive | 58 | 55.5 | 46.6 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 41.4 | 42.7 | 46.9 | 48.2 | 48.1 | 47.1 | 42.0 | 32.3 |
| 49 | Dod St/Burdett Rd | 58 | 48.3 | 34.7 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 26.7 | 30.5 | 34.6 | 38.6 | 40.0 | 36.2 | 32.3 | 24.9 |
| 50 | Rich Street | 58 | 51.5 | 43.0 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 37 | 39.5 | 36.9 | 39.6 | 43.5 | 41.6 | 37.1 | 28.5 |
| 51 | Watney Market | 33 | 43.8 | 29.5 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | missing | missing | 32.6 | 38.6 | missing | 36.1 | 30.2 | 23.2 |
| 52 | Wick Lane/Autumn St | 50 | 51.9 | 40.2 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 42.6 | 37.0 | missing | 46.4 | 48.3 | 44.4 | 38.7 | 29.8 |
| 53 | Fairfield Road/Tredegar Road | 50 | null | 47.5 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 44.8 | 50.3 | 48.7 | 49.6 | 50.0 | 48.5 | 44.8 | 34.5 |
| 54 | Bow Rd /Glebe Terrace | 42 | 65.8 | 53.4 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | missing | missing | 54.2 | 57.6 | 54.5 | 57.1 | 48.0 | 37.0 |
| 55 | TH Cemetery Park | 58 | 31.8 | 22.6 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 18.7 | 22.8 | 20.9 | 25.5 | 27.7 | 24.3 | 21.6 | 16.7 |
| 56 | Bow Common Lane/St Paul's Way | 58 | 44.8 | 34.8 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 29.6 | 31.2 | 38.4 | 43.1 | 42.6 | 37.8 | 33.7 | 25.9 |
| 57 | Augusta St/Giraud St | 58 | 41.5 | 30.3 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 23.8 | 28.6 | 31.8 | 36.9 | 38.3 | 33.0 | 29.5 | 22.7 |
| 58 | Dolphin Lane | 58 | 45.7 | 34.0 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 27.3 | 26.5 | 30.8 | 37 | 34.4 | 33.7 | 30.0 | 23.1 |
| 59 | Westferry Road/Limehouse Link jnct | 50 | 51.0 | missing | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 36 | 35.5 | 40.4 | 42.3 | 40.8 | 41.0 | 36.5 | 28.1 |
| 60 | Cascades, Westferry Road | 58 | 56.0 | 40.4 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 44.3 | 43.9 | 51.3 | 54.4 | 51.1 | 48.8 | 43.5 | 33.5 |
| 61 | Bow Rd/Alfred St | 58 | 51.1 | 41.4 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 30.2 | 32.0 | 37.5 | 46.7 | 42.6 | 40.2 | 35.9 | 27.6 |
| 62 | Mast House Terrace | 58 | 47.2 | 34.2 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 32 | 34.6 | 37 | 45.2 | 42.1 | 38.9 | 34.7 | 26.7 |
| 63 | Millwall Park | 50 | 44.8 | 37.0 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | null | 22.9 | 26.7 | 31.1 | 28.1 | 31.8 | 27.3 | 21.1 |
| 64 | Limeharbour | 58 | 56.7 | 54.3 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 45.6 | 52.2 | 51.3 | 54.3 | 54.3 | 52.7 | 47.0 | 36.2 |
| 65 | Manchester Road/East Ferry Road | 50 | 43.5 | 32.5 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 30.5 | 33.0 | 33.7 | 40.8 | missing | 35.7 | 32.0 | 24.6 |
| 66 | Millwall Park | 58 | 31.5 | 25.7 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 18.6 | 24.8 | 29.6 | 29.4 | 25.7 | 26.5 | 23.6 | 18.2 |
| 67 | Seyssel Street | 58 | 48.6 | 32.2 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 33.9 | 35.5 | 39.9 | 38.5 | 42.3 | 38.7 | 34.5 | 26.6 |
| 68 | Manchester Road/Ollife Street | 42 | 45.7 | 31.0 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | missing | missing | 39.3 | 44.5 | 42.4 | 40.6 | 34.1 | 26.3 |
| 69 | Lawnhouse Close | 58 | 44.9 | 37.3 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 26.7 | 35.6 | 40.5 | 45.1 | 41.2 | 38.8 | 35.6 | 27.4 |
| 70 | Admirals Way | 58 | 50.6 | 38.4 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 24.4 | 29.1 | 33.2 | 40 | 26.2 | 34.6 | 30.8 | 23.7 |
| 71 | Toynbee St/Commercial St | 42 | 63.2 | 56.7 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | missing | missing | 47 | 54.1 | 51.0 | 54.4 | 45.7 | 35.2 |
| 72 | Prestons Road/ Coldharbour | 58 | 50.7 | 39.2 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 39.9 | 43.8 | 43.7 | 48.1 | 44.4 | 44.3 | 39.5 | 30.4 |
| 73 | John Smith Mews | 58 | 42.9 | 32.5 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 28.3 | 32.1 | 36.6 | 38.9 | 39.7 | 35.9 | 32.0 | 24.6 |
| 74 | Poplar High St/Cotton St | 58 | 102.2 | 74.6 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 97 | 90.5 | 85.9 | 85.2 | 62.0 | 85.3 | 76.1 | 58.6 |
| 75 | Hale Street | 58 | 42.0 | 34.4 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 24.6 | 30.0 | 35.4 | 37.2 | 36.1 | 34.2 | 30.5 | 23.5 |
| 76 | Chrisp Street/E India Dock Road | 58 | 54.9 | 44.4 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 43 | 46.1 | 46.4 | 51.6 | 48.5 | 47.8 | 42.7 | 32.8 |
| 77 | Morris/Barchester Street | 50 | 51.0 | 39.3 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 33.2 | missing | 31 | 38.1 | 38.3 | 38.5 | 33.8 | 26.0 |
| 78 | Devons Road / Campbell Road | 50 | 52.9 | 39.9 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 41.3 | 37.9 | missing | 71.7 | 42.8 | 47.8 | 41.6 | 32.0 |
| 79 | Hatfield Terrace/Fairfield Road | 58 | 38.8 | 31.2 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 30.8 | 29.4 | 32.3 | 39.2 | 39.1 | 34.4 | 30.7 | 23.6 |
| 80 | Wrexham Road | 58 | 44.8 | 34.0 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 35.1 | 38.3 | 39.1 | 46 | 43.9 | 40.2 | 35.8 | 27.6 |
| 81 | Bromley High Street/ St Leonards | 58 | 49.4 | 34.9 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 35.8 | 40.0 | 41.7 | 43.6 | 45.3 | 41.5 | 37.0 | 28.5 |
| 82 | Devas Street /Devons road | 50 | 52.7 | 38.5 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 38.3 | missing | 25.4 | 50.8 | 48.6 | 42.4 | 37.2 | 28.7 |
| 83 | Zetland Street/A12 | 58 | 63.1 | 48.5 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 70.3 | 64.0 | 55 | 58.3 | 60.6 | 60.0 | 53.5 | 41.2 |
| 84 | Blair Street (End of Street) | 58 | 67.4 | 58.8 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 42.1 | 40.5 | 52.7 | 51.5 | 51.5 | 52.1 | 46.4 | 35.8 |
| 85 | Portree Street | 58 | 57.3 | 45.2 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 41.7 | 43.8 | 50.3 | 56.8 | 54.8 | 50.0 | 44.6 | 34.3 |
| 86 | Newport Avenue | 58 | 39.7 | 28.8 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 25.4 | 30.0 | 25.2 | 37.3 | 34.9 | 31.6 | 28.2 | 21.7 |
| 87 | Mile End Road Corner Bancroft Rd | 58 | 48.5 | 35.8 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 38.4 | 43.9 | 40.6 | 43.2 | 44.4 | 42.1 | 40.0 | 30.8 |
| 88 | Shirbutt St o/s Holy Family School | 58 | 39.6 | 28.1 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 23.3 | 25.6 | 28.5 | 36.7 | 35.6 | 31.1 | 27.7 | 21.3 |
| 89 | Thames Path Storers Quay | 42 | 39.1 | 28.3 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | missing | missing | 31.7 | 40.4 | 35.6 | 35.0 | 29.4 | 22.7 |
| 90 | Sextant Avenue | 50 | 35.8 | 26.3 | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | 1st Covid Lockdown | missing | 25.2 | 27 | 32.1 | 32.3 | 29.8 | 25.6 | 19.7 |

# Notes

Concentrations are presented as μg m-3.

Exceedances of the NO2 annual mean AQO of 40μg m-3 are shown in bold.

NO2 annual means in excess of 60μg m-3, indicating a potential exceedance of the NO2 hourly mean AQS objective are shown in bold and underlined.

All means have been “annualised” in accordance with LLAQM Technical Guidance if valid data capture for the calendar year is less than 75% and greater than 33%.

(a) Data capture for the monitoring period, in cases where monitoring was only carried out for part of the year.

(b) Data capture for the full calendar year (e.g. if monitoring was carried out for six months the maximum data capture for the full calendar year would be 50%).

LLAQM Policy and Technical Guidance 2019 (LLAQM.TG(19))

New BAM PM 2.5 monitor installed in 2019

Site operated by Transport for London